S-SG30A-W

Product Overview

• Delivers high power white light for night time illumination, it’s significantly improves

monitor effects of night vision camera.

• Adopted with Compound-eye LED lens, the product performs more smoothly and

evenly light spot radiation.

• Excellent color rendering index, truly reserve object colors.

• Innovating high-efficiency driver provides a zero maintenance product.

• Good heat dissipation and IP66 waterproof aluminum housing ensures

efficient, reliable and continuous operation of LED.

Product Features

• Outstanding illumination performance

• Auto day/night switch, customize to be synchronized with camera

• To improve performance of night vision camera and enhance image quality

• IP66 waterproof, indoor/outdoor use

• Low temperature operate

• Lifetime 50,000 hours

• Energy saving technology

• 2 year warranty

Wide Application

Parking lot, Residence Community, Building, Road, Garden, Field

Viewing picture at night

With white light       Without white light

Technical Specification                    Drawing

LED type: 30pieces high power white light LED

Luminous flux: 3600lm

Color temperature: 5000K-6500K

Beam angle: 30 to 60°

Beam distance: 130 to 70m

Bracket: wall mount bracket

Working voltage: 85-265V AC

Power consumption: 36W

Day/night switch: automatic

Working temperatures: -30 to 50°C

Protection level: IP66

Color: beige

Panel: tempered glass panel

Housing: aluminum die-casting housing

N.W: 1.8KG

G.W: 2.1KG

Dimension: L221*H172*W90mm

Packing dimension: L250*H200*W120mm

Model for Selection                       Product Certificate

Model         Beam angle   Viewing distance

S-SG30A-30-A-W       30°          130m

S-SG30A-45-A-W       45°           80m

S-SG30A-60-A-W       60°           70m

*Viewing distance is tested by Sony 1/3 CCD Camera with

0.01LUX.

*To achieve the distance needed, please pay attention to your

camera specification and lens before purchase. It is normal

phenomenon that distance may vary due to different cameras

and lens.