Vari-Focal IR Camera
Instruction Manual

VER.:1.0, NO.:040156

Thank you very much for purchasing our product. Before operating this product, please read this instruction manual carefully to ensure proper use.

1. Safety Precautious

|  | Disposal of Old Electrical & Electronic Equipment (Applicable in the European Union and other European countries with separate collection systems). |
| --- | --- |

- Please be extra careful not to shake the camera

- Please avoid places where there is direct sunlight

- When using this camera in places where the lighting differs greatly, please use the auto iris lens with ND filter.

- Please avoid places where temperatures exceed 50℃ or more, high humidity or where direct rain drops hit, frequent vibrations, or shocks occur.

- During the night, if a minimum brightness of 0.1 Lux can not be achieved, install appropriate light fixtures.

- When the camera is not used, keep the lens or the lens cap attached to protect the sensor.

2. Description

The camera uses SONY high sensitive super HAD interline transfer image sensor, and employs digital signal processor (DSP) chip-set for image control, and all integrated state circuitry which provide extremely long life and high reliability. This camera offers excellent image quality with low lag and high burn resistance, and is not subject to distortions from magnetic fields.

Highly resistance to shock and vibration, easy to install, this camera is a very good choice for your color CCTV system.

3. Feature

- Camera with 1/3“ SONY Super HAD CCD sensor in metal case.

- Employs Digital Signal Processor (DSP) chip-set for image control.

- Pixel number: NTSC=380K/ PAL=440K for high resolution models, and NTSC=270K/ PAL=320K for normal resolution models.

- High sensitivity, low smear, high anti-blooming, and high S/N ratio.

- Powerful functions: Auto Electronic Shutter (AES), Auto Iris (AI), Auto Gain Control (AGC), Auto White Balance (AWB), and Back Light Compensation (BLC).

- Built-in anti-aliasing and IR-cut optical filter.

- Cds auto switching IR-LED control.

4. Contents

| Item | Name of Part | Quantity |
| --- | --- | --- |
| 1 | Camera | 1 |
| 2 | Appurtenance Bag | 1 |
| 3 | Screws | 2 |
| 4 | Mounting Block | 1 |
| 5 | Instruction Manual | 1 |

5. Name of Each Part

Fig. 1

Fig. 2

Fig. 3

6. Installation & Operation

Please refer to Fig. 1 or Fig. 2 (P3 & P4) and setup guide below:

- Connect the video output (1) to the monitor or other video device through a 75 Ohms type coaxial cable.

- Connect the power source (2), AC with the power cord, or DC by using the DC Jack (+12V DC is in the center of the jack plug), then the LED indicator will lit.

- Once the picture appears on the monitor, open the cover (4) adjust the lens wrench of “WIDE←→TELE” (5) to get the view angle that you desire, then adjust the focus wrench (6) of the lens to obtain the best picture.

- Tighten the two wrench (5) and (6), then put the cover (4) back.

- Mount the mounting bracket to hole (7) on the top or bottom of the camera, using the enclosed 8mm mounting block with 2 screws furnished.

- DIP Switch (8) Setting

  - BLC FUNCTION

Set Switch-OFF/ BLC to BLC to enable Back Light Compensation (BLC) function.

  - AUTO IRIS FUNCTION

When using an Auto Iris Lens, the Switch-AES/ OFF should be set to OFF, to disable the AES function. Adjust the VR LEVEL (9) to the proper level.

  - AGC-Hi / AGC-Lo Function

This switch setting to High or Low of the AGC maximum gain.

  - Flickerless Function

Set switch-OFF/FLON to FLON to enable flickerless function, in this mode, the switch-AES/OFF is auto disabled.

  - AES Function

When using the lens (other than Auto Iris Lens), the Switch-AES/ OFF should be set to AES.

7. Specification

According to the camera purchased, select and refer to the appropriate specification below:

| Note: Design and specifications are subject to change without prior notice. Color Camera |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Image Device |  | 1/3” Color SONY Super HAD | 1/3” Color SONY Hi-Res. Super HAD | 1/3” Color SONY Ex-View HAD | 1/3” Color SONY Hi-Res. Ex-View HAD |
| Picture Elements |  | NTSC:510*492 PAL:500*582 | NTSC:768*494 PAL:752*582 | NTSC:510*492 PAL:500*582 | NTSC:768*494 PAL:752*582 |
| Resolution |  | 380 TVL | 470 TVL | 380 TVL | 470 TVL |
| Min. Illumination |  | 0.2 Lux / F2.0 | 0.3 Lux / F2.0 | 0.05 Lux / F2.0 | 0.15 Lux / F2.0 |
| S/N Ratio |  | More than 48 dB |  |  |  |
| Electronic Shutter |  | NTSC:1/60s~1/100,000s, PAL:1/50s~1/110,000s |  |  |  |
| Flickerless Mode |  | On/ Off (NTSC:1/100s, PAL:1/120s) |  |  |  |
| Gamma |  | 0.45 |  |  |  |
| Iris Control |  | DC Drive |  |  |  |
| Lens Furnished |  | Vari-Focal: f3.7~12.0mm/ F1.6~360 (Wide:H71.6°V23.8°, Tele:H55.8°V17.9°) Vari-Focal: f3.7~12.0mm (ICR)/ F1.6~360 (Wide:H71.6°V23.8°, Tele:H55.8°V17.9°) Vari-Focal: f4.0~9.0mm/ F1.6~360 (Wide:H68.0°V50.2°, Tele:H32.1°V24.1°) Vari-Focal: f9.0~22.0mm/ F1.6~360 (Wide:H29.2°V13.0°, Tele:H21.7°V9.7°) |  |  |  |
| ICR Function |  | Auto (optional) |  |  |  |
| Gain Control |  | High / Low |  |  |  |
| White Balance |  | Auto |  |  |  |
| Back Light Comp. |  | ON/OFF |  |  |  |
| Sync. System |  | Internal |  |  |  |
| Video Output |  | 1 Vp-p / 75 Ohms. |  |  |  |
| Audio Output |  | 2 Vp-p / 50 Ohms. (optional) |  |  |  |
| Power Supply |  | DC12V ±10％, AC24V, AC100~240V (optional) |  |  |  |
| Power Consumption | IR OFF | DC/ 170mA max., AC/ 2.5 W max. |  |  |  |
|  | IR ON | DC/ 280mA max., AC/ 3.8 W max. |  |  |  |
| Operating Temp. |  | -10℃ to 50℃ (14℉ to 122℉) |  |  |  |
| Infrared Illuminator Module |  |  |  |  |  |
| Wavelength |  | 850nm |  |  |  |
| Illuminate Distance |  | 10M |  |  |  |