Waterproof Infrared

Illumination Camera

Instruction Manual

VER.:1.0, NO.: R040057/4

Thank you very much for purchasing our product. Before operating this product, please read this instruction manual carefully to ensure proper use.

1. Safety Precautious

|  | Disposal of Old Electrical & Electronic Equipment (Applicable in the European Union and other European countries with separate collection systems). |
| --- | --- |

- When in use, please avoid camera window directly contacting the eyes and do not touch the camera body, for it can be heated up to 70℃.

- When installing the camera, please avoid pointing it directly to paper or flammable materials.

- Although the camera is provided with tempered glass, please avoid all direct contact to eliminate contamination, and use cotton balls dipped in cleaning alcohol to clean the window.

2. Description

This inbuilt Infrared Illuminator camera uses a high sensitive 1/3” (1/4”) interline transfer Charge Coupled Device (CCD) image sensor, and all state circuitry, which provides extremely long life and high reliability. This camera offers excellent image quality and functions with low lag and high burn resistance, and is not subject to distortions from magnetic fields.

Highly resistant to shock and vibration and easy to install, this camera is an excellent choice for your CCTV system.

3. Feature

- Built-in high quality CCD camera and high performance infrared illuminator module.

- Infrared illuminator has effective range of up to 10 (15) meters and wide angle at 35, also assists cameras in providing a clear image when under completely dark area.

- Built-in light-source sensor can automatically turn on/ off the infrared illuminator by detecting the light source of the environment.

- Waterproof suitable for indoor and outdoor use.

# 4. Content

| Item | Name of Part |  | Quantity |
| --- | --- | --- | --- |
| 0 | Instruction Manual |  | 1 |
| 1 | Infrared Camera |  | 1 |
| 2 | Mounting Bracket Bag |  | 1 |
|  | A | Base Plate | 1 |
|  | B | Supporting Arm | 1 |
|  | C | Mounting Bracket A | 1 |
| 3 | Appurtenances Bag |  | 1 |
|  | D | Fix Retaining Screw for Bracket A | 3 |
|  | E | Anchor | 3 |

# 5. Installation & Operation

  - Use the 3 screws provided to attach the mounting bracket to the surface or wall, assemble all spare parts to the base plate and tighten the camera wing nut (Fig. 1).

Fig. 1

2. Screw the mounting bracket into the camera mounting post; then adjust the camera to approximate location and tighten the camera wing nut (Fig. 2).

Fig. 2

3. Connect the video output and the power input (Fig. 2).

# 6. specifications

According to the camera purchased, select and refer to the appropriate specification below:

| #### Color Camera |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Image Device |  | 1/3” SONY Super HAD |  | 1/3” SONY Hi-Res. Super H AD | 1/3” Hi- Res. CCD | 1/3” SONY Ultra Hi- Res. Super HAD | 1/4” SONY Super HAD |
| Picture Elements |  | NTSC: 510x492 PAL: 500x582 |  | NTSC: 768x494 PAL: 752x582 |  |  | NTSC: 510x492 PAL: 500x582 |
| Resolution |  | 380 TVL | 420 TVL | 470 TVL | 540 TVL | 550 TVL | 380 TVL |
| Min Illumination |  | 0.02Lux/ F2.0 |  | 0.03Lux/ F2.0 |  | 0.04Lux/ F2.0 | 0.03Lux/ F2.0 |
| S/N Ratio |  | More than 48dB |  |  |  |  |  |
| Electronic Shutter |  | NTSC: 1/60S~1/100,000S PAL: 1/50S~1/110,000S |  |  |  |  |  |
| Gain Control |  | Auto |  |  |  |  |  |
| White Balance |  | Color Temperature: 2500K~9500K |  |  |  |  |  |
| Scanning System |  | Interlace 2:1 |  |  |  |  |  |
| Gamma |  | 0.45 |  |  |  |  |  |
| Lens Furnished |  | Board Lens |  |  |  |  |  |
| Sync System |  | Internal |  |  |  |  |  |
| Video Output |  | 1Vp-p/ 75 Ohms |  |  |  |  |  |
| Power Supply |  | DC 12V±10％ |  |  |  |  |  |
| Power Consumption | IR OFF | 120mA max. | 75mA max. | 120mA max. | 75mA max. | 150mA max. | 75mA max. |
|  | 12 IR LED ON | 280mA max. | 235mA max. | 280mA max. | 235mA max. | 320mA max. | 235mA max. |
|  | 24 IR LED ON | 340mA max. | 295mA max. | 340mA max. | 395mA max. | 380mA max. | 295mA max. |

| #### B/W Camera |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Image Device |  |  |  | 1/3” SONY Super HAD | 1/3” SONY Hi-Res. Super HAD |  |  | 1/3” B/W Camera |
| Picture Elements |  |  |  | EIA: 510X492 CCIR: 500x582 | EIA: 768x494 CCIR: 752x582 |  |  | EIA: 510X492 CCIR: 500x582 |
| Resolution |  |  |  | 420 TVL | 600 TVL |  |  | 400 TVL |
| Min Illumination |  |  |  | 0.1Lux/ F2.0 | 0.2Lux/ F2.0 |  |  | 0.3Lux/ F2.0 |
| S/N Ratio |  |  |  | More than 48Db |  |  |  |  |
| Electronic Shutter |  |  |  | EIA: 1/60S~1/100,000S, CCIR: 1/50S~1/110,000S |  |  |  |  |
| Gain Control |  |  |  | Auto |  |  |  |  |
| Scanning System |  |  |  | Interlace 2:1 |  |  |  |  |
| Gamma |  |  |  | 0.45 |  |  |  |  |
| Lens Furnished |  |  |  | Board Lens |  |  |  |  |
| Sync System |  |  |  | Internal |  |  |  |  |
| Video Output |  |  |  | 1Vp-p/ 75 Ohms |  |  |  |  |
| Power Supply |  |  |  | DC 12V±10％ |  |  |  |  |
| Power Consumption | IR OFF |  |  | 120mA max. |  |  |  | 100mA max. |
|  | 12 IR LED ON |  |  | 280mA max. |  |  |  | 320mA max. |
|  | 24 IR LED ON |  |  | 340mA max. |  |  |  | 360mA max. |
| #### Infrared Illuminator Module |  |  |  |  |  |  |  |  |
| Infrared Luminary |  |  | 12 IR LED |  |  | 24 IR LED |  |  |
| Filter |  |  | Tempered Glass |  |  |  |  |  |
| Wavelength |  |  | 850nm |  |  |  |  |  |
| Illuminant Distance |  |  | 10 M |  |  | 20 M |  |  |
| Power Supply |  |  | DC 12V±10％ |  |  |  |  |  |
| Power Consumption |  |  | 2 W |  |  |  | 3 W |  |
| #### System Device |  |  |  |  |  |  |  |  |
| Operating Temp. |  | -10℃ to 50℃ (14℉ to 122℉) |  |  |  |  |  |  |
| Construction |  | Aluminum Case |  |  |  |  |  |  |
| Waterproofing Criterion |  | IP 67 |  |  |  |  |  |  |
| Dimensions |  | φ53x58 (L) mm (Body only) |  |  |  |  |  |  |

(Note: Design and specifications are subject to change without prior notice.)