Space Dome Camera
Instruction Manual

VER.:1.0, NO.: R040160A/3

Thank you very much for purchasing our product. Before operating this product, please read this instruction manual carefully to ensure proper use.

1. Safety Precautious

|  | Disposal of Old Electrical & Electronic Equipment (Applicable in the European Union and other European countries with separate collection systems). |
| --- | --- |

- Please do not directly touch the sensor element. If necessary, use a soft cloth moistened with alcohol to wipe off any dust.

- Please be extra careful not to shake the camera.

- Please avoid places where there is direct sunlight.

- When using this camera in places where the lighting differs greatly, please use the auto iris lens with ND filter.

- Please avoid places where temperatures exceed 50℃ or more, high humidity or where direct rain drops hit, frequent vibrations, or shocks occur.

- During the night, if a minimum brightness of 0.1 Lux can not be achieved, install appropriate light fixtures.

- When the camera is not used, keep the lens or the lens cap attached to protect the sensor.

2. Description

The camera uses SONY high sensitive super HAD interline transfer image sensor, and employs digital signal processor (DSP) chip-set for image control, and all integrated state circuitry which provide extremely long life and high reliability. This camera offers excellent image quality with low lag and high burn resistance, and is not subject to distortions from magnetic fields.

Highly resistance to shock and vibration, easy to install, this camera is a very good choice for your color CCTV system.

3. Feature

- Sony Super HAD CCD sensor.

- Employs Digital Signal Processor (DSP) chip-set for image control.

- Pixel number: NTSC=380K/ PAL=440K (EIA=440K/ CCIR=380K) for high resolution models, and NTSC=250K/ PAL=290K (EIA=290K/ CCIR=250K) for normal resolution models.

- High sensitivity, low smear, high anti-blooming and high S/N ratio.

- Powerful functions: Auto Electronic Shutter (AES), Auto Iris (AI), Auto Gain Control (AGC), Auto White Balance (AWB), and Back Light Compensation (BLC).

4. Contents

| Item | Name of Part |  | Quantity |
| --- | --- | --- | --- |
| 1 | Camera |  | 1 |
| 2 | Instruction Manual |  | 1 |
| 3 | Appurtenance Bag |  | 1 |
|  | A | Screws | 4 |

5. Installation & Operation

- Remove the dome cover from the mounting base by rotating the dome cover clockwise.

- Camera Image Adjustment:

    - 2-Axis – Set proper image by moving the camera module and set the focus by turning the lens to the left or right direction.

    - 3-Axis – First tilt then rotate the camera module to obtain desired viewing angle, and then turn the horizontal adjustment ring to correct the image and achieve proper orientation.

Shade Facing Left

Shade Facing Top

Note: When adjusting 3-Axis Vari-Focal lens, before following the Camera Image Adjustment setup, you must turn the horizontal adjustment ring so that the shaded area is facing toward you on the top or left side.

- Connect the video output to the monitor or other video device through a 75 Ohms type coaxial cable and the DC-Jack or AC/ DC-Terminator to the power source.

Note: Power adapter is sold separately.

- Vari-Focal Dome Operation Guide 
Once the picture appears on the monitor, open the cover and adjust the lens wrench to “NEAR←→FAR”, get the view zoom that you desire, and then adjust the focus wrench of the lens to obtain the best picture. After adjustment, tighten both wrenches.

DIP Switch Setting

    - Flickerless Function

Set switch-OFF/ F.L. to Flicker to enable flickerless function, in this mode, the switch-AES/ OFF is auto disabled.

    - AGC-Hi/ AGC-Lo Function

Adjust the switch to AGC indicates AGC-Hi and OFF indicates AGC-Low. AGC-Hi mode--The maximum AGC gain is approximately 26dB. AGC-Lo mode--The maximum AGC gain is approximately 16dB.

    - Auto Iris Function

When using an auto iris lens, the switch-AES/ OFF should be in the OFF position to disable the AES function. Adjust VR to the proper level.

    - BLC Function

Set switch-OFF/ BLC to BLC to enable Back Light Compensation (BLC) function.

6. Specification

According to the camera purchased, select and refer to the appropriate specification below:

| Color Camera |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Image Device |  |  | 1/3“ SONY Super HAD |  |  | 1/3“ SONY Hi-Res. Super HAD |  | 1/3” SONY Ultra Hi-Res. Super HAD |  | 1/4” SONY Super HAD |
| Picture Elements |  |  | NTSC: 510 x 492 PAL: 500 x 582 |  |  | NTSC: 768 x 494 PAL: 752 x 582 |  |  |  | NTSC: 510 x 492 PAL: 500 x 582 |
| Resolution |  |  | 380 TVL |  |  | 470 TVL |  | 550 TVL |  | 330 TVL |
| Min. Illumination |  |  | 0.2Lux/ F2.0 |  | 0.3Lux/ F2.0 | 0.3Lux/ F2.0 |  | 0.4Lux/ F2.0 |  | 0.5Lux/ F2.0 |
| S/N Ratio |  |  | More than 48 dB |  |  |  |  |  |  |  |
| Electronic Shutter |  |  | NTSC:1/60~1/100,000, PAL:1/50~1/110,000 |  |  |  |  |  |  |  |
| Flickerless Mode |  |  | On/ Off (NTSC: 1/100, PAL: 1/120) |  |  |  |  |  |  |  |
| Gamma |  |  | 0.45 |  |  |  |  |  |  |  |
| Iris Control |  |  | DC Drive (for Vari-Focal Lens) |  |  |  |  |  |  |  |
| Lens Furnished |  |  | Vari-Focal Lens and Board Lens |  |  |  |  |  |  |  |
| White Balance |  |  | Auto |  |  |  |  |  |  |  |
| Gain Control |  |  | Hi/ Low (for Vari-Focal model) or Auto |  |  |  |  |  |  |  |
| Back Light Comp. |  |  | On/ Off |  |  |  |  |  |  |  |
| Video Output |  |  | 1 Vp-p/ 75 Ohms |  |  |  |  |  |  |  |
| Power Supply |  |  | DC12V±10% (optional: DC12V/ AC24V Dual) |  |  |  |  |  |  |  |
| Power Consumption | Board Lens |  | 150mA max. | 160mA max. |  |  | 160mA max. |  | 200mA max. | 160mA max. |
|  | Board Lens+IR | IR On | 430mA max. | 440mA max. |  |  | 440mA max. |  | 480mA max. | 440mA max. |
|  |  | IR Off | 150mA max. | 160mA max. |  |  | 160mA max. |  | 200mA max. | 160mA max. |
|  | Vari-Focal Lens |  | 180mA max. | 190mA max. |  |  | 190mA max. |  | 230mA max. | 190mA max. |
| Operating Temp. |  |  | -10℃ to 50℃ |  |  |  |  |  |  |  |

| BW Camera |  |  |  |  |
| --- | --- | --- | --- | --- |
| Image Device |  | 1/3“ SONY Super HAD | 1/3“ SONY Hi-Res. Super HAD |  |
| Picture Elements |  | EIA: 510 x 492 CCIR: 500 x 582 | EIA: 768 x 494 CCIR: 752 x 582 |  |
| Resolution |  | 420 TVL | 600 TVL |  |
| Min. Illumination |  | 0.1Lux/ F2.0 | 0.2Lux/ F2.0 |  |
| S/N Ratio |  | More than 48 dB |  |  |
| Electronic Shutter |  | EIA:1/60~1/100,000, CCIR:1/50~1/110,000 |  |  |
| Gamma |  | 0.45 |  |  |
| Iris Control |  | DC Drive (for Vari-Focal Lens) |  |  |
| Lens Furnished |  | Vari-Focal Lens and Board Lens |  |  |
| Gain Control |  | Hi/ Low (for Vari-Focal model) or Auto |  |  |
| Back Light Comp. |  | On/ Off |  |  |
| Video Output |  | 1 Vp-p/ 75 Ohms |  |  |
| Power Supply |  | DC12V±10% (optional: DC12V/ AC24V Dual) |  |  |
| Power Consumption | Board Lens | 140mA max. |  | 150mA max. |
|  | Vari-Focal Lens | 180mA max. |  | 200mA max. |
| Operating Temp. |  | -10℃ to 50℃ |  |  |

Note: Design and specifications are subject to change without prior notice.