Outdoor Infrared Illumination Camera

Instruction Manual

VER.:1.0, P/N.:R040138A/2

Thank you very much for purchasing our product. Before operating this product, please read this instruction manual carefully to ensure proper use.

1. Safety Precautions

|  | Disposal of Old Electrical & Electronic Equipment (Applicable in the European Union and other European countries with separate collection systems). |
| --- | --- |

- Please do not directly touch the sensor element. If necessary, use a soft cloth moistened with alcohol to wipe off any dust.

- Please be extra careful not to shake the camera.

- Please avoid places where there is direct sunlight.

- When using this camera in places where the lighting differs greatly, please use a manual iris lens with a ND filter.

- Please avoid places where temperatures exceed 50℃ or more, high humidity and/ or direct rainfall, frequent vibrations, or shocks occur.

- During the night, if a minimum brightness of 0.1 Lux cannot be achieved, install appropriate light fixtures.

- When the camera is not used, keep the lens or the lens cap attached to protect the sensor.

2. Description

The camera uses SONY high sensitive super HAD interline transfer image sensor, and employs digital signal processor (DSP) chip-set for image control, and all integrated state circuitry which provide extremely long life and high reliability. This camera offers excellent image quality with low lag and high burn resistance, and is not subject to distortions from magnetic fields.

Highly resistance to shock and vibration, easy to install, this camera is a very good choice for your color CCTV system.

3. Feature

- Built-in high quality CCD camera with high performance Infrared illuminator module.

- Built-in 84 pieces of 850nm wavelength Infrared Light-Emitting Diode.

- Light-source sensor can automatically turn on/ off the infrared illuminator by detecting the light source of environment.

- Weatherproofing suitable for indoor and outdoor use.

- AC type (optional).

4. Contents

| Item | Name of Part | Quantity |
| --- | --- | --- |
| 1 | Camera | 1 |
| 2 | Mounting Block | 1 |
| 3 | Instruction Manual | 1 |
| 4 | Appurtenance Bag | 1 |
| 5 | Fix retaining screw for bracket | 4 |

5. Installation & Operation

Fig. 1

Use the supplied screws (in the appurtenance bag), and secure the camera to the mounting block. (Fig. 1).

  - Secure the camera mounting block to the mounting bracket (purchase separately), using the screw supplied by the appurtenance bag (Fig. 2).

Fig. 2

6. Specification

| Color Camera |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Image Device |  | 1/3“ Color SONY Super HAD |  |  | 1/3“ Color SONY Hi-Res. Super HAD | 1/3“ Color Hi-Res. CCD | 1/3“ Color SONY Ultra Hi-Res. Super HAD |
| Picture Elements |  | NTSC: 510x492 PAL: 500x582 |  |  | NTSC: 768x494 PAL: 752x582 |  |  |
| Resolution |  | 380 TVL | 420 TVL |  | 470 TVL | 540 TVL | 550 TVL |
| Min. Illumination |  | 0.02 Lux/ F2.0 (Day) 0 Lux (IR On) |  |  | 0.03 Lux/ F2.0 (Day) 0 Lux (IR On) |  | 0.04Lux/ F2.0 (Day) 0 Lux (IR On) |
| S/N Ratio |  | More than 48 dB |  |  |  |  |  |
| Electronic Shutter |  | NTSC: 1/60~1/100,000 PAL: 1/50~1/110,000 |  |  |  |  |  |
| Gamma |  | 0.45 |  |  |  |  |  |
| Lens Furnished |  | Board Lens |  |  |  |  |  |
| White Balance |  | Auto |  |  |  |  |  |
| Gain Control |  | Auto |  |  |  |  |  |
| Sync. System |  | Internal |  |  |  |  |  |
| Video Output |  | 1 Vp-p / 75 Ohms |  |  |  |  |  |
| Infrared Luminary |  | 84 pieces IR-LED |  |  |  |  |  |
| Wavelength |  | 850nm |  |  |  |  |  |
| Illuminate Distance |  | 30M / 50M |  |  |  |  |  |
| Waterproofing Criterion |  | IP66 |  |  |  |  |  |
| Power Supply |  | DC12V10%, AC24V, AC100~240V (optional) |  |  |  |  |  |
| Power consumption | IR OFF | 140mA max. |  | 95mA max. | 140mA max. | 95mA max. | 170mA max. |
|  | IR ON | 1040mA max. |  | 1000mA max. | 1040mA max. | 1000mA max. | 1000mA max. |
| Operating Temp. |  | -10 to 50℃ (14 to 122℉) |  |  |  |  |  |

| B/W Camera |  |  |
| --- | --- | --- |
| Image Device | 1/3“ B/W SONY Super HAD | 1/3“ B/W SONY Hi-Res. Super HAD |
| Picture Elements | EIA: 510x492 CCIR: 500x582 | EIA: 768x494 CCIR: 752x582 |
| Resolution | 420 TVL | 600 TVL |
| Min. Illumination | 0.1 Lux/ F2.0 (Day) 0 Lux (IR On) | 0.2 Lux/ F2.0 (Day) 0 Lux (IR On) |
| S/N Ratio | More than 48 dB |  |
| Electronic Shutter | EIA:1/60s~1/100,000s; CCIR:1/50s~1/110,000s |  |
| Gamma | 0.45 |  |
| Lens Furnished | Board Lens |  |
| White Balance | Auto |  |
| Gain Control | Auto |  |
| Sync. System | Internal |  |
| Video Output | 1 Vp-p / 75 Ohms |  |
| Infrared Luminary | 84 pieces IR-LED |  |
| Wavelength | 850nm |  |
| Illuminate Distance | 30M / 50M |  |
| Waterproofing Criterion | IP66 |  |
| Power Supply | DC12V10%, AC24V, AC100~240V (optional) |  |
| Power consumption | IR On: 1000mA max. IR Off: 125mA max. |  |
| Operating Temp. | -10 to 50℃ (14 to 122℉) |  |

(Note: Design and specifications are subject to change without prior notice.)