WIDE DYNAMIC RANGE CAMERA

USER MANUAL

Contents

1. SAFETY PRECAUTIONS	2

2. INTRODUCTION	3

3. FEATURES	4

4. PACKING LIST	5

5. NAME and FUNCTION of EACH PART	6

6. INSTALLATION	8

6.1 Native Keyboard Installation	9

6.2 PELCO Keyboard (or compatible) Installation	9

7. OPERATION	10

7.1 Operating Procedure	10

7.2 Native Keyboard Operation	10

7.3 PELCO Keyboard (or compatible) Operation	11

8. SYSTEM SETUP	12

8.1 Digital Zoom Operation:	12

8.2 OSD (On Screen Display) Main Menu Description	12

9. CAMERA CONTROL PROTOCOL	19

10. SPECIFICATION	22

The author assumes no responsibility for any errors or omissions that may appear in this document nor does the author make a commitment to update the information herein.

VER 1.0, P/N: 040112/2

# 1. SAFETY PRECAUTIONS

It is advised to read the Safety Precaution Guide through carefully before operating the product, to prevent any possible danger.

WARNING: Alert the user to the presence of un-insulated “dangerous voltage”.

CAUTION: Alert the user the presence of important operating and maintenance (Servicing) instructions in the literature accompanying the appliance.

Disposal of Old Electrical & Electronic Equipment (Applicable in the European Union and other European countries with separate collection systems).

This symbol on the product or on its packaging indicates that this product shall not be treated as household waste. Instead it shall be handed over to the applicable collection point for the recycling of electrical and electronic equipment. By ensuring this product is disposed of correctly, you will help prevent potential negative consequences for the environment and  human health, which could otherwise be caused by inappropriate waste handling of this product. The recycling of materials will help to conserve natural resources. For more detailed information about recycling of this product, please contact your local city office , your household waste disposal service or the shop where you purchased the product.

Please be extra careful not to shake the product.

Please avoid places where frequent vibrations or shocks.

Do not install the product in extreme temperature conditions.
Only use the camera under conditions where temperatures are between -10℃ and +50℃. Be especially careful to provide ventilation when operating under high temperatures.

Do not install the product in an environment where the humidity is high.
Unless the product is waterproof or weatherproof, otherwise it can cause the image quality to be poor.

Never keep the product to direct strong light or sunlight.

It can damage the product.

Do not spill liquid of any kind on the product.

If it gets wet, wipe it dry immediately. Alcohol or beverage can contain minerals that corrode the electronic components.

When any abnormal occurs, make sure to unplug the unit, and contact your local dealer.

# 2. INTRODUCTION

This WDR digital image camera uses a high sensitive color 1/3” interline transfer Charge Coupled Device (CCD) image sensor, featuring approx.: NTSC = 380,000/ PAL = 440,000 effective pixels, producing pictures reaching 470 lines of horizontal resolution. Wide-dynamic range function, the ability to respond quickly and accurately to scenes or objects with high contrast and wide variations in light. Built-in with ICR module and DSS function provides excellent low lux effect. Remote control is available via RS-485/ RS-422 or RS-232 (optional) interface.

# 3. FEATURES

- High sensitivity, low smear, high anti-blooming, and high S/N ratio.

- Vertical double density and wide-dynamic range function, the ability to respond quickly and accurately to scenes or objects with high contrast and wide variations in light.

- * Mechanic IR cut-filter driving unit with AE/ Illumination Detector (sensor), provides change from color to B&W mode automatically for day and night 24-hour surveillance.

- OSD (On Screen Display) Setup Menu.

- Camera tile setup of up to 16 alphanumeric letters.

- Privacy image masking with free position.

- Camera control via either RS-422/RS485 or RS232 interface.

- PELCO (D and P)/ LG Protocol, selectable.

- Supports AES (Automatic Electronic Shutter), AI ( Auto Iris), GC (Gain Control), WB (White Balance), BLC (Back Light Compensation), and *Digital Slow Shutter (DSS).

- Provides flickerless mode, color rolling suppression, and line-lock function.

- The camera accepts 2 types of auto iris lenses Video type (Video-Drive) and DC type (DC-Drive).

- Performance: 3x digital zoom, *freeze image, positive/ negative image, mirror function (left/ right), *reverse turn (up/ down), and *180° rotation.

<Note>

* : The symbol indicates that the function is inapplicable to the economic type camera.

# 4. PACKING LIST

Check and make sure all the items shown below are included in your product package. If something is missing, contact your dealer as soon as possible.

| No. | Item | Picture | Quantity |
| --- | --- | --- | --- |
| 1 | Camera |  | 1 |
| 3 | Manual |  | 1 |
| 3 | Auto Iris Lens Plug Mounting Block Fixed Screws L-Wrench |  | 1 |

# 5. NAME and FUNCTION of EACH PART

AC24V/DC12V

Front View

Rear View

Side View

- Video Output: Composite Video 1Vp-p, 75 Ohms, BNC.

- Key Control:

| No. | Name | Function |
| --- | --- | --- |
| 1 | (T)/ | Zoom Tele or Up () |
| 2 | (W)/ | Zoom Wide or Down () |
| 3 | (+)/► | Increase Value (＋) |
| 4 | (-)/◄ | Decrease Value (－) |
| 5 | Menu | Enter or Exit Setup Menu |

- Protocol: AUTO/ NATIVE/ PELCO/ LG

- Baud Rates: 2.4K/ 4.8K/ 9.6K/ 19.2K BPS

RS-232:

| No. | Name | Function |
| --- | --- | --- |
| 1 | RX | RS-232 Interface RX |
| 2 | GND | GND |
| 3 | TX | RS-232 Interface TX |
| 4 | GND | GND |

RS-485/ RS-422:

| No. | Name | Function |
| --- | --- | --- |
| 1 | R+ | RS-485/RS-422 Interface R+ |
| 2 | R- | RS-485/RS-422 Interface R- |
| 3 | T+ | RS-485/RS-422 Interface T+ |
| 4 | T- | RS-485/RS-422 Interface T- |

# 6. INSTALLATION

Figure 6-1

Figure 6-2

Figure 6-3

## 6.1 Native Keyboard Installation

  - RS-485 interface is used for communicating native keyboard. Refer to Figure 6-3; connect R+ and T+ of the camera together; connect R+ and T+ of the camera to D+ of the native keyboard. Connect R- and T- of the camera together; connect R- and T- of the camera to D- of the native keyboard.

  - User may adjust Camera ID, Protocol, Speed and Parity should only be adjusted via rear panel keys.

| Communication | Setting |
| --- | --- |
| CAMERA ID | 1 - 127 |
| PROTOCOL | AUTO/ NATIVE |
| SPEED | 9600 |
| PARITY | NONE |

The speed of native keyboard is fixed (9600), and the adjusted function is only effective, after exiting the OSD setup menu.

## 6.2 PELCO Keyboard (or compatible) Installation

  - RS-422 interface is used for communicating PELCO keyboard. Refer to Figure

6-2; connect R+ of the camera to T+ of the PELCO keyboard. Connect R- of the camera to T- of the PELCO keyboard.

  - User may adjust Camera ID via rear panel keys or via remote commands.       Protocol, Speed and Parity should only be adjusted via rear panel keys.

| Communication | Setting |
| --- | --- |
| CAMERA ID | 0 - 253 for P protocol |
|  | 1 - 255 for D protocol |
| PROTOCOL | AUTO/ PELCO |
| SPEED | 2400/ 4800/ 9600/ 19200 |
| PARITY | NONE |

The speed of camera should be the same as the speed of the keyboard, and the adjusted function is only effective, after exiting the OSD setup menu.

<Note>

Maximum cable distance for RS-422/ RS-485 communication over 24-gauge wire is 4,000 feet (1,219 m). Recommend using shielded twisted pair cable that meets the basic requirements for EIA RS-422 / RS-485 applications.

# 7. OPERATION

## 7.1 Operating Procedure

- Mount the camera on the mounting bracket by using the hole on the top or bottom of the camera, and by using the enclosed mounting block, finished by 2 screws.

- Connect the video output to the monitor or other video device via a 75 Ohms type coaxial cable.

- Power Input Terminal (Dual Power): the camera accepts both AC 24V and DC 12 V power sources (non-polarity).

- When applying video lens, please do not adjust “VR LEVEL” too low, it will result glittering images, please adjust it appropriately to obtain the optimum image quality.

## 7.2 Native Keyboard Operation

#### Normal Display Mode

| #### Native Keyboard | #### Camera Function |
| --- | --- |
| BRIGHTNESS+ | Brightness + |
| BRIGHTNESS- | Brightness - |
| Zoom TELE | Zoom Tele |
| Zoom WIDE | Zoom Wide |
| NEAR | None |
| FAR | None |
| Move Joystick Left | None |
| Move Joystick Right | None |
| Move Joystick Up | None |
| Move Joystick Down | None |
| MENU key | Accessing OSD Main Menu |

#### OSD Setup Menu Mode

| #### Native Keyboard | #### Camera Function |
| --- | --- |
|  | Decrease (-) |
|  | Increase (+) |
|  | Cursor Up |
|  | Cursor Down |
| BRIGHTNESS+ | None |
| BRIGHTNESS- | None |
| Zoom TELE | None |
| Zoom WIDE | None |
| NEAR | None |
| FAR | None |

Please refer to the native keyboard manual for more information.

## 7.3 PELCO Keyboard (or compatible) Operation

#### Normal Display Mode

| #### PELCO Keyboard | #### Camera Function |
| --- | --- |
| OPEN | Brightness + |
| CLOSE | Brightness - |
| Twist Joystick clockwise or Zoom In | Zoom Tele |
| Twist Joystick counterclockwise or Zoom Out | Zoom Wide |
| NEAR | None |
| FAR | None |
| Move Joystick Left | None |
| Move Joystick Right | None |
| Move Joystick Up | None |
| Move Joystick Down | None |
| Enter 95; Hold the PRESET key (approximately five seconds) until the main menu appears on the screen. | Accessing OSD Main Menu |

#### OSD Setup Menu Mode

| #### PELCO Keyboard | #### Camera Function |
| --- | --- |
| OPEN | Sub Menu Enter |
| CLOSE | Sub Menu Exit |
| NEAR | Cursor Up |
| FAR | Cursor Down |
| Move Joystick Left | Decrease (-) |
| Move Joystick Right | Increase (+) |
| Move Joystick Up | Cursor Up |
| Move Joystick Down | Cursor Down |
| Twist Joystick clockwise or Zoom In | None |
| Twist Joystick counterclockwise or Zoom Out | None |

Please refer to PELCO Keyboard (or compatible) manual for more information.

# 8. SYSTEM SETUP

## 8.1 Digital Zoom Operation:

Under normal display (before entering the OSD menu), use (T)/ (W) button to control the digital zoom (zoom range: 1x ~3x). Set WDR function to “ON”, the digital zoom auto switches to 1x zoom.

## 8.2 OSD (On Screen Display) Main Menu Description

Main Menu Display

| MAIN MENU VER.xxxx |  |
| --- | --- |
| EXPOSURE WDR * DAY/NIGHT WHITE BALANCE PRIVACY MASK EFFECT DISPLAY COMMUNICATION DEFAULT EXIT |         ON  |

Main Menu Setup

- In order to display the setup menu on the screen, set the menu command or press the button panel.

- Use (T)/ (W) control buttons to select each item.

- Use ◄(－)/ ►(＋) control buttons to change the data.

- Use MENU control button to ENTER/ EXIT the menu display.

<Note>

* : The symbol indicates that the function is inapplicable to the economic type camera.

8.3 Sub Menu Description

Exposure Setup

| EXPOSURE |  |
| --- | --- |
| LENS TYPE BRIGHTNESS AGC MODE AGC MAX AGC ADJ FLICKERLESS SHUTTER SPD *DSS MAX DEFAULT RETURN | DC/ VIDEO 16 AUTO 165 NA NA NA OFF ON  |

- Lens type can be switched to DC/ VIDEO or MANUAL. When the system is set to DC/ VIDEO, please put the switch (situated on the back of the camera) in “DC” or “VIDEO” position respectively according to the type of the lens.

- Brightness setup range: 0~36.

- AGC MODE: AUTO/ MANUAL selectable.

| (1) | AGC MAX: | Set AGC MODE to AUTO (AGC MAX setup range = 0~255). When AGC MAX < 165, AE function of Day/ Night Mode becomes an ineffective function (data value “---” for D/N Mode will be shown on the display). |
| --- | --- | --- |
|  | AGC MODE: | Set to MANUAL, AGC MAX becomes an NA (Not Available) item. |
| (2) | AGC ADJ: | Set AGC MODE to AUTO, AGC ADJ becomes an NA (Not Available) item. |
|  | AGC MODE: | Set to MANUAL, AGC ADJ can be set to fixed GAIN value (0~255). |

- SHUTTER SPD mode (NTSC: 1/100 and PAL: 1/120) is fixed, when FLICKERLESS mode is set to “ON”.

- Ten Types of SHUTTER SPD Mode: AUTO, NORMAL (NTSC: 1/60 and PAL: 1/50), 1/250, 1/500, 1/1000, 1/2000, 1/4000, 1/10000, 1/20000, and 1/50000.

- AGC MODE auto switches to manual, when SHUTTER SPD is set to 1/250, 1/500, 1/1000, 1/2000, 1/4000, 1/10000, 1/20000, 1/50000 or FLICKERLESS (set to “ON”).

- Set WDR to “ON”, FLICKERLESS and SHUTTER SPD becomes an NA (Not Available) item.

- *DSS MAX: Low speed shutter control, offers optimal brightness level (The  higher the value, the higher the brightness level).

- DEFAULT: Return to the factory-default configuration.

- RETURN: Return to previous page.

<Note>

* : The symbol indicates that the function is inapplicable to the economic type camera.

WDR Setup:

| WDR |  |
| --- | --- |
| WDR MODE WDR LEVEL BLC MODE BLC LEVEL BLC AREA DEFAULT RETURN | ON 96 NA NA NA ON  |

- Set WDR MODE to “ON”, BLC MODE/ BLC LEVEL/ BLC AREA becomes an NA (Not Available) item, and the digital zoom will auto switch to 1x zoom.

- WDR LEVEL (0~255): The higher the value, the higher the brightness level.

- BLC MODE: Set WDR MODE to “OFF”, BLC MODE can be set to “ON/ OFF”.

- BLC LEVEL (0~15): The higher the value, the higher the backlight compensation level.

- Seven Types of BLC AREA Mode:

(1) CENTER (2) TOPS (3) TOPL (4) BOTTOMS (5) BOTTOML (6) LEFT (7) RIGHT

(1) CENTER             (2) TOPS                 (3) TOPL

(4) BOTTOMS           (5) BOTTOML            (6) LEFT

(7) RIGHT

*DAY/NIGHT Setup:

| DAY/NIGHT |  |
| --- | --- |
| D/N MODE DAYNIGHT NIGHTDAY SENSOR LEVEL FILTER DELAY DEFAULT RETURN AE VALUE SENSOR VALUE | AE 83 212 NA NA ON  ??? ??? |

Note:

- D/N MODE: Four types of setup (AE/ SENSOR/ DAY/ NIGHT).

- AE: Setup according to DAY→NIGHT and NIGHT→DAY (threshold value), auto switches to Color or B/ W. When AGC MAX < 165, AE function Day/ Night Mode becomes an ineffective function (data value “---” for D/N Mode will be shown on the display).

- SENSOR: Auto switches to Color or B/ W according to SENSOR LEVEL and FILTER DELAY setup.

- DAY: Fixed to Color mode.

- NIGHT: Fixed to B/ W mode.

- DAY→NIGHT: The threshold value of color change (from color to B/ W) under AE mode. Switches to B/W when “Day→Night” ≧ “AE Value”.

- NIGHT→DAY: The threshold value of color change (from B/ W to color) under AE mode. Switches to Color when “Night→Day” ≦ “AE Value”.

- SENSOR LEVEL

- Higher the value: When the environment illumination is insufficient the process of switching to B/ W will be delayed.

- Lower the value: When the environment illumination is insufficient advanced switching process from Color to B/ W will quickened.

- FILTER DELAY: The delay time of the system switching from color to B&W mode when environment illumination change occurs (Filter Delay Time: 0~15 Sec.).

- When D/N MODE is set to “AE/ DAY/ NIGHT” mode, SENSOR LEVEL and FILTER DELAY become an NA (Not Available) item.

- AE VALUE: When D/ N is set to AE, it will reflect the illumination degree of the current scene.

- SENSOR VALUE: When D/ N is set to SENSOR, it will reflect the illumination   degree of the current scene.

- The table shown below are reference values, please setup according to the current scene.

| Lens Type | Shutter Speed | WDR | DAY→NIGHT | NIGHT→DAY |
| --- | --- | --- | --- | --- |
| DC Drive Video Drive | NA/ Normal | On / Off | 83 | 212 |
| Manual Iris | NA | On | 83 | 195 |
| Manual Iris | Auto | Off | 83 | 212 |
|  | Normal |  | 83 | 212 |

<Note>

* : The symbol indicates that the function is inapplicable to the economic type camera.

WHITE BALANCE Setup:

| WHITE BALANCE |  |
| --- | --- |
| WB MODE RED GAIN BLUE GAIN PUSH AUTO DEFAULT RETURN | AUTO NA NA NA ON  |

- Five Types of WB MODE: AUTO, INDOOR, OUTDOOR, MANUAL, and PUSH AUTO.

- RED GAIN/ BLUE GAIN operates only when WB MODE is set to MANUAL, otherwise it is an NA (Not Available) item.

- When WB MODE is set to PUSH AUTO:

- PUSH AUTO ON: Enable auto tracing white balance.

- PUSH AUTO OFF: Disable auto tracing white balance.

PRIVACY MASK Setup:

| PRIVACY MASK |  |
| --- | --- |
| AREA MASK START X END X START Y END Y DEFAULT RETURN | 1 OFF 20 70 20 30 ON  |

- AREA: PRIVACY MASK (Maximum 8 mask area setups).

- MASK: Select “ON/ OFF” to enable or disable mask area.

- START X: Horizontal start position (0~180).

- END X: Horizontal end position (0~180).

- START Y: Vertical start position (0~140).

- END Y: Vertical end position (0~140).

EFFECT Setup:

| EFFECT |  |
| --- | --- |
| SHARPNESS POSI/NEGA MIRROR FREEZE SYNC V PHASE DEFAULT RETURN | 7 POSI OFF OFF INT NA ON  |

- SHARPNESS: Video sharpness level setup (0 ~15).

- POSI/ NEGA: Images can be set to POSI (Positive Image ) or NEGA (Negative Image).

- Four Types of MIRROR Mode: OFF/ HORIZONTAL/ *VERTICAL/ *ROTATE.

- *FREEZE: Set FREEZE to “ON” to enable still field image.

- SYNC: When using a DC 12V adaptor, the sync setting is set to internal sync.

- V PHASE: When L/L mode is selected, you can adjust the desired phase (0 ~ 358), otherwise it is an NA (Not Available) item.

<Note>

* : The symbol indicates that the function is inapplicable to the economic type camera.

DISPLAY Setup:

| DISPLAY |  |
| --- | --- |
| TITLE TITLE DISP TITLE POS ID DISP ID POS ZOOM DISP ZOOM POS DEFAULT RETURN | ████████ ON RIGHT DOWN ON RIGHT DOWN ON LEFT DOWN ON  |

1. TITLE :You can enter up to 16 characters.

2. Four Setup Positions (TITLE/ ID/ ZOOM):

RIGHT DOWN/ LEFT UP/ RIGHT UP/ LEFT DOWN.

COMMUNICATION Setup:

| DISPLAY |  |
| --- | --- |
| CAMERA ID PROTOCOL SPEED PARITY RETURN | 1 AUTO 9600 NONE  |

Please refer to “6. INSTALLATION” for more information on Communication.

# 9. CAMERA CONTROL PROTOCOL

Native Protocol Format

Interface Type: RS485

Version: 1.02

| Command Packet | Byte 1 | Byte 2 | Byte 3 | Byte 4 | Byte 5 | Byte 6 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Receiver ID | Transmitter ID | OP Code | Data 0 | Data 1 | Check Sum |

Definitions: (in Hexadecimal)

Keyboard ID: XX

Camera ID: YY

Check Sum: ZZ (Byte1 xor Byte2 xor Byte3 xor Byte4 xor Byte5).

Note: “xor” means Exclusive OR.

| Event | Keyboard Send | Camera Response | Remarks |
| --- | --- | --- | --- |
| Linking | step1.01 XX 00 00 00 ZZ step2.02 XX 00 00 00 ZZ step3.03 XX 00 00 00 ZZ … … step128.80 XX 00 00 00 ZZ | XX YY 00 00 00 ZZ | The camera will respond at once after keyboard sent correct device ID. |
| Button NUM+”ENTER” or “CH+” or “CH-” | step1.YY XX 4A 00 00 ZZ step3.YY XX 01 00 00 ZZ | step2.XX YY 4A 00 ## ZZ step4.XX YY 01 && $$ ZZ | “##”: Device Type “$$”: Device State |
| Button “BRIGHTNESS +” | step1.YY XX 23 02 00 ZZ step3.YY XX 23 05 00 ZZ | step2.XX YY 23 02 00 ZZ step4.XX YY 23 05 00 ZZ |  |
| Button “BRIGHTNESS –” | step1.YY XX 23 03 00 ZZ step3.YY XX 23 05 00 ZZ | step2.XX YY 23 03 00 ZZ step4.XX YY 23 05 00 ZZ |  |
| Button “ZOOM TELE” | step1.YY XX 24 01 00 ZZ step3.YY XX 24 04 00 ZZ | step2.XX YY 24 01 00 ZZ step4.XX YY 24 04 00 ZZ |  |
| Button “ZOOM WIDE” | step1.YY XX 24 00 00 ZZ step3.YY XX 24 04 00 ZZ | step2.XX YY 24 00 00 ZZ step4.XX YY 24 04 00 ZZ |  |
| Button “MENU” for call OSD | step1.YY XX 10 00 00 ZZ step3.YY XX 28 04 00 ZZ step5.YY XX 28 FF 00 ZZ | step1.YY XX 10 00 00 ZZ step3.YY XX 28 04 00 ZZ step5.YY XX 28 FF 00 ZZ | OSD MODE |
| OSD MODE Button “►” | step1.YY XX 28 03 00 ZZ step3.YY XX 28 FF 00 ZZ | step2.XX YY 28 03 00 ZZ step4.XX YY 28 FF 00 ZZ | OSD MODE |
| OSD MODE Button “◄” | step1.YY XX 28 02 00 ZZ step3.YY XX 28 FF 00 ZZ | step2.XX YY 28 02 00 ZZ step4.XX YY 28 FF 00 ZZ | OSD MODE |
| OSD MODE Button “▲” | step1.YY XX 28 00 00 ZZ step3.YY XX 28 FF 00 ZZ | step2.XX YY 28 00 00 ZZ step4.XX YY 28 FF 00 ZZ | OSD MODE |
| OSD MODE Button “▼” | step1.YY XX 28 01 00 ZZ step3.YY XX 28 FF 00 ZZ | step2.XX YY 28 01 00 ZZ step4.XX YY 28 FF 00 ZZ | OSD MODE |
| OSD MODE Button “ENTER” | step1.YY XX 10 00 00 ZZ step3.YY XX 28 06 00 ZZ | step2.XX YY 10 20 03 ZZ step4.XX YY 28 06 ** ZZ | “**”: 11 OSD off 00 OSD on |
| OSD MODE Button “ESC” | step1.YY XX 10 00 00 ZZ step3.YY XX 28 05 00 ZZ | step2.XX YY 10 20 03 ZZ step4.XX YY 28 05 ** ZZ | “**”: 11 OSD off 00 OSD on |

| DEVICE TYPE | ## |
| --- | --- |
| CAMERA | 00 |
| RECEIVER BOX | 01 |

| DEVICE STATE | $$ |
| --- | --- |
| RESERVED | BIT0 |
| RESERVED | BIT1 |
| RESERVED | BIT2 |
| OSD MENU | BIT3 |
| RESERVED | BIT4 |
| RESERVED | BIT5 |
| RESERVED | BIT6 |
| RESERVED | BIT7 |

Examples:

To control the camera device (address = 1) via keyboard (address = F0), the command string (in Hexadecimal) would be:

Zoom Tele Command

Keyboard: 01 F0 24 01 00 D4

Device:	F0 01 24 01 00 D4

Zoom Stop Command

Keyboard: 01 F0 24 04 00 D1

Device:	F0 01 24 04 00 D1

Zoom Wide Command

Keyboard: 01 F0 24 00 00 D5

Device:	F0 01 24 00 00 D5

Zoom Stop Command

Keyboard: 01 F0 24 04 00 D1

Device:	F0 01 24 04 00 D1

Brightness＋ Command

Keyboard: 01 F0 23 02 00 D0

Device:	F0 01 23 02 00 D0

Brightness Stop Command

Keyboard: 01 F0 23 05 00 D7

Device:	F0 01 23 05 00 D7

Brightness－ Command

Keyboard: 01 F0 23 03 00 D1

Device:	F0 01 23 03 00 D1

Brightness Stop Command

Keyboard: 01 F0 23 05 00 D7

Device:	F0 01 23 05 00 D7

OSD Menu Command

Keyboard: 01 F0 28 04 00 DD

Device:	F0 01 28 04 00 DD

OSD Menu Exit Command

Keyboard: 01 F0 28 05 00 DC

Device:	F0 01 28 05 11 CD

OSD Menu Enter Command

Keyboard: 01 F0 28 06 00 DF

Device:	F0 01 28 06 00 DF

# 10. SPECIFICATION

| Specification | Standard Type Camera | Economic Type Camera |
| --- | --- | --- |
| Image Device | 1/3" Color Sony Solid State Image Device CCD |  |
| A/D (Analog to Digital) Converter | 10 bit |  |
| Picture Elements | NTSC: 768 x 494 (H x V) / PAL: 752 x 582 (H x V) |  |
| Resolution | 470 TV lines |  |
| Min. Illumination | - 0.1 Lux (Day & Night on), Color 0.003 Lux (DSS) / F1.2 | 0.1Lux/ F1.2 |
| S/N Ratio | More than 48 dB (AGC OFF) |  |
| Electronic Shutter | Auto: NTSC 1/60~1/100,000s, PAL1/50~1/100,000s Normal:1/60(N),1/50(P) Manual:1/250, 1/500, 1/1000, 1/2000, 1/4000, 1/10000, 1/20000,1/50000 |  |
| Scanning System | NTSC: 525 lines / PAL: 625 lines |  |
| Flickerless Mode | ON/OFF |  |
| Iris Control | Video / DC Driver |  |
| Gain Control | Auto/manual |  |
| White Balance | Auto / Indoor / Outdoor/Manual/Push Auto |  |
| Back Light Comp. | ON/OFF (0~15 adjustable), 7 types selectable |  |
| WDR (Wide Dynamic Range) | ON/OFF (0~255 adjustable) |  |
| Day & Night (ICR), | AE /Sensor/ Day/ Night | - |
| Digital Zoom | 3 x |  |
| Camera ID | 000 ~ 255 |  |
| Camera Title | Up to 16 Character |  |
| Mirror | OFF / Horizontal / Vertical / Rotate | OFF/Horizontal |
| Freeze | ON/OFF | - |
| Positive / Negative | ON/OFF |  |
| Digital Slow Shutter (DSS) | OFF, 2~20, 40, 80, 160 FLD | - |
| Video Output | 1Vp-p / 75 Ohms |  |
| Remote Control | RS422/485 (RS232 optional) |  |
| Power Supply | AC 24V / DC 12V, 5.8W AC 100~240V, <7W |  |
| Gamma | 0.45 |  |
| Sync. System | INT / LL |  |
| Operating Temp. | -10℃ ~ 50 ℃ (14 ℉ ~ 122 ℉) |  |
| Storage Temp. | -20℃ ~ 60 ℃ (-4 ℉ ~ 140 ℉) |  |
| Operating Humidity | 0% RH ~ 70 % RH |  |
| Storage Humidity | 0% RH ~ 85 % RH |  |
| Dimensions (WxHxD) | 66 mm x 52 mm x 146mm |  |
| Weight | 0.54 kg |  |

(Note: Design and specifications are subject to change without prior notice.)