HI-RES. OSD CAMERA CCAMERA

USER MANUAL

Contents

1. SAFETY PRECAUTIONS	2

2. INTRODUCTION	3

3. FEATURES	4

4. PACKAGE LIST	5

5. NAME and FUNCTION of EACH PART	6

6. INSTALLATION	8

6.1 Native Keyboard Installation	9

6.2 PELCO Keyboard (or compatible) Installation	9

7. OPERATION	10

7.1 Operating Procedure	10

7.2 Native Keyboard Operation	10

7.3 PELCO Keyboard (or compatible) Operation	11

8. SYSTEM SETUP	12

8.1 OSD (On Screen Display) Main Menu Description	12

8.2 Sub Menu Description	13

9. CAMERA CONTROL PROTOCOL	19

10. SPECIFICATION	22

The author assumes no responsibility for any errors or omissions that may appear in this document nor does the author make a commitment to update the information herein.

VER.: 1.0, P/N: 040117/2

# 1. SAFETY PRECAUTIONS

It is advised to read the Safety Precaution Guide through carefully before operating the product, to prevent any possible danger.

WARNING: Alert the user to the presence of un-insulated “dangerous voltage”.

CAUTION: Alert the user the presence of important operating and maintenance (Servicing) instructions in the literature accompanying the appliance.

Please be extra careful not to shake the product.

Please avoid places where frequent vibrations or shocks.

Do not install the product in extreme temperature conditions.
Only use the camera under conditions where temperatures are between -10℃ and +50℃. Be especially careful to provide ventilation when operating under high temperatures.

Do not install the product in an environment where the humidity is high.
Unless the product is waterproof or weatherproof, otherwise it can cause the image quality to be poor.

Never keep the product to direct strong light or sunlight.

It can damage the product.

Do not spill liquid of any kind on the product.

If it gets wet, wipe it dry immediately. Alcohol or beverage can contain minerals that corrode the electronic components.

When any abnormal occurs, make sure to unplug the unit, and contact your local dealer.

# 2. INTRODUCTION

This digital image camera uses a high sensitive color 1/3” interline transfer Charge Coupled Device (CCD) image sensor, featuring approx.: NTSC = 380,000/ PAL = 440,000 effective pixels, producing pictures reaching 550 TVL of horizontal resolution. Built-in ICR module provides excellent low lux effect. Remote control is available via RS-485/ RS-422 or RS-232 (optional) interface.

# 3. FEATURES

- High sensitivity, low smear, high anti-blooming, and high S/N ratio.

- Mechanic IR cut-filter driving unit with AE/ Illumination Detector (sensor), provides change from color to B&W mode automatically for day and night 24-hour surveillance.

- OSD (On Screen Display) Setup Menu.

- Camera tile setup of up to 16 alphanumeric letters.

- Privacy image masking with free position.

- Camera control via either RS-422/RS485 or RS232 interface.

- Supports PELCO (D and P) Protocol/ NATIVE Protocol/ LG Protocol.

- Supports AES (Automatic Electronic Shutter), AI ( Auto Iris), GC (Gain Control), WB (White Balance), and BLC (Back Light Compensation).

- Provides flickerless mode, color rolling suppression, and line-lock function.

- The camera accepts 2 types of auto iris lenses Video type (Video-Drive) and DC type (DC-Drive).

- Performance: Positive/ negative image, mirror function (left/ right).

- Under low lux environment camera with Hi-Res.+ Ex-view function has better ability to work in low-light conditions (more sensitive to light)  than camera with Hi-Res. function.

# 4. PACKAGE LIST

Check and make sure all the items shown below are included in your product package. If something is missing, contact your dealer as soon as possible.

| No. | Item | Picture | Quantity |
| --- | --- | --- | --- |
| 1 | Camera |  | 1 |
| 2 | Manual |  | 1 |
| 3 | Auto Iris Lens Plug Mounting Block Fixed Screws L-Wrench |  | 1 |

# 5. NAME and FUNCTION of EACH PART

Side View

Front View

Rear View

- Video Output: Composite Video 1Vp-p, 75 Ohms, BNC.

- Key Control:

| No. | Name | Function |
| --- | --- | --- |
| 1 |  | Up () |
| 2 |  | Down () |
| 3 | (+)/► | Increase Value (＋) |
| 4 | (-)/◄ | Decrease Value (－) |
| 5 | Menu | Enter or Exit Setup Menu |

- Protocol: AUTO/ NATIVE/ PELCO/ LG

- Baud Rates: 2.4K/ 4.8K/ 9.6K/ 19.2K BPS

RS-232:

| No. | Name | Function |
| --- | --- | --- |
| 1 | RX | RS-232 Interface RX |
| 2 | GND | GND |
| 3 | TX | RS-232 Interface TX |
| 4 | GND | GND |

RS-485/ RS-422:

| No. | Name | Function |
| --- | --- | --- |
| 1 | R+ | RS-485/RS-422 Interface R+ |
| 2 | R- | RS-485/RS-422 Interface R- |
| 3 | T+ | RS-485/RS-422 Interface T+ |
| 4 | T- | RS-485/RS-422 Interface T- |

# 6. INSTALLATION

Figure 6-1

Figure 6-2

Figure 6-3

## 6.1 Native Keyboard Installation

- RS-485 interface is used for communicating native keyboard. Refer to Figure 6-3; connect R+ and T+ of the camera together; connect R+ and T+ of the camera to D+ of the native keyboard. Connect R- and T- of the camera together; connect R- and T- of the camera to D- of the native keyboard.

- User may adjust Camera ID, Protocol, Speed and Parity should only be adjusted via rear panel keys.

| Communication | Setting |
| --- | --- |
| CAMERA ID | 1 - 127 |
| PROTOCOL | AUTO/ NATIVE |
| SPEED | 9600 |
| PARITY | NONE |

The speed of native keyboard is fixed (9600), and the adjusted function is only effective, after exiting the OSD setup menu.

## 6.2 PELCO Keyboard (or compatible) Installation

- RS-422 interface is used for communicating PELCO keyboard. Refer to Figure 6-2; connect R+ of the camera to T+ of the PELCO keyboard. Connect R- of the camera to T- of the PELCO keyboard.

- User may adjust Camera ID via rear panel keys or via remote commands. Protocol, Speed and Parity should only be adjusted via rear panel keys.

| Communication | Setting |
| --- | --- |
| CAMERA ID | 0 - 253 for P protocol |
|  | 1 - 255 for D protocol |
| PROTOCOL | AUTO/ PELCO |
| SPEED | 2400/ 4800/ 9600/ 19200 |
| PARITY | NONE |

The speed of camera should be the same as the speed of the keyboard, and the adjusted function is only effective, after exiting the OSD setup menu.

NOTE: Maximum cable distance for RS-422/ RS-485 communication over 24-gauge wire is 4,000 feet (1,219 m). Recommend using shielded twisted pair cable that meets the basic requirements for EIA RS-422 / RS-485 applications.

# 7. OPERATION

## 7.1 Operating Procedure

- Mount the camera on the mounting bracket, secure the camera (find the hole on the top or bottom of the camera) to the bracket with the supplied mounting block and 2 screws.

- Connect the video output to the monitor or other video device via a 75 Ohms type coaxial cable.

- Power Input Terminal (Dual Power): the camera accepts both AC 24V and DC 12 V power sources (non-polarity).

## 7.2 Native Keyboard Operation

#### Normal Display Mode

| #### Native Keyboard | #### Camera Function |
| --- | --- |
| BRIGHTNESS+ | Brightness + |
| BRIGHTNESS- | Brightness - |
| Zoom TELE | None |
| Zoom WIDE | None |
| NEAR | None |
| FAR | None |
| Move Joystick Left | None |
| Move Joystick Right | None |
| Move Joystick Up | None |
| Move Joystick Down | None |
| MENU key | Accessing OSD Main Menu |

#### OSD Setup Menu Mode

| #### Native Keyboard | #### Camera Function |
| --- | --- |
|  | Decrease (-) |
|  | Increase (+) |
|  | Cursor Up |
|  | Cursor Down |
| BRIGHTNESS+ | None |
| BRIGHTNESS- | None |
| Zoom TELE | None |
| Zoom WIDE | None |
| NEAR | None |
| FAR | None |

Please refer to the native keyboard manual for more information.

## 7.3 PELCO Keyboard (or compatible) Operation

#### Normal Display Mode

| #### PELCO Keyboard | #### Camera Function |
| --- | --- |
| OPEN | Brightness + |
| CLOSE | Brightness - |
| Twist Joystick clockwise or Zoom In | None |
| Twist Joystick counterclockwise or Zoom Out | None |
| NEAR | None |
| FAR | None |
| Move Joystick Left | None |
| Move Joystick Right | None |
| Move Joystick Up | None |
| Move Joystick Down | None |
| Enter 95; Hold the PRESET key (approximately five seconds) until the main menu appears on the screen. | Accessing OSD Main Menu |

#### OSD Setup Menu Mode

| #### PELCO Keyboard | #### Camera Function |
| --- | --- |
| OPEN | Sub Menu Enter |
| CLOSE | Sub Menu Exit |
| NEAR | Cursor Up |
| FAR | Cursor Down |
| Move Joystick Left | Decrease (-) |
| Move Joystick Right | Increase (+) |
| Move Joystick Up | Cursor Up |
| Move Joystick Down | Cursor Down |
| Twist Joystick clockwise or Zoom In | None |
| Twist Joystick counterclockwise or Zoom Out | None |

Please refer to PELCO Keyboard (or compatible) manual for more information.

# 8. SYSTEM SETUP

## 8.1 OSD (On Screen Display) Main Menu Description

Main Menu Display

| MAIN MENU VER.xxxx |  |
| --- | --- |
| EXPOSURE BLC DAY/NIGHT WHITE BALANCE PRIVACY MASK EFFECT DISPLAY COMMUNICATION DEFAULT EXIT |         ON  |

Main Menu Setup

- In order to display the setup menu on the screen, set the menu command or press the button panel.

- Use /  control buttons to select each item.

- Use ◄(－)/ ►(＋) control buttons to change the data.

- Use MENU control button to ENTER/ EXIT the menu display.

## 8.2 Sub Menu Description

Exposure Setup

| EXPOSURE |  |
| --- | --- |
| LENS TYPE BRIGHTNESS AGC MODE AGC MAX AGC ADJ FLICKERLESS SHUTTER SPD DEFAULT RETURN | DC/ VIDEO 30 AUTO 100 NA OFF NORMAL ON  |

Note:

- Lens type can be switched to DC/ VIDEO or MANUAL. When the system is set to DC/ VIDEO, please put the switch (situated on the back of the camera) in “DC” or “VIDEO” position respectively according to the type of the lens used (Different lens types used will influence the image brightness, please adjust according to actual condition.).

- Brightness setup range: 0~36.

- AGC MODE: AUTO/ MANUAL selectable.

| (1) | AGC MAX: | Set AGC MODE to AUTO (AGC MAX setup range = 0~255). |
| --- | --- | --- |
|  |  | When AGC MAX < 100 is setup, AE function of Day/ Night Mode becomes an ineffective function (data value “---” for D/N Mode will be shown on the display). |
|  |  | Set to MANUAL, AGC MAX becomes an NA (Not Available) item. |
| (2) | AGC ADJ: | Set AGC MODE to AUTO, AGC ADJ becomes an NA (Not Available) item. |
| (3) | AGC MODE: | Set to MANUAL, AGC ADJ can be set to fixed GAIN value (0~255). |

- SHUTTER SPD mode (NTSC: 1/100 and PAL: 1/120) is fixed, when FLICKERLESS mode is set to “ON”.

- Nine types of SHUTTER SPD Mode: AUTO, NORMAL (NTSC: 1/60 and PAL: 1/50), 1/250, 1/500, 1/1000, 1/2000, 1/4000, 1/10000, and 1/100000.

- AGC MODE auto switches to manual, when SHUTTER SPD is set to 1/250, 1/500, 1/1000, 1/2000, 1/4000, 1/10000, 1/100000 or FLICKERLESS (set to “ON”).

- DEFAULT: Return to the factory-default configuration.

- RETURN: Return to previous page.

BLC Setup:

| BLC |  |
| --- | --- |
| BLC MODE BLC LEVEL BLC AREA DEFAULT RETURN | OFF 15 CENTER ON  |

Note:

- BLC MODE: BLC MODE can be set to “ON/ OFF”.

- BLC LEVEL (1~15): The higher the value, the higher the backlight compensation level.

- Seven Types of BLC AREA Mode:

(1) CENTER (2) TOPS (3) TOPL (4) BOTTOMS (5) BOTTOML (6) LEFT (7) RIGHT

(1) CENTER             (2) TOPS                 (3) TOPL

(4) BOTTOMS           (5) BOTTOML            (6) LEFT

(7) RIGHT

DAY/ NIGHT Setup:

| DAY/ NIGHT |  |
| --- | --- |
| D/N MODE DAYNIGHT NIGHTDAY SENSOR LEVEL FILTER DELAY DEFAULT RETURN AE VALUE SENSOR VALUE | AE 50 200 NA NA ON  ??? ??? |

Note:

- D/N MODE: Four types of setup (AE*/ SENSOR/ DAY/ NIGHT).

  - AE: Setup according to DAY→NIGHT and NIGHT→DAY (threshold value), auto switches to Color or B/ W. When AGC MAX < 100 is setup, AE function of Day/ Night Mode becomes an  ineffective function (data value “---” for D/N Mode will be shown on the display).

  - SENSOR: Auto switches to Color or B/ W according to SENSOR LEVEL and FILTER DELAY setup.

  - DAY: Fixed to Color mode.

  - NIGHT: Fixed to B/ W mode.

- DAY→NIGHT: The threshold value of color change (from color to B/ W) under AE mode. Switches to B/W when “Day→Night” ≧ “AE Value”.

- NIGHT→DAY: The threshold value of color change (from B/ W to color) under AE mode. Switches to Color when “Night→Day” ≦ “AE Value”.

- SENSOR LEVEL

  - Higher the value: When the environment illumination is insufficient the process of switching to B/ W will be delayed.

  - Lower the value: When the environment illumination is insufficient advanced switching process from Color to B/ W will quickened.

- FILTER DELAY: The delay time of the system switching from color to B&W mode when environment illumination change occurs (Filter Delay Time: 0~15 Sec.).

- When D/N MODE is set to “AE/ DAY/ NIGHT” mode, SENSOR LEVEL and FILTER DELAY become an NA (Not Available) item.

- AE VALUE: When D/ N is set to AE, it will reflect the illumination degree of the current scene.

- SENSOR VALUE: When D/ N is set to SENSOR, it will reflect the illumination   degree of the current scene.

- The table shown below are reference values, please setup according to the current scene.

| CCD Type | Lens Type | Shutter Speed | BLC | DAY→NIGHT | NIGHT→DAY |
| --- | --- | --- | --- | --- | --- |
| EX-View | DC Drive Video Drive Manual IRIS | Normal | On / Off | 50 | 200 |
| Non EX-View | DC drive |  | On / Off | 50 | 200 |
|  | Video drive |  | On / Off | 50 | 150 |
|  | Manual drive |  | On / Off | 50 | 180 |
| EX-View Non EX-View | Manual IRIS | Auto | On | 20 | 100 |
|  |  |  | Off | 20 | 110 |

WHITE BALANCE Setup:

| WHITE BALANCE |  |
| --- | --- |
| WB MODE RED GAIN BLUE GAIN PUSH AUTO DEFAULT RETURN | AUTO NA NA NA ON  |

Note:

- Five Types of WB MODE: AUTO, INDOOR, OUTDOOR, MANUAL, and PUSH AUTO.

- RED GAIN/ BLUE GAIN operates only when WB MODE is set to MANUAL, otherwise it is an NA (Not Available) item.

- When WB MODE is set to PUSH AUTO:

- PUSH AUTO ON: Enable auto tracing white balance.

- PUSH AUTO OFF: Disable auto tracing white balance.

PRIVACY MASK Setup:

| PRIVACY MASK |  |
| --- | --- |
| AREA MASK START X END X START Y END Y DEFAULT RETURN | 1 OFF 50 100 30 40 ON  |

Note:

- AREA: PRIVACY MASK (Maximum 8 mask area setups).

- MASK: Select “ON/ OFF” to enable or disable mask area.

- START X: Horizontal start position (2~225).

- END X: Horizontal end position (2~225).

- START Y: Vertical start position (0~225).

- END Y: Vertical end position (0~225).

EFFECT Setup:

| EFFECT |  |
| --- | --- |
| SHARPNESS POSI/NEGA MIRROR SYNC V PHASE DEFAULT RETURN | 10 POSI OFF INT NA ON  |

Note:

- SHARPNESS: Video sharpness level setup (0 ~15).

- POSI/ NEGA: Images can be set to POSI (Positive Image ) or NEGA (Negative Image).

- Two Types of MIRROR Mode: OFF/ ON (HORIZONTAL).

- SYNC: When using a DC 12V adaptor, the sync setting is set to internal sync.

- V PHASE: When L/L mode is selected, you can adjust the desired phase (0 ~ 358), otherwise it is an NA (Not Available) item.

DISPLAY Setup:

| DISPLAY |  |
| --- | --- |
| TITLE TITLE DISP TITLE POS ID DISP ID POS DEFAULT RETURN | ████████ ON LEFT DOWN ON RIGHT DOWN ON  |

Note:

- TITLE :You can enter up to 16 characters.

- TITLE/ ID DISP: Set to ON to display or OFF not to display.

- TITLE/ ID POS (Four ID Setup Positions):

RIGHT DOWN/ LEFT UP/ RIGHT UP/ LEFT DOWN.

COMMUNICATION Setup:

| COMMUNICATION |  |
| --- | --- |
| CAMERA ID PROTOCOL SPEED PARITY RETURN | 1 AUTO 9600 NONE  |

Note:

- Please refer to “6. INSTALLATION” for more information on Communication.

# 9. CAMERA CONTROL PROTOCOL

Native Protocol Format

Interface Type: RS485

Version: 1.02

| Command Packet | Byte 1 | Byte 2 | Byte 3 | Byte 4 | Byte 5 | Byte 6 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Receiver ID | Transmitter ID | OP Code | Data 0 | Data 1 | Check Sum |

Definitions: (in Hexadecimal)

Keyboard ID: XX

Camera ID: YY

Check Sum: ZZ (Byte1 xor Byte2 xor Byte3 xor Byte4 xor Byte5).

Note: “xor” means Exclusive OR.

| Event | Keyboard Send | Camera Response | Remarks |
| --- | --- | --- | --- |
| Linking | step1.01 XX 00 00 00 ZZ step2.02 XX 00 00 00 ZZ step3.03 XX 00 00 00 ZZ … … step128.80 XX 00 00 00 ZZ | XX YY 00 00 00 ZZ | The camera will respond at once after keyboard sent correct device ID. |
| Button NUM+”ENTER” or “CH+” or “CH-” | step1.YY XX 4A 00 00 ZZ step3.YY XX 01 00 00 ZZ | step2.XX YY 4A 00 ## ZZ step4.XX YY 01 && $$ ZZ | “##”: Device Type “$$”: Device State |
| Button “BRIGHTNESS +” | step1.YY XX 23 02 00 ZZ step3.YY XX 23 05 00 ZZ | step2.XX YY 23 02 00 ZZ step4.XX YY 23 05 00 ZZ |  |
| Button “BRIGHTNESS –” | step1.YY XX 23 03 00 ZZ step3.YY XX 23 05 00 ZZ | step2.XX YY 23 03 00 ZZ step4.XX YY 23 05 00 ZZ |  |
| Button “ZOOM TELE” | step1.YY XX 24 01 00 ZZ step3.YY XX 24 04 00 ZZ | step2.XX YY 24 01 00 ZZ step4.XX YY 24 04 00 ZZ |  |
| Button “ZOOM WIDE” | step1.YY XX 24 00 00 ZZ step3.YY XX 24 04 00 ZZ | step2.XX YY 24 00 00 ZZ step4.XX YY 24 04 00 ZZ |  |
| Button “MENU” for call OSD | step1.YY XX 10 00 00 ZZ step3.YY XX 28 04 00 ZZ step5.YY XX 28 FF 00 ZZ | step1.YY XX 10 00 00 ZZ step3.YY XX 28 04 00 ZZ step5.YY XX 28 FF 00 ZZ | OSD MODE |
| OSD MODE Button “►” | step1.YY XX 28 03 00 ZZ step3.YY XX 28 FF 00 ZZ | step2.XX YY 28 03 00 ZZ step4.XX YY 28 FF 00 ZZ | OSD MODE |
| OSD MODE Button “◄” | step1.YY XX 28 02 00 ZZ step3.YY XX 28 FF 00 ZZ | step2.XX YY 28 02 00 ZZ step4.XX YY 28 FF 00 ZZ | OSD MODE |
| OSD MODE Button “▲” | step1.YY XX 28 00 00 ZZ step3.YY XX 28 FF 00 ZZ | step2.XX YY 28 00 00 ZZ step4.XX YY 28 FF 00 ZZ | OSD MODE |
| OSD MODE Button “▼” | step1.YY XX 28 01 00 ZZ step3.YY XX 28 FF 00 ZZ | step2.XX YY 28 01 00 ZZ step4.XX YY 28 FF 00 ZZ | OSD MODE |
| OSD MODE Button “ENTER” | step1.YY XX 10 00 00 ZZ step3.YY XX 28 06 00 ZZ | step2.XX YY 10 20 03 ZZ step4.XX YY 28 06 ** ZZ | “**”: 11 OSD off 00 OSD on |
| OSD MODE Button “ESC” | step1.YY XX 10 00 00 ZZ step3.YY XX 28 05 00 ZZ | step2.XX YY 10 20 03 ZZ step4.XX YY 28 05 ** ZZ | “**”: 11 OSD off 00 OSD on |

| DEVICE TYPE | ## |
| --- | --- |
| CAMERA | 00 |
| RECEIVER BOX | 01 |

| DEVICE STATE | $$ |
| --- | --- |
| RESERVED | BIT0 |
| RESERVED | BIT1 |
| RESERVED | BIT2 |
| OSD MENU | BIT3 |
| RESERVED | BIT4 |
| RESERVED | BIT5 |
| RESERVED | BIT6 |
| RESERVED | BIT7 |

Examples:

To control the camera device (address = 1) via keyboard (address = F0), the command string (in Hexadecimal) would be:

Brightness＋ Command

Keyboard:		01 F0 23 02 00 D0

Device:		F0 01 23 02 00 D0

Brightness Stop Command

Keyboard:		01 F0 23 05 00 D7

Device:		F0 01 23 05 00 D7

Brightness－ Command

Keyboard:		01 F0 23 03 00 D1

Device:		F0 01 23 03 00 D1

Brightness Stop Command

Keyboard:		01 F0 23 05 00 D7

Device:		F0 01 23 05 00 D7

OSD Menu Command

Keyboard:		01 F0 28 04 00 DD

Device:		F0 01 28 04 00 DD

OSD Menu Exit Command

Keyboard:		01 F0 28 05 00 DC

Device:		F0 01 28 05 11 CD

OSD Menu Enter Command

Keyboard:		01 F0 28 06 00 DF

Device:		F0 01 28 06 00 DF

| Image Device | 1/3" Color Sony Solid State Image Device CCD |  |
| --- | --- | --- |
| A/ D (Analog to Digital) Converter | 10 bits |  |
| Effective Picture Elements | NTSC: 768 x 494 (H x V)/ PAL: 752 x 582 (H x V) |  |
| Horizontal Resolution | 550 TV lines |  |
| Minimum Illumination | HR_EX 0.01Lux (Day & Night on), 0.5Lux (Color) / F2.0 HR 0.05Lux (Day & Night on), 0.8Lux (Color) / F2.0 |  |
| S/N Ratio | More than 48 dB (AGC OFF) |  |
| Shutter Speed Control | Auto: NTSC 1/60~1/100,000s, PAL1/50~1/100,000s Normal: 1/60(N), 1/50(P) Manual: 1/250, 1/500, 1/1000, 1/2000, 1/4000, 1/10000, 1/100000 |  |
| Scanning System | NTSC: 525 lines/ PAL: 625 lines |  |
| Flickerless Mode | ON/ OFF |  |
| Auto Iris Control | Video/ DC Driver |  |
| Gain Control | Auto/ Manual |  |
| White Balance | Auto/ Indoor/ Outdoor/ Manual/ Push Auto |  |
| Back Light Compensation | ON/ OFF (1~15 adjustable), 7 types selectable |  |
| Day & Night (ICR) | AE/ Sensor/ Day/ Night |  |
| Camera ID | 000 ~ 255 |  |
| Camera Title | Up to 16 Characters |  |
| Mirror | ON/ OFF |  |
| Positive/ Negative | ON/OFF |  |
| Video Output | 1Vp-p/ 75 Ohms |  |
| Remote Control | RS422/ RS485 (RS232 optional) |  |
| Power Supply | AC 24V/ DC 12V | AC 100~240V |
| Power Consumption | <4.1W | <4.9W |
| Gamma Characteristic | 0.45 |  |
| Synchronous System | INT/ LL |  |
| Operating Temperature | -10℃ ~ 50 ℃ (14 ℉ ~ 122 ℉) |  |
| Storage Temperature | -20℃ ~ 60 ℃ (-4 ℉ ~ 140 ℉) |  |
| Operating Humidity | 0% RH ~ 70 % RH |  |
| Storage Humidity | 0% RH ~ 85 % RH |  |
| Dimensions (WxHxD) | 66 mm x 52 mm x 146mm |  |
| Weight | AC 24V/ DC 12V: 0.54 Kg | AC 100~240V: 0.57 Kg |
| (Note: Design and specifications are subject to change without prior notice.) |  |  |

# 10. SPECIFICATION