D7715 Mini Dome Series

Features:

- Color Aluminum Flush Mount Dome Camera

  - 1/ 4” color DSP CCD with mini fixed iris lens 3.6mm

  - 2 Lux illumination

  - 350 TV lines

  - Auto white balance control

NTSC or PAL system

D7715-L

  - Size 86 x 57mm

- Color Aluminum Flush Mount Pin-Hole Dome Camera

  - 1/ 4” color DSP CCD with pin-hole lens 5.5mm

  - 2 Lux illumination

  - 350 TV lines

  - Auto white balance control

  - NTSC or PAL system

  - Size 86 x 57mm

D7715-P

Overview:

- D7715-L

- D7715-P

DYNAC●L●R

SPECIFICATION

| ITEMS | D7715-L | D7715-P |
| --- | --- | --- |
| TYPE | COLOR |  |
| SIGNAL FORMAT | PAL/ NTSC |  |
| PICK UP DEVICE | 1/4” CCD IMAGE SENSOR |  |
| PICTURE ELEMENTS | EIA/ NTSC: 512 H x 492 V, CCIR/ PAL 512 H x 582 V |  |
| HORIZONTAL RESOLUTION | 350 Lines |  |
| MINI ILLUMINATION | 2 Lux |  |
| HORIZ. SYNC. FREQUENCY | EIA / NTSC: 15.734 KHz - CCIR / PAL: 15.625 KHz |  |
| VERTICAL FREQUENCY | EIA / NTSC: 60 Hz – CCIR / PAL: 50 Hz |  |
| CLOCK FREQUENCY | EIA: 19.0699 MHz – CCIR: 18.9375 MHz |  |
| SCANNING SYSTEM | 2:1 INTERLACE |  |
| S/N RATIO | 46 dB |  |
| ELECTRONIC SHUTTER | Up to 1/125,000 sec. |  |
| LENS | Board Lens 3.6 mm / F 2.0 | Pin-Hole 5.5 mm / F 3.5 |
| WHITE BALANCE | AUTO TRACING |  |
| BACK LIGHT COMPENSATION | NO |  |
| AUTO GAIN CONTROL | NO |  |
| GAMMA | 0.45 |  |
| VIDEO OUTPUT | 1Vp-p, 75 Ohms |  |
| POWER SUPPLY | 12VDC (+ / - 0.5V) |  |
| POWER CONSUMPTION | 90mA |  |
| DIMENSIONS Dx H mm | Φ86x57 |  |
| WEIGHT g. | 340 | 315 |
| FIXATION Screws | 3 |  |
| STORAGE TEMPERATURE | –30 to +60℃ |  |
| OPERATING TEMPERATURE | –10 to +45℃ |  |

Remark:

The camera are only operated with regulative adaptor DC 12V,

AC to DC non-regulative adaptor may cause camera damaged.

PRODUCT SELECTION

L: Board Lens

P: Pin-Hole

N: NTSC

P: PAL

D 7 7 1 5 - X1   X2

Represented by:

Worldwide Distribution and Network

Developd and Manufactured by:

DYNACOLOR, Inc.

No. 116, Jou Tz St., Neihu, Taipei 114, Taiwan, R.O.C.

Tel: 886-2-2659-8898

Fax: 886-2-2659-8868

E-mail: sales@dynacolor.com.tw

Http://www.dynacolor.com.tw