Integrated High Speed Dome Camera

Outdoor Dome

User’s Manual

Version 2.1

Preface

The information given in this manual was current when published. The company reserves the right to revise and improve its products. All specifications are subject to change without notice.

Notice

To work with the Integrated High Speed Dome Cameras, any installer or technician must have the following minimum qualifications:

- A basic knowledge of CCTV systems and components

- A basic knowledge of electrical wiring and low-voltage electrical hookups

- Have read this manual completely

Copyright

Under copyright laws, the contents of this user manual may not be copied, photocopied, translated, reproduced or reduced to any electronic medium or machine-readable format, in whole or in part, without prior written permission of the company.

Important Information

Before proceeding, please read and observe all instructions and warnings in this manual. Retain this manual with the original bill of sale for future reference and, if necessary, warranty service. When unpacking your unit, check for missing or damaged items. If any item is missing, or if damage is evident, DO NOT INSTALL OR OPERATE THIS PRODUCT. Contact your dealer for assistance.

Regulation

This device complies with Part 15 of the FCC Rules. Operation is subject to the following two conditions:

(1) this device may not cause harmful interference, and (2) this device must accept any interference received, including interference that may cause undesired operation.

This symbol on the product or on its packaging indicates that this product shall not be treated as household waste in accordance with Directive 2002/96/EC. Instead it shall be handed over to the applicable collection point for the recycling of electrical and electronic equipment. By proper waste handling of this product you ensure that it has no negative consequences for the environment and human health, which could otherwise be caused if this product is thrown into the garbage bin. The recycling of materials will help to conserve natural resources.

For more details information about recycling of this product, please contact your local city office, your household waste disposal service or the shop where you purchased the product.

Compliance is evidenced by written declaration from our suppliers, assuring that any potential trace contamination levels of restricted substances are below the maximum level set by EU Directive 2002/95/EC, or are exempted due to their application. Since conditions of use are out of our control, Dynacolor Inc. makes no warranties, expressed or implied and assumes no liability in connection with the use of this information.

Change Log

1.	Released Version

Ver. 2.1

- New Features

  - Multilanguage OSD

The camera supports multi-language OSD operation, including English, Chinese, French, German, etc.

  - Image Inverse

The displayed image can be inversed vertically and horizontally for certain occasions.

  - FW upgrade via ISP

The added ISP makes FW upgrade easier.

  - New Model: C Model; E Model: Optional

Please refer to Appendix A: Technical Specification for information about the C model.

- Revised Items

  - Backlight

|  | Ver. 1.4 | Ver. 2.1 |
| --- | --- | --- |
| Model | R/K | R/K |
| BLC Level | 0~100 | 0~30 |

  - Focus

|  | Ver. 1.4 | Ver. 2.1 |
| --- | --- | --- |
| Model | R/K Model | R/K Model |
| Focus Speed | 01~08 | 0~3 |

  - AE Mode

|  | Ver. 1.4 | Ver. 2.1 |
| --- | --- | --- |
| Model | E/F/U | C/F/U |
| Exposure Comp. | 1~15 | -10.5dB~10.5dB |

|  | Ver. 1.4 | Ver. 2.1 |
| --- | --- | --- |
| Model | R/K | R/K |
| Iris Offset | 0~99 | 0~15 |

  - WBC Mode

|  | Ver. 1.4 | Ver. 2.1 |
| --- | --- | --- |
| Model | E/F/U | C/F/U |
| R Gain & B Gain | 0~128 | 0~127 |

  - Sequence

|  | Ver. 1.4 | Ver. 2.1 |
| --- | --- | --- |
| Model | E/F/U | C/F/U |
| Sequence (Preset Pos.) | 1~256 | 1~255, End |

  - Privacy Mask

|  | Ver. 1.4 | Ver. 2.1 |
| --- | --- | --- |
| Model | R/K | R/K |
| H Center/V Center | 0~255 | 0~256 |

Cautions

- Handle the camera carefully

Do not abuse the camera. Avoid striking, shaking, etc. The camera could be damaged by improper handing or storage.

- Do not disassemble the camera

To prevent electric shock, do not remove screws or covers. There are no user serviceable parts inside. Ask a qualified service person for servicing.

- Do not block cooling holes on the bracket

This camera has a cooling fan inside. Blocking the cooling holes leads to build up of heat the camera and may cause malfunction.

- Do not operate the camera beyond the specified temperature, humidity or power source ratings

Use the camera under conditions where temperature is between -30°C ~ 45°C (-22°F ~ 104°F), and relative humidity is below 90%.

- Do not use strong or abrasive detergents when cleaning the camera body

Use a dry cloth to clean the camera when dirty. In case the dirt is hard to remove, use a mild detergent and wipe gently.

- Never face the camera towards the sun

Do not aim the camera at bright objects. Whether the camera is in use or not, never aim it at the sun or other extremely bright objects. Otherwise, the camera may be smeared or damaged.

Contents

1. 	Overview	7

1.1	Product Features	8

1.2	Product Application	10

2.	Connecting the High Speed Dome	11

2.1	Package Content	11

2.2	Switch Definition	12

2.3	Communication Switch Setting	12

2.4	Dome ID Setting	13

2.5	Dome Control Protocol	14

2.6	22-Pin Connector Definition	15

2.7	RS-485 Connector	16

3.	Operation and Configuration	17

3.1	OSD Display Format	17

3.2	OSD Menu Tree	18

3.2.1	C/F/U Model	18

3.2.2	R/K Model	20

3.3	Configuration Menu	23

3.3.1	LANGUAGE	23

3.3.2	DEFAULT CAMERA	24

3.3.3	BACKLIGHT	24

3.3.4	FOCUS	25

3.3.5	AE MODE	26

3.3.6	WBC MODE	28

3.3.7	SETUP MENU 1	30

	ZOOM SPEED	31

	DIGITAL ZOOM	31

	SLOW SHUTTER	31

	IMAGE INVERSE (F/K/R/U Model)	32

	APERTURE	32

	EXIT	33

3.3.8	SETUP MENU 2	33

	FLIP	33

	ANGLE ADJUSTER	34

	SPEED BY ZOOM	34

	AUTO CALI. (Auto Calibration)	34

	SYSTEM RESET	34

	EXIT	35

3.3.9	ID DISPLAY	35

3.3.10	TITLE DISPLAY	35

3.3.11	TITLE SETTING	36

3.3.12	PRESET	37

3.3.13	SEQUENCE	37

3.3.14	AUTOPAN	39

3.3.15	CRUISE	40

3.3.16	HOME SETTING	42

3.3.17	IR FUNCTION (Removable IR Cut)	43

3.3.18	ALARM SETTING	45

3.3.19	ALARM DETECT (C/F/U Model Only)	47

3.3.20	WDR FUNCTION (K Model Only)	49

3.3.21	PRIVACY MASK	50

3.3.22	TIME SETTING	54

3.3.23	SCHEDULE	55

3.3.24	EXIT OSD	56

Appendix A: Technical Specification	57

OSD Menu Notes	58

<C/F/U Model>	58

<R/K Model>	61

# 1. 	Overview

The dome camera is a new weather resistant integrated high speed dome camera designed to apply to both indoor and outdoor installations. The latest dome camera possesses the additional functions of Image Inverse and FW upgrade via ISP and multi-language (up to 10 languages) OSD operation. The dome camera also supports one cabling for easy installation and can be integrated with CCTV products, such as Digital Video Recorders (DVRs), Control Keyboards and CCTV accessories for a total surveillance solution.

The Integrated High Speed Dome Camera provides five models of new generation advanced DSP color camera:

- U Model: 26× optical zoom multiply 12× digital magnifier

- K Model: 23× optical zoom multiply 12× digital magnifier

- R Model: 22× optical zoom multiply 12× digital magnifier

- F Model: 18× optical zoom multiply 12× digital magnifier

- C Model: 18× optical zoom multiply 12× digital magnifier

The dome delivers up to 312x zoom ratio to capture clear image in the distance. Continuous Auto Focus, Back Light Compensation, Auto Exposure and Digital Slow Shutter functions are provided for clear and high quality image. IR cut filter removable ensures 24 hours operation, while Privacy Masks are specially designed to avoid any intrusive monitoring at specific region; all of the salient functions can be incorporated to meet your needs. The Home function allows user to specify a preset position as the ‘home position’ or home functions (Sequence/Auto-pan/Cruise). Under the model, dome cameras can come back to the preset home position or functions when the camera has been idle for a user-defined period of time. Additionally, the unique Scheduling function enables users to program a preset point or function (Sequence/Auto-pan/Cruise) so that these actions can be automatically performed in certain period of time.

The dome provides variable pan/tilt speeds ranging from a fast patrol of 400° per second to a slow ramble of 5° per second with 0.225° pan accuracy for fast and accurate tracking ability. The 360° endless rotation and -10°~190° tilt travel make tracking the object passing directly beneath the dome. Maximum 256 preset points can be programmed for precise location of target areas, and users can also define 8 sequence, 4 auto-pan and 1 cruise routes for the camera to operate automatically. In addition, RS-485 communication port is available for remote control purposes.

The Integrated High Speed Dome Camera provides 8 alarm inputs and 1 alarm output, and the smart alarm management mechanism can be programmed through the OSD setup menu; certain function (Preset/Sequence/Auto-Pan/Cruise) can be activated when an alarm is triggered.

Large set of built-in protocols provide connectivity to other surveillance systems. The built-in protocols include DynaColor, Pelco, VCL, Philips, AD-422 (Manchester), etc, which allow the Integrated High Speed Dome Camera series to be integrated with other suppliers' surveillance systems.

Dependability and ultra high reliability are key factors in the speed dome’s design cycle. Every speed dome is assembled with meticulous care and thorough testing at our ISO 9001 compliant factory. High performance, reliability and reasonably pricing make this speed dome to be an ideal solution to users’ tough surveillance requirement.

## 1.1	Product Features

Precise and Accurate Tracking

  - Auto Calibration

  - Home Function

  - Scheduling Function

  - Pan driver accuracy of 0.225°

  - Preset speed up to 400°/sec.

  - Pan & Tilt proportional to Zoom Ratio

  - 256 Preset Position/8 Sequence /4 Auto-Pan /1 Cruise

Low-Light Applications

  - Removable IR Cut Filter (F, K, U Model)

  - Minimum illumination 0.01 Lux

  - Digital Slow Shutter

  - Electronic Shutter

Perfect Contrast Solution for High Image Quality

  - Wide Dynamic Range (K Model)

  - Auto White Balance

  - Auto Gain Control

  - Backlight Compensation

  - Auto Iris Control

Multiple Built-in Protocols Enhanced High Compatibility

  - DynaColor

  - Pelco D & P

  - VCL

  - Philips

  - AD/AD-422

  - Chiper

Advanced Speed Dome Functions

  - Up to 24 programmable privacy zones for camera view

  - Digital Image Flip

  - New Image Inverse

  - New Built-In Multilanguage OSD

  - New Easy FW upgrade via ISP

Privacy Mask for Privacy Protection

  - Up to 24 privacy zones of camera view programmable

Dynamic Dome Configuration

  - Flexible In/Outdoor mountings

  - Compact lightweight design for easy installation

  - Weather resistant housing for temperature, sun ray, and rain

Integrated with Web, Enhanced Internet Capability (Optional)

  - Remote monitoring operation/system configuration/software upgrade

  - Include Window active applications

## 1.2	Product Application

Connect the dome camera to other devices as shown in the diagram to complete a video surveillance solution.

NOTE: To extend the network distance up to 1.2 km (4000 feet) and to protect the connected devices, it is highly recommended to place a repeater in the mid-point. However, a repeater may be needed in the network distance less than 1.2 km if the used cables are not the CAT 5, 24-gauge cables (see 2.7 RS-485 Connector Definition). Refer to the repeater’s manual for detailed information.

# Connecting the High Speed Dome

Please refer to the following sections to connect, set and operate the dome camera. In order to control the integrated high speed dome, basically a control keyboard or other control device is required.

## 2.1	Package Content

Before proceeding, please check the box contains the items listed here. If any item is missing or has defects, DO NOT install or operate the product and contact your dealer for assistance.

| Dome Body | 50-cm Data Cable for Power Supply, Video and RS-485 | Optical Cover |
| --- | --- | --- |
| Lubricant | 50-cm, 13-pin Alarm Cable | M3×8 Screws, M5×8 Screw |
| Quick Guide | CD: User’s Manual & Installation Guide |  |

## 2.2	Switch Definition

First of all, configuring the dome ID and communication protocol is required before connecting the dome camera to other devices. The switches used for configuring these settings are located on the bottom of the dome camera.

| A | Reserved |
| --- | --- |
| B | Communication Switch |
| C | Dome ID Switch |
| D | Dome Control Protocol Switch |
| E | RJ-45 Connector (for IP dome only) |
| F | 22-Pin Connector |
| G | ISP Connector (for FW upgrade) |

## 2.3	Communication Switch Setting

The table below shows the function of each pin within the Communication Switch.

| Communication Switch | SW 1 | RS-485 Setting |
| --- | --- | --- |
|  | SW 2 |  |
|  | SW 3 | Termination |
|  | SW 4 | Line Lock |
|  | SW 5 | System Initialization |
|  | SW 6 | Reserved |

RS-485 is the interface that communicates the dome camera and its control device; for this reason, the RS-485 setup of the dome and the control device must be the same. The RS-485 default setting is half-duplex (see the diagram follows). Please do not change the default setting without qualified specialist or supplier’s notice. As for the SW 3 and SW 4, they are used for termination and Line Lock adjustment respectively. The SW 5 is mainly used when users want to set the camera to the factory default status; moreover, once firmware upgrade is carried out, users also need to adjust the SW 5 afterward.

| RS-485 Setting |  |
| --- | --- |
| Half-duplex | Full-duplex |

## 2.4	Dome ID Setting

Please change the dome ID if there is more than one dome on the same installation site. Use the switch to change your speed dome ID by turning the arrow to the desired number respectively. For instance, if the dome ID is 123, the ID switch should be set as below.

NOTE: No two domes should be given the same ID, or communication conflict may occur.

Centesimal Digit  Decimal Digit   Single Digit

NOTE: The number “0” should locate upwards as shown in above diagram for correct switch definition.

## 2.5	Dome Control Protocol

Protocol is a specific set of rules, procedures used for data communications. Basing on the devices of your surveillance system and define the protocol you are going to use. Generally, use one protocol even the devices are provided from different manufacturers. Use the switch to set your dome control protocol and the baud rate. Refer to below table and turn the arrow to choose a protocol for your speed dome.

| Switch No. | Protocol | Baud Rate |
| --- | --- | --- |
| 00 | VCL | 9600 |
| 01 | Pelco D | 2400 |
| 02 | Pelco P | 4800 |
| 04 | Chiper | 9600 |
| 05 | Philips | 9600 |
| 07 | DSCP | 9600 |
| 08 | AD422 | 4800 |
| 09 | DM P | 9600 |
| 11 | Pelco D | 4800 |
| 12 | Pelco D | 9600 |
| 13 | Pelco P | 2400 |
| 14 | Pelco P | 9600 |
| 15 | JVC | 9600 |
| 21 | Kalatel-485 | 9600 |
| 22 | Kalatel-422 | 4800 |

Select protocol: Pelco D, with switch no. 01 and baud rate 2400, for instance, the protocol switch should be set as below.

Decimal Digit   Single Digit

NOTE: The number “0” should locate upwards as shown in above diagram for correct switch definition.

## 2.6	22-Pin Connector Definition

A 50-cm data cable (shown as the figure below) is shipped with the integrated high speed dome for quick installation for demo or testing usage. Additionaly, the section will also provide the definition of each pin within the 22-pin connector on the data cable. For more information about RS-485 connector, see 2.7 RS-485 Connector Definition.

|  | Power Input | 3-Pin Terminal Block |
| --- | --- | --- |
|  | Video Output | BNC connector |
|  | RS485 Connector | 2 wires |
|  |  | 4 wires |

NOTE: When wiring the power cable, make sure the G/Y wire (Ground) inserted into the mid-pin of the terminal block.

The 22-pin connector definition is listed as below.

| Pin | Definition | Cable |
| --- | --- | --- |
| 1 | AC 24-1/DC GND | 20AWG |
| 2 | ALM NC |  |
| 3 | AC 24-2/DC 12(+) | 20AWG |
| 4 | ALM NO |  |
| 5 | FG | 20AWG |
| 6 | ALM COM |  |
| 7 | T+ | 24AWG |
| 8 | R- |  |
| 9 | T- |  |
| 10 | R+ |  |
| 11 | ISOG |  |
| 12~19 | ALM-1~ALM-8 |  |
| 20 | ALM GND |  |
| 21 | VGND | 24AWG |
| 22 | Video |  |

## 2.7	RS-485 Connector

RS-485 is the interface that communicates the dome camera and its control device. Please connect the control keyboard to the speed dome through the terminal block. The recommended cables for RS-485 communication are CAT 5 cables; maximum cable length for over 24-gauge wire is 4000 feet (1219 meters). If the total cable length exceeds 4000 feet, using a repeater to maintain the signals is recommended. Please refer to the figure and table below for pin defination and wiring.

| Pin | Corresponding Pins (22-Pin Connector) | Definition |
| --- | --- | --- |
| 1 | 7,10 | T+, R+ (D+) |
| 2 | Reserved |  |
| 3 | Reserved |  |
| 4 | Reservied |  |
| 5 | 8,9 | T-, R- (D-) |

# 3.	Operation and Configuration

## 3.1	OSD Display Format

The information shown on the screen are described in terms of OSD display,  position and function description in the table below.

| Position | Function | OSD Display | Description |
| --- | --- | --- | --- |
| 1 | Focus Modes | A | Auto Focus Mode |
|  |  | M | Manual Focus Mode |
| 2 | Backlight | X | Back Light Compensation OFF |
|  |  | B | Back Light Compensation ON |
| 3 | Alarm | ALARM | Alarm Message |
| 4 | Zoom Ratio | ×1 | Present Zoom Ratio (Optical Zoom/Digital Zoom) |
| 5 | Title | - Maximum 20 characters for each title. - 16 sets of title are available. |  |
| 6 | Camera ID | Show the camera ID |  |

## 3.2	OSD Menu Tree

The OSD setup menu structure of C/F/U and R/K model are listed seperately in the following section. Ths star symbol indicates the factory default.

For detailed function description, please see 3.3 Configuration Menu.

### C/F/U Model

| Item | Layer 1 |  | Layer 2 | Layer 3 | Default |
| --- | --- | --- | --- | --- | --- |
| LANGUAGE | <ENGLISH>, <CHINESE>, <FRENCH>, <GERMAN>, <ITALIAN>, <JAPANESE>, <POLISH>, <PORTUGUESE>, <RUSSIAN>, <SPANISH> |  |  |  | ENGLISH |
| DEFAULT CAMERA | <ON>, <OFF> |  |  |  | ON |
| BACKLIGHT | <ON>, <OFF> |  |  |  | OFF |
| FOCUS | AUTO |  | AF MODE <NORMAL>, <INTERVAL>, <ZOOM TRIG> |  | NORMAL |
|  | MANUAL |  | FOCUS SPEED <01>~<08> |  |  |
| AE MODE | EXPODURE COMP. |  | <OFF>, EXPOSURE VALUE: <-10.5dB> ~ <10.5dB> |  | OFF |
|  | AE MODE | AUTO |  |  |  |
|  |  | BRIGHT |  | BRIGHT VALUE <00> ~ <31> |  |
|  |  | SHUTTER |  | SHUTTER SPEED <1> ~ <1/10000> SEC. |  |
|  |  | IRIS |  | IRIS VALUE <CLOSE>, <F1.6> ~ <F28> |  |
|  |  | MANUAL |  | BRIGHT VALUE: AUTO |  |
|  |  |  |  | SHUTTER SPEED <1/10000> ~ <1> |  |
|  |  |  |  | IRIS VALUE <F1.6> ~ <F28> |  |
|  |  |  |  | GAIN VALUE <-3>dB ~ <28>dB |  |
|  | EXIT |  | YES |  |  |
| WBC MODE | AUTO (Auto White Balance) |  |  |  | ☆ |
|  | INDOOR |  |  |  |  |
|  | OUTDOOR |  |  |  |  |
|  | ATW (Auto-tracing WBC) |  |  |  |  |
|  | MANUAL |  | R GAIN <000> ~ <127> |  |  |
|  |  |  | B GAIN <000> ~ <127> |  |  |
| SETUP MENU 1 | ZOOM SPEED |  | <1> ~ <8> |  | 8 |
|  | DIGITAL ZOOM |  | <ON>, <OFF> |  | ON |
|  | SLOW SHUTTER (F/U Model only) |  | <ON>, <OFF> |  | OFF |
|  | IMAGE INVERSE (F/U Model only) |  | <ON>, <OFF> |  | OFF |
|  | APERTURE |  | <01> ~ <16> |  | 11 |
|  | EXIT |  | YES |  |  |
| SETUP MENU 2 | FLIP |  | C Model: <OFF>, <M.E.> F/U Model: <OFF>, <M.E.>, <IMAGE> |  | OFF |
|  | ANGLE ADJUSTER |  | MIN ANGLE<-10 ~ +10 DEG>, |  | 0 |
|  |  |  | MAX ANGLE <080 ~ 100 DEG> |  | 90 |
|  | SPEED BY ZOOM |  | <ON>, <OFF> |  | OFF |
|  | AUTO CALI. |  | <ON>, <OFF> |  | OFF |
|  | SYSTEM RESET |  | YES |  |  |
|  | EXIT |  | YES |  |  |
| ID DISPLAY | <ON>, <OFF> |  |  |  | ON |
| TITLE DISPLAY | <ON>, <OFF> |  |  |  | OFF |
| TITLE SETTING | <01> ~ <16> |  |  |  | 01 |
| PRESET | PRESET SET |  | <001>~<256> |  | ENTER |
|  | PRESET RUN |  | <001>~<256> |  | ENTER |
|  | EXIT |  | YES |  | ENTER |
| SEQUENCE | SEQUENCE LINE |  | <1> ~ <8> |  | 1 |
|  | SEQUENCE POINT |  | <01> ~ <32> |  | 01 |
|  | PRESET POS. |  | <001> ~ <255>, <END> |  | 001 |
|  | SPEED |  | <01> ~ <15> |  | 01 |
|  | DWELL TIME |  | <000> ~ <127> SEC. |  | 000 |
|  | RUN SEQUENCE |  | ENTER |  |  |
|  | EXIT |  | YES |  |  |
| AUTOPAN | AUTOPAN LINE |  | <1> ~ <4> |  | 1 |
|  | START POINT |  | <TO FIND>, <TO SAVE> |  |  |
|  | END POINT |  | <TO FIND>, <TO SAVE> |  |  |
|  | DIRECTION |  | <RIGHT>, <LEFT> |  | RIGHT |
|  | SPEED |  | <01> ~ <04> |  | 01 |
|  | RUN AUTOPAN |  | ENTER |  |  |
|  | EXIT |  | YES |  |  |
| CRUISE | RECORD START |  | ENTER |  |  |
|  | RECORD END |  | ENTER |  |  |
|  | RUN CRUISE |  | ENTER |  |  |
|  | EXIT |  | YES |  |  |
| HOME SETTING | HOME FUNCTION |  | <ON>, <OFF> |  | OFF |
|  | SELECT MODE |  | <PRESET>, <SEQUENCE>, <AUTOPAN>, <CRUISE> |  | PRESET |
|  | PRESET POINT SEQUENCE LINE AUTOPAN LINE CRUISE LINE |  | <001> ~ <256> |  | 001 |
|  |  |  | <1> ~ <8> |  | 1 |
|  |  |  | <1> ~ <4> |  | 1 |
|  |  |  | <1> |  | 1 |
|  | RETURN TIME |  | <1> ~ <128> MIN. |  | 1 |
|  | GO |  | ENTER |  |  |
|  | EXIT |  | YES |  |  |
| IR FUNCTION (F/U Model only) | <AUTO>, <ON> |  |  |  | AUTO |
| ALARM SETTNG | ALARM PIN |  | <1> ~ <8> |  | 1 |
|  | ALARM SWITCH |  | <ON>, <OFF> |  | OFF |
|  | ALARM TYPE |  | <NO> (Normal Open), <NC> (Normal Close) |  | N.C. |
|  | ALARM ACTION |  | <PRESET>, <SEQUENCE>, <AUTOPAN>, <CRUISE> |  | PRESET |
|  | PRESET POINT SEQUENCE LINE AUTOPAN LINE CRUISE LINE |  | <001> ~ <256> |  | 001 |
|  |  |  | <1> ~ <8> |  | 1 |
|  |  |  | <1> ~ <4> |  | 1 |
|  |  |  | 1 |  | 1 |
|  | DWELL TIME |  | <001> ~ <127> Sec., <ALWAYS> |  | ALWAYS |
|  | EXIT |  | YES |  |  |
| ALARM DETECT | DETECT SWITCH |  | <ON>, <OFF> |  | OFF |
|  | DETECT MODE |  | <INT FOCUS>, <FIX FOCUS>, <INT AE>, <FIX AE> |  | INT FOCUS |
|  | EXIT |  | YES |  |  |
| PRIVACY MASK | PRIVACY SWITCH | <ON>, <OFF> |  |  | OFF |
|  | TRANSPARENCY | <ON>, <OFF> |  |  | OFF |
|  | COLOR | <BLACK>, <LI. GRAY>, < GRAY>, <WHITE>, <RED>, <GREEN>, <BLUE>, <CYAN>, <YELLOW>, <MAGENTA> |  |  | BLACK |
|  | SET MASK | <01> ~ <24> |  | H CENTER: L/R |  |
|  |  |  |  | V CENTER: D/U |  |
|  |  |  |  | H SIZE <000> ~ <080> |  |
|  |  |  |  | V SIZE <000> ~ <060> |  |
|  |  |  |  | EXIT + SAVE |  |
|  | CLEAR MASK | <01> ~ <24> |  |  |  |
|  | EXIT | YES |  |  |  |
| TIME SETTING | TIME DISPLAY |  | <ON>, <OFF> |  |  |
|  | SET YEAR |  | <00> ~ <99> |  |  |
|  | SET MONTH |  | <01> ~ <12> |  |  |
|  | SET DAY |  | <00> ~ <31> |  |  |
|  | SET HOUR |  | <00> ~ <23> |  |  |
|  | SET MINUTE |  | <00> ~ <59> |  |  |
|  | EXIT+SAVE |  |  |  |  |
| SCHEDULE | SWITCH |  | <ON>, <OFF> |  |  |
|  | POINT |  | <01> ~ <32> |  | 01 |
|  | HOUR |  | <00> ~ <23> |  | 00 |
|  | MINUTE |  | <00> ~ <59> |  | 00 |
|  | MODE |  | NONE | NO FUNCTION | ☆ |
|  |  |  | PRESET | PRESET POINT <001> ~ <256> |  |
|  |  |  | SEQUENCE | SEQUENCE LINE <1> ~ <8> |  |
|  |  |  | AUTOPAN | AUTOPAN LINE <1> ~ <4> |  |
|  |  |  | CRUISE | CRUISE LINE <1> |  |
|  |  |  | IR FUNC. | IR FUNCTION AUTO |  |
|  | SCHEDULE RESET |  | YES |  |  |
|  | EXIT | YES |  |  |  |
| EXIT OSD | YES |  |  |  |  |

### R/K Model

| Item | Layer 1 | Layer 2 | Layer 3 | Default |
| --- | --- | --- | --- | --- |
| LANGUAGE | <ENGLISH>, <CHINESE>, <FRENCH>, <GERMAN>, <ITALIAN>, <JAPANESE>, <POLISH>, <PORTUGUESE>, <RUSSIAN>, <SPANISH> |  |  | ENGLISH |
| DEFAULT CAMERA | <ON>, <OFF> |  |  | ON |
| BACKLIGHT | <ON> | BLC LEVEL <00> ~ <30> |  | OFF |
|  | <OFF> |  |  |  |
| FOCUS | AUTO | TUNING VALUE <1CM>, <10CM>, <30CM>, <1M> |  | 10CM |
|  | MANUAL | FOCUS SPEED <0> ~ <3> |  |  |
| AE MODE | AUTO | IRIS OFFSET <00> ~ <15> |  |  |
|  | SHUTTER | SHUTTER SPEED K Model:<1/30000> ~ <1/2> R Model:<1/30000> ~ <1/50> (PAL); <1/30000> ~ <1/60> (NTSC) |  |  |
|  | IRIS | <00> ~ <09> |  |  |
|  | AGC | <00> ~ <05> |  |  |
| WBC MODE | AUTO (Auto White Balance) |  |  | ☆ |
|  | MANUAL | R GAIN <00> ~ <99> B GAIN <00> ~ <99> |  |  |
| SETUP MENU 1 | ZOOM SPEED | <FAST>, <SLOW> |  | FAST |
|  | DIGITAL ZOOM | <OFF>, <02> ~ <12> |  | OFF |
|  | SLOW SHUTTER (K Model only) | <1/2> ~ <1/60> (NTSC) |  | 1/30 |
|  |  | <1/1.5> ~ <1/50> (PAL) |  | 1/25 |
|  | IMAGE INVERSE | <ON>, <OFF> |  | OFF |
|  | APERTURE | <AUTO> |  | ☆ |
|  |  | <MANUAL> | H APERTURE <00> ~ <31> V APERTURE <00> ~ <31> |  |
|  | EXIT | YES |  |  |
| SETUP MENU 2 | FLIP | <OFF>, <M.E.>, <IMAGE>(K Model only) |  | OFF |
|  | ANGLE ADJUSTER | MIN ANGLE<-10 ~ +10 DEG>, |  | 0 |
|  |  | MAX ANGLE <080 ~ 100 DEG> |  | 90 |
|  | SPEED BY ZOOM | <ON>, <OFF> |  | OFF |
|  | AUTO CALI. | <ON>, <OFF> |  | OFF |
|  | SYSTEM RESET | YES |  |  |
|  | EXIT | YES |  |  |
| ID DISPLAY | <ON>, <OFF> |  |  | ON |
| TITLE DISPLAY | <ON>, <OFF> |  |  | OFF |
| TITLE SETTING | <01> ~ <16> |  |  | 01 |
| PRESET | PRESET SET | <001>~<256> |  | ENTER |
|  | PRESET RUN | <001>~<256> |  | ENTER |
|  | EXIT | YES |  | ENTER |
| SEQUENCE | SEQUENCE LINE | <1> ~ <8> |  | 1 |
|  | SEQUENCE POINT | <01> ~ <32> |  | 01 |
|  | PRESET POS. | <001> ~ <255>, <END> |  | 001 |
|  | SPEED | <01> ~ <15> |  | 01 |
|  | DWELL TIME | <000> ~ <127> SEC. |  | 000 |
|  | RUN SEQUENCE | ENTER |  |  |
|  | EXIT | YES |  |  |
| AUTOPAN | AUTOPAN LINE | <1> ~ <4> |  | 1 |
|  | START POINT | <TO FIND>, <TO SAVE> |  |  |
|  | END POINT | <TO FIND>, <TO SAVE> |  |  |
|  | DIRECTION | <RIGHT>, <LEFT> |  | RIGHT |
|  | SPEED | <01> ~ <04> |  | 01 |
|  | RUN AUTOPAN | ENTER |  |  |
|  | EXIT | YES |  |  |
| CRUISE | RECORD START | ENTER |  |  |
|  | RECORD END | ENTER |  |  |
|  | RUN CRUISE | ENTER |  |  |
|  | EXIT | YES |  |  |
| HOME SETTING | HOME FUNCTION | <ON>, <OFF> |  | OFF |
|  | SELECT MODE | <PRESET>, <SEQUENCE>, <AUTOPAN>, <CRUISE> |  | PRESET |
|  | PRESET POINT SEQUENCE LINE AUTOPAN LINE CRUISE LINE | <001> ~ <256> |  | 001 |
|  |  | <1> ~ <8> |  | 1 |
|  |  | <1> ~ <4> |  | 1 |
|  |  | <1> |  | 1 |
|  | RETURN TIME | <1> ~ <128> MIN. |  | 1 |
|  | GO | ENTER |  |  |
|  | EXIT | YES |  |  |
| IR FUNCTION (K Model only) | <AUTO>, <ON> | THREADHOLD <MID>, <HI>, <LOW> IR COLOR <B/W>, <COLOR> EXIT <YES> |  | AUTO |
| ALARM SETTNG | ALARM PIN | <1> ~ <8> |  | 1 |
|  | ALARM SWITCH | <ON>, <OFF> |  | OFF |
|  | ALARM TYPE | <NO> (Normal Open), <NC> (Normal Close) |  | N.C. |
|  | ALARM ACTION | <PRESET>, <SEQUENCE>, <AUTOPAN>, <CRUISE> |  | PRESET |
|  | PRESET POINT SEQUENCE LINE AUTOPAN LINE CRUISE LINE | <001> ~ <256> |  | 001 |
|  |  | <1> ~ <8> |  | 1 |
|  |  | <1> ~ <4> |  | 1 |
|  |  | 1 |  | 1 |
|  | DWELL TIME | <001> ~ <127> Sec., <ALWAYS> |  | ALWAYS |
|  | EXIT | YES |  |  |
| WDR FUNCTION (K Model only) | <ON> | RATIO LEVEL <000> ~ <128> SHUTTER SPEED <000> ~ <128> IRIS OFFSET <000> ~ <128> EXIT <YES> |  |  |
|  | <OFF> |  |  | ☆ |
| PRIVACY MASK (K Model only) | PRIVACY SWITCH | <ON>, <OFF> |  | OFF |
|  | MASK SHADE | <GRAY>, <WHITE>, <BLACK> |  | BLACK |
|  | SET MASK | <01> ~ <08> | H CENTER <000> ~ <255> |  |
|  |  |  | V CENTER <000> ~ <255> |  |
|  |  |  | H SIZE <000> ~ <127> |  |
|  |  |  | V SIZE <000> ~ <127> |  |
|  |  |  | EXIT + SAVE |  |
|  | CLEAR MASK | <01> ~ <08>, <RESET> |  | 01 |
|  | MASK DISPLAY | <FIRST>, <LAST> |  | FIRST |
|  | EXIT | YES |  |  |
| TIME SETTING | TIME DISPLAY | <ON>, <OFF> |  |  |
|  | SET YEAR | <00> ~ <99> |  |  |
|  | SET MONTH | <01> ~ <12> |  |  |
|  | SET DAY | <00> ~ <31> |  |  |
|  | SET HOUR | <00> ~ <23> |  |  |
|  | SET MINUTE | <00> ~ <59> |  |  |
|  | EXIT+SAVE |  |  |  |
| SCHEDULE | SWITCH | <ON>, <OFF> |  |  |
|  | POINT | <01> ~ <32> |  | 01 |
|  | HOUR | <00> ~ <23> |  | 00 |
|  | MINUTE | <00> ~ <59> |  | 00 |
|  | MODE | NONE | NO FUNCTION | ☆ |
|  |  | PRESET | PRESET POINT <001> ~ <256> |  |
|  |  | SEQUENCE | SEQUENCE LINE <1> ~ <8> |  |
|  |  | AUTOPAN | AUTOPAN LINE <1> ~ <4> |  |
|  |  | CRUISE | CRUISE LINE <1> |  |
|  |  | IR FUNC. | IR FUNCTION AUTO |  |
|  | SCHEDULE RESET | YES |  |  |
|  | EXIT | YES |  |  |
| EXIT OSD | YES |  |  |  |

## 3.3	Configuration Menu

The detailed functions and parameter settings of your high speed dome can be set through the OSD (On Screen Display) menu with a control device, such as a control keyboard. The items in each model’s OSD menu (C, F, U, R and K model) are described in the following sections.

To enter the OSD menu of the selected camera, press <CAMERA MENU> button on the control keyboard and hold for 3 seconds to enter the OSD menu.

To select the setup item, use direction keys on keyboard to move the OSD cursor in the OSD menu.

To setup item, use direction keys on keyboard to move the OSD cursor in the OSD menu. For items with →, press right/left direction buttons on the control keyboard to select. For items with ↓, press <CAMERA MENU> button on the control keyboard to enter sub menu. For items with →↓, users can use the right/left direction buttons to select functions then press the <CAMERA MENU> button on the control keyboard to enter its sub menu.

For further detailed setup procedures, please refer to the user’s manual of your installed control devices.

### 3.3.1	LANGUAGE

The camera supports multi-language OSD function; the available languages include English, French, German, Italian, Japanese, Polish, Portuguese, Russian and Spanish. You can straight set a wanted language on the MAIN PAGE 1, as shown below. As you select a language and press the <CAMERA MENU> button, the OSD will automatically change to the language you selected. The default language is <ENGLISH>.

MAIN PAGE 1

LANGUAGE         ENGLISH

DEFAULT CAMERAON

BACKLIGHTOFF

FOCUSAUTO

AE MODEENTER

WBC MODEAUTO

SETUP MENU 1ENTER

SETUP MENU 2ENTER

### 3.3.2	DEFAULT CAMERA

The DEFAULT CAMERA is used to restore some camera setting to default setting, including Backlight, Focus, AE, WBC, Aperture, Zoom Speed and Digital Zoom. Once any one of the items is modified, the setting will become <OFF> automatically. Select <ON> for this item to recall the mentioned camera parameters.

### 3.3.3	BACKLIGHT

The Backlight compensation function prevents the center object from being too dark in surroundings where excessive light is behind the object.

C/F/U Model:

Select <ON> to activate the function; the center object will be brightened in contrast to the edge of the picture (where a backlight would be most likely located).

After completing setup of backlight, go back to the Main Page 1 and continue to set the focus relevant values.

MAIN PAGE 1

LANGUAGE         ENGLISH

DEFAULT CAMERAON

BACKLIGHTOFF

FOCUSAUTO

AE MODEENTER

WBC MODEAUTO

SETUP MENU 1ENTER

SETUP MENU 2ENTER

R/K Model:

The Backlight Compensation Level ranges from 00 to 30.

BLC LEVEL00

EXIT

NOTE: If this function is enabled, the WDR function (for K model only) will be disabled automatically. For details, refer to section 3.3.20 WDR Setting.

After completing setup of backlight, go back to the Main Page 1 and continue to set the focus values.

MAIN PAGE 1

LANGUAGE         ENGLISH

DEFAULT CAMERAON

BACKLIGHTOFF

FOCUSAUTO

AE MODEAUTO

WBC MODEAUTO

SETUP MENU 1ENTER

SETUP MENU 2ENTER

### 3.3.4	FOCUS

The focus of the dome camera can be operated in two modes: Auto Focus mode and Manual Focus mode. Various setting for different models are described as follows.

C/F/U Model:

- AUTO

The optimum focus is achieved by the internal digital circuit. There are 3 modes for users to select for different conditions.

Normal AF (Auto Focus) Mode: The dome will automatically adjust the focus of the picture.

Zoom Trigger Mode: When users press the TELE or the WIDE buttons on a control keyboard or other control devices to change the zoom, the dome will automatically adjust its focus after a period of time (the initial preset value is five seconds) until the commands of TELE/WIDE being quitted.

Interval AF Mode: The mode is used for AF movements carried out at particular intervals. If users pan/tilt the dome, the dome will focus automatically after a period of time; the initial value is five seconds.

- MANUAL

In this focus mode, users can adjust the focus speed, ranging from 01 ~ 08.

FOCUS SPEED01

After completing setup of focus, go back to the Main Page 1 and continue to set the AE mode.

MAIN PAGE 1

LANGUAGE         ENGLISH

DEFAULT CAMERAON

BACKLIGHTOFF

FOCUSAUTO

AE MODEENTER

WBC MODEAUTO

SETUP MENU 1ENTER

SETUP MENU 2ENTER

R/K Model:

- AUTO

The optimum focus is achieved by the internal digital circuit. Users can adjust the minimum auto focus range for some special conditions; the options are <1 cm>, <10 cm>, <30 cm> and <1 m>.

TURNING VALUE10CM

- MANUAL

In this focus mode, users can adjust the focus speed, ranging from 0 ~ 3.

FOCUS SPEED0

After completing setup of focus, go back to the Main Page 1 and continue to set the AE mode.

MAIN PAGE 1

LANGUAGE         ENGLISH

DEFAULT CAMERAON

BACKLIGHTOFF

FOCUSAUTO

AE MODEAUTO

WBC MODEAUTO

SETUP MENU 1ENTER

SETUP MENU 2ENTER

### 3.3.5	AE MODE

The exposure is the amount of light received by the image sensor and is determined by how wide you open the lens diaphragm (iris adjustment), how long you keep the sensor exposed (shutter speed), and other exposure parameters. With this item, users can define how the Auto Exposure (AE) function works.

C/F/U Model:

- EXPOSURE COMPENSATION

The exposure value rages from -10.5dB ~ 10.5dB. Select <OFF> to disable the function.

- AE MODE

AUTO

In this mode, the camera’s Brightness, Shutter Speed, IRIS and AGC (Auto Gain Control) control circuits work together automatically to get consistent video output level.

BRIGHT

The brightness control function adjusts IRIS and AGC using an internal algorithm. Brightness is controlled by gain when the light condition is dark and by iris when the light condition is bright. The bright value ranges from 00 ~ 03.

SHUTTER

With this option, SHUTTER speed takes main control of exposure, and both IRIS and AGC will function automatically in cooperation with shutter speed to achieve consistent exposure output. The shutter speed ranges from 1/10000 ~ 1.

IRIS

With this option, the IRIS function adjusts exposure in higher property. SHUTTER speed and AGC circuit will function automatically in cooperating with IRIS to get consistent exposure output. The opening of a lens controls the amount of light reaching to the surface of the selected device. By increasing the F-stop number (F/1.6, F/2, F/2.4, etc.), less light is permitted to pass.

MANUAL

In the mode, users can adjust shutter speed (1/10000 ~ 1), iris value (F1.6 ~ F28) and gain value (-3dB ~ 28dB).

- EXIT

Exit the AE MODE menu and go back to the Main Page 1 to continue to set the WBC mode.

MAIN PAGE 1

LANGUAGE         ENGLISH

DEFAULT CAMERAON

BACKLIGHTOFF

FOCUSAUTO

AE MODEENTER

WBC MODEAUTO

SETUP MENU 1ENTER

SETUP MENU 2ENTER

R/K Model:

- AUTO

In this mode, the camera’s Shutter, IRIS and AGC control function work automatically to compensate the light exposure of image sensor for consistent video output level. IRIS OFFSET is used to set the level of IRIS variation (00 ~ 15).

- SHUTTER

With this option, the priority of SHUTTER is higher than IRIS and AGC; IRIS and AGC circuit will function automatically in cooperating with SHUTTER to get consistent exposure. The range of shutter speed for K model is: 1/30000 ~ 1/2 and for R model: 1/30000 ~ 1/50 (PAL) or 1/60 (NTSC).

- IRIS

With this option, the priority of IRIS is higher than SHUTTER and AGC; SHUTTER and AGC circuit will function automatically in cooperating with IRIS to get consistent exposure. If the IRIS is modified manually, the action of exposure compensation depends on the AGC circuit. The range of Iris level is between 00 and 09.

- AGC

With this option, the priority of AGC is higher than SHUTTER and IRIS; SHUTTER and IRIS circuit will function automatically in cooperating with AGC to get consistent exposure. If AGC is adjusted manually, the exposure compensation depends on the changing of IRIS. The range of Iris level is between 00 and 05.

After completing various parameter setups, please exit the AE MODE menu and go back to the Main Page 1 to continue to set the WBC mode.

MAIN PAGE 1

LANGUAGE         ENGLISH

DEFAULT CAMERAON

BACKLIGHTOFF

FOCUSAUTO

AE MODEAUTO

WBC MODEAUTO

SETUP MENU 1ENTER

SETUP MENU 2ENTER

### 3.3.6	WBC MODE

A digital camera needs to find reference color temperature, which is a way of measuring the quality of a light source, for calculating all the other colors. The unit for measuring this ratio is in degree Kelvin (K). You can select one of the White Balance Control modes according to the condition. The following table shows the color temperature of some light sources.

| Light Sources | Color Temperature in K |
| --- | --- |
| Cloudy Sky | 6,000 to 8,000 |
| Noon Sun and Clear Sky | 6,500 |
| Household Lighting | 2,500 to 3,000 |
| 75-watt Bulb | 2,820 |
| Candle Flame | 1,200 to 1,500 |

C/F/U Model:

- AUTO

In this mode, white balance works within its color temperature range. This mode computes the white balance value output using color information from the entire screen. It outputs the proper value using the color temperature radiating from a black subject based on a range of values from 3000K to 7500K.

- INDOOR

3200 K Base mode.

- OUTDOOR

5800 K Base mode.

- ATW (Auto Tracing White Balance)

The dome takes out the signals in a screen in the range from 2000 K to 10000 K.

- MANUAL

In this mode, users can change the White Balance value manually; R gain and B gain are adjustable and range from 000 to 127.

WBC MENU

R GAIN050

B GAIN050

B GAIN50

After WBC relevant parameter setups are completed, please exit the WBC MODE menu and go back to the Main Page 1 to continue to set other functions under the setup menu 1.

MAIN PAGE 1

LANGUAGE         ENGLISH

DEFAULT CAMERAON

BACKLIGHTOFF

FOCUSAUTO

AE MODEENTER

WBC MODEAUTO

SETUP MENU 1ENTER

SETUP MENU 2ENTER

R/K Model:

- AUTO

In this mode, white balance works within its color temperature range and calculates the best-fit white balance.

- MANUAL

In this mode, users can change the White Balance value manually; adjustable R gain and B gain range from 00 to 99.

WBC MENU

R GAIN50

B GAIN50

After WBC relevant parameter setups are completed, please exit the WBC MODE menu and go back to the Main Page 1 to continue to set other functions under the setup menu 1.

MAIN PAGE 1

LANGUAGE         ENGLISH

DEFAULT CAMERAON

BACKLIGHTOFF

FOCUSAUTO

AE MODEAUTO

WBC MODEAUTO

SETUP MENU 1ENTER

SETUP MENU 2ENTER

### 3.3.7	SETUP MENU 1

C Model:                         F/U Model:

SETUP MENU 1

ZOOM SPEED         8

DIGITAL ZOOMON

SLOW SHUTTERNONE

IMAGE INVERSENONE

APERTURE11

EXITYES

SETUP MENU 1

ZOOM SPEED         8

DIGITAL ZOOMON

SLOW SHUTTEROFF

IMAGE INVERSEOFF

APERTURE11

EXITYES

R/K Model:

SETUP MENU 1

ZOOM SPEED         FAST

DIGITAL ZOOMOFF

SLOW SHUTTEROFF

IMAGE INVERSEOFF

APERTUREAUTO

EXITYES

#### ZOOM SPEED

This item is used to set the zoom speed of the dome camera.

C/F/U Model:

For these models, the zoom speed value ranges from <1> (slow) to <8> (fast). The default value is <8>.

R/K Model:

For the two models, the options are <FAST> (default) and <SLOW>.

#### DIGITAL ZOOM

With this item, users can enable or disable the 12× Digital Zoom. The Digital Zoom will be activated after the full Optical Zoom level is reached.

NOTE: The difference between optical and digital zoom is that optical zoom uses the lens within the camera to draw the image closer via zoom in or out to achieve the desired effect. Optical zoom remains the same quality and full resolution of the zoomed image. On the other hand, Digital zoom takes a portion of an image and expands the partial image to the full size of the original image; therefore, the image quality will be reduced.

C/F/U Model:

For these models, maximum 12× digital zoom function is allowed to be enabled. The default setting is <ON>.

R/K Model:

For the two models, Digital zoom ratio is adjustable from <02> to <12>. The default setting is <OFF>.

#### SLOW SHUTTER

The shutter speed determines how long the image sensor is exposed to light. To see clear image in a dark environment, please enable this function and select a slower shutter speed.

F/U Model:

As enable the digital slow shutter function, the dome will automatically adjust the shutter speed basing on the light condition of installation environment. It enables users to see objects in a dark environment under 0.2 lux.

K Model:

The shutter speed is adjustable in K model. With the slowest shutter speed, users can see objects in a dark environment under 0.2 lux or see smooth video image with a higher shutter speed. The options are from <1/2> to <1/60> for NTSC and <1/1.5> to <1/50> for PAL.

#### IMAGE INVERSE (F/K/R/U Model)

Users can select <ON> to make the displayed image inversed vertically and horizontally. Occasions to employ the function include conferences, demonstration, testing, etc. For R and K models, when this function is enabled, the preset mask(s) will be set off automatically (see 3.3.21 Privacy Mask). The default setting is <OFF>.

Application: Users can see the displayed images, as shown below, when a dome is placed on the desk top in a conference, for instance.

IMAGE INVERSE (OFF)        IMAGE INVERSE (ON)

#### APERTURE

Under this setup menu, users can adjust enhancement of the edges of objects in the picture.

C/F/U Model:

There are 16 levels of adjustment; the options are <01> ~ <16>; <01> represents “no enhancement”. When shooting text, this function could make it sharp.

R/K Model:

Users can select either the <AUTO> mode or <MANUAL> mode. Under the <MANUAL> mode, the parameters of H aperture and V aperture are adjustable, ranging from 00 to 31.

APERTURE MENU

H APERTURE00

V APERTURE00

B GAIN50

#### EXIT

Exit the SETUP MENU 1 and go back to the MAIN PAGE 1 to set other functions under the setup menu 1.

C/F/U Model:                     R/K Model:

MAIN PAGE 1

LANGUAGE         ENGLISH

DEFAULT CAMERAON

BACKLIGHTOFF

FOCUSAUTO

AE MODEENTER

WBC MODEAUTO

SETUP MENU 1ENTER

SETUP MENU 2ENTER

MAIN PAGE 1

LANGUAGE         ENGLISH

DEFAULT CAMERAON

BACKLIGHTOFF

FOCUSAUTO

AE MODEAUTO

WBC MODEAUTO

SETUP MENU 1ENTER

SETUP MENU 2ENTER

### 3.3.8	SETUP MENU 2

C/F/U/R/K Model:

SETUP MENU 2

FLIP         ENTER

ANGLE ADJUSTERENTER

SPEED BY ZOOMOFF

AUTO CALI.OFF

SYSTEM RESETYES

EXITYES

#### FLIP

Users can track an object continuously when it passes through under the dome camera with setting Flip to IMAGE (digital flip) or M.E. (mechanical flip).

FLIP SETTING

FLIPOFF

EXITYES

B GAIN50

IMAGE (F/K/R/U Model)

IMAGE represents digital IMAGE FLIP, which enables users to keep tracking objects seamlessly; under the mode, almost no delay occurs in comparing with that under the M.E. mode.

NOTE: The Privacy Mask function will be automatically disabled if the Image Flip function is enabled, and the screen will show “MASK WILL BE SET OFF.”

M.E. (Mechanical Flip)

M.E. is a standard mechanical operation. As the dome tilts 90°, it will pan 180°, and then continue tilting to keep tracking objects.

OFF

Select this item to disable the flip function.

NOTE: To make the dome tilt between -10° to +100° or -10° ~

+190°, please go to ANGLE ADJUSTER (see next section) to complete setting. Otherwise, the dome will only be able to tilt 90°.

#### ANGLE ADJUSTER

The item is for adjusting the angle of view. The Range of the view angle varies in different FLIP modes: the angle ranges from -10° to +100° in the M.E. FLIP and FLIP OFF modes, and from -10° ~ +190° in the IMAGE FLIP mode.

ANGLE ADJUSTER

ADJUST MIN ANGLE-10DEG

ADJUST MAX ANGLE100DEG

EXIT + SAVE   YES

B GAIN50

#### SPEED BY ZOOM

If the item is set to <ON>, the pan/tilt speed will be automatically adjusted by internal algorithm when zooming. The larger zoom ratio leads to the lower rotating speed.

#### AUTO CALI. (Auto Calibration)

There are one horizontal point and one vertical infrared rays check point in each dome. During installation or maintenance, the dome camera’s position may be moved. Therefore, the relative distance between the original set point and the check point will be changed. If enable the Auto Calibration function, the dome will automatically detect the matter and reset the horizontal point back to the original position.

#### SYSTEM RESET

Select this item for remote resetting.

#### EXIT

Exit the SETUP MENU 2 and go back to the MAIN PAGE 1.Then go to the MAIN PAGE 2 to carry on setting other functions.

MAIN PAGE 2

ID DISPLAY         ON

TITLE DISPLAY         OFF

TITLE SETTING01

PRESETENTER

SEQUENCE              ENTER

AUTOPANENTER

CRUISE ENTER

HOME SETTING    ENTER

### 3.3.9	ID DISPLAY

Press the direction button down to turn the MAIN MENU page from 1 to 2, and then the menu item <ID DISPLAY> will be shown on the top. Users are allowed to choose whether the dome ID will be displayed on screen for identifying the domes. For more information, please refer to 2.4 Dome ID Setting.

- ON

Display the ID address of the selected dome on the right bottom of the screen.

- OFF

Hide the ID address of the selected dome.

### 3.3.10	TITLE DISPLAY

Users are allowed to name a view area, where the title will be displayed on screen for easy recognition.

- ON

Select <ON> to display the title set for a view area on screen while the camera shooting the view area.

- OFF

When TITLE DISPLAY is set <OFF>, no title will be displayed on screen even titles have been set in advance.

### 3.3.11	TITLE SETTING

Up to 16 zone titles can be set with maximum 20 characters for each title. Each view area’s title can be named with a privacy mask ID number for future recognition.

NOTE: For the K model, the available area for setting a privacy mask is restricted within tilt angle 45°.

Follow the steps to set a camera title.

STEP 1: Operate the dome to a view area where you want to set a title for it.

STEP 2:	Turn on the OSD and go to the MAIN PAGE 2 to select <TITLE 	SETTING>.

STEP 3: Select a number to represent the view area.

STEP 4: Press <ENTER> to go into the editing page.

| TITLE SETTING: 01 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | EXIT |
|  | A | B | C | D | E | F | G | H | I | J | SAVE |
|  | K | L | M | N | O | P | Q | R | S | T | LEFT |
|  | U | V | W | X | Y | Z | : | / | . | , | RIGHT |
|  | [ | ] | + | ? | - |  |  |  |  |  | DELETE |
|  | TITLE: ABC |  |  |  |  |  |  |  |  |  |  |

STEP 5: Choose a character with direction keys and then press <ENTER> to 	input. For example: <A > <ENTER>, <B> <ENTER>, <C> <ENTER>

TITLE: ABC

STEP 6:	To delete input characters, move the cursor to <LEFT> or 	<RIGHT> and press <ENTER> to select a character in the entry field. 	Then move the cursor to <DELETE> and press <ENTER> to delete 	the selected character.

STEP 7:	When the setting is completed, move the cursor to <SAVE> and 	press <ENTER> to save.

After completing tile setting, go back to the MAIN PAGE 2 to carry on setup of preset points.

MAIN PAGE 2

ID DISPLAY         ON

TITLE DISPLAY         OFF

TITLE SETTING01

PRESETENTER

SEQUENCE              ENTER

AUTOPANENTER

CRUISE ENTER

HOME SETTING    ENTER

### 3.3.12	PRESET

- PRESET SET

Move the camera to the targeted shooting area/point. When the cursor flashes, press “ENTER” to set the area/point as preset point 1, 2, 3, etc. Totally 256 preset points can be set.

- PRESET RUN

Select the preset point that you want to execute. After pressing “ENTER”, the camera will turn to the appointed point.

- EXIT

Exit the PRESET menu and go back to the MAIN PAGE 2 to carry on setup of sequence.

MAIN PAGE 2

ID DISPLAY         ON

TITLE DISPLAY         OFF

TITLE SETTING01

PRESETENTER

SEQUENCE              ENTER

AUTOPANENTER

CRUISE ENTER

HOME SETTING    ENTER

Users could set preset points through a keyboard. Please refer to the dome camera’s quick guide for further information.

### 3.3.13	SEQUENCE

The function executes pre-positioning of the pan, tilt, zoom and focus features in a certain sequence for a camera. Before setting this function, users must preset at least two preset points.

SEQUENCE

SEQUENCE LINE1

SEQUENCE POINT01

PRESET POSITION001

SPEED01

DWELL TIME001

RUN SEQUENCEENTER

EXITYES

- SEQUENCE LINE

There are eight sets of sequence lines built in the dome camera. Using LEFT/RIGHT direction keys to select a line first and then set its sequence points.

- SEQUENCE POINT

Up to 32 points can be specified for each sequence line. The sequence points represent order of the preset points that the dome will automatically run. The following setup items, including PRESET POSITION, SPEED and DWELL TIME, will influence how the camera runs through each sequence point.

- PRESET POSITION

Users can assign a specific preset position to the selected sequence point with this item.

- SPEED

Users can set the speed of one sequence point to the next one, and the range of setup speed is from 1 to 15. Within the range, PAN speed is adjustable from 10 ~ 400(degree/sec.), and TILT speed is adjustable from 8 ~ 400(degree/sec.).

- DWELL TIME

The DWELL TIME is the duration time that the dome will stay at a sequence point, and the range is from <0> to <127> seconds. The dome will go to the next sequence point when the DEWEL TIME expires. If the setting is <0>, the dome will stay at this sequence point until users manually move the dome.

- RUN SEQUENCE

Users can command the dome camera to run the selected sequence line manually.

- EXIT

Select the item to exit the SEQUENCE menu; go back to the MAIN PAGE 2 to carry on setup of auto-pan.

MAIN PAGE 2

ID DISPLAY         ON

TITLE DISPLAY         OFF

TITLE SETTING01

PRESETENTER

SEQUENCE              ENTER

AUTOPANENTER

CRUISE ENTER

HOME SETTING    ENTER

Users could execute the sequence function through a keyboard. Please refer to the dome camera’s quick guide for further information.

### 3.3.14	AUTOPAN

Auto-pan means motion of scanning an area horizontally so that the dome camera can catch horizontal view. The parameters are listed as follows.

AUTOPAN

AUTOPAN LINE1

START POINTTO FIND

END POINTTO FIND

DIRECTIONRIGHT

SPEED01

RUN AUTOPANENTER

EXITYES

- AUTOPAN LINE

There are four sets of auto-pan line built in a dome camera. Users can choose a line to execute using LEFT/RIGHT direction keys. In addition, users are able to command the dome camera to do endless panning by setting the start point the same as the end point.

- START POINT

Follow the description below to set the start position of the AUTOPAN path.

- Move the cursor to <START POINT> and press <ENTER> while the item, <TO FIND>, is flashing. Then the item will turn <TO SAVE> automatically.

- Move the dome to a desired position and press <ENTER> to save the position as the start point; the cursor will move to <END POINT> automatically. Ensure setting the end point to complete auto-pan setting.

NOTE: The tilt and zoom values of the start point will be recorded and fixed for the selected auto-pan line.

- END POINT

Users are able to set the end point after the start point is defined. Pan the dome to another position and press <ENTER> to save the position as the end point.

- DIRECTION

The item is for setting the AUTOPAN direction of the dome camera. The dome will start to pan clockwise from the start point to the end point if your selection is <RIGHT>, and then return to the start point. The dome will start to pan anti-clockwise from the start point to the end point if your selection is <LEFT>. Refer to the diagram below.

Start Point

LEFT

(anti-clockwise)

RIGHT

(clockwise)

End Point

Dome Camera

RIGHT

LEFT

- SPEED

The item is for defining the dome camera rotation speed while running auto-pan. The speed is adjustable from 1 to 4 (10 ~ 45 degree/sec.).

- RUN AUTOPAN

After all setting related to auto-pan are completed, select this item to execute the Auto-pan function.

- EXIT

Exit the AUTOPAN setup menu; go back to the MAIN PAGE 2 to carry on setup of cruise.

MAIN PAGE 2

ID DISPLAY         ON

TITLE DISPLAY         OFF

TITLE SETTING01

PRESETENTER

SEQUENCE              ENTER

AUTOPANENTER

CRUISE ENTER

HOME SETTING    ENTER

Users could execute the auto-pan function through a keyboard. Please refer to the dome camera’s quick guide for further information.

### 3.3.15	CRUISE

CRUISE is a route formed with manual operation, through adjusting pan, tilt position and zoom parameters, which can be stored and recalled to execute repeatedly (zoom setting is only available for the E/F/U model).

CRUISE

RECORD STARTENTER

RECORD ENDENTER

RUN CRUISEENTER

EXITYES

- RECORD START

Follow the description below to record the CRUISE path.

- Rotate the dome camera to a desired view area, and press <ENTER> to build the cruise path using the joystick on the control device. The percentage of the memory buffer will be displayed on the screen.

- Pan and tilt the dome camera to form a path. The zoom setting is only available for the E/F/U model.

NOTE: Beware of the memory size when building a cruise path. Once the buffer percentage reaches 100%, recording of the path will stop.

- RECORD END

The cursor will be moved to RECORD END while building the cruise line; when the setting is completed, press <ENTER> to save the path.

- RUN CRUISE

After cruise setting is completed, select this item to execute the Cruise function.

- EXIT

Exit the CRUISE setup menu ; go back to the MAIN PAGE 2 to carry on setup of home setting.

MAIN PAGE 2

ID DISPLAY         ON

TITLE DISPLAY         OFF

TITLE SETTING01

PRESETENTER

SEQUENCE              ENTER

AUTOPANENTER

CRUISE ENTER

HOME SETTING    ENTER

Users could execute the cruise function through a keyboard. Please refer to the dome camera’s quick guide for further information.

### 3.3.16	HOME SETTING

Users are able to set an operation mode to ensure constant monitoring. If the dome idles for a period of time, the preset function will be activated automatically; this is the HOME function. The HOME function allows constant and accurate monitoring so that to avoid the dome idling or missing events.

HOME SETTING

HOME FUNCTIONOFF

SELECT MODEPRESET

PRESET POINT001

RETURN TIME001 MIN.

GOENTER

EXITYES

- HOME FUNCTION

The item is used to enable or disable the HOME function. Use the left/right direction keys of the control keyboard to change the setting.

- SELECT MODE

Select one of the modes that the dome should execute when the HOME function is enabled and the RETURN TIME expires. The options include <AUTOPAN>, <SEQUENCE>, <CRUISE> and <PRESET>. Use the left/right direction keys of the control keyboard to change the setting, and the items below will change in cooperating with your selection.

- PRESET POINT

Select a preset point where the dome should go after the Return Time function, which will be mentioned later, is activated. The preset point(s) should be set prior either in the PRESET setup menu or through the keyboard.

SEQUENCE LINE

Select a sequence line that the dome camera should execute after the Return Time function is activated. The sequence line(s) should be defined prior either in the SEQUENCE setup menu or through the keyboard.

AUTOPAN LINE

Select an auto-pan line that the dome camera should execute after the Return Time function is activated. The auto-pan line(s) should be defined prior either in the AUTOPAN setup menu or through the keyboard.

CRUISE LINE

Select a cruise line that the dome camera should execute after the Return Time function is activated. The cruise line should be defined prior either in the CRUISE setup menu or through the keyboard.

- RETURN TIME

The dome starts to count down RETURN TIME when the dome idles, and then execute the SELECT MODE function when the return time is up. The RETURN TIME ranges from 0 to 128 minutes.

- GO

If HOME function is enabled, users are allowed to execute HOME function by selecting this item.

- EXIT

Exit the HOME SETTING menu. Then go to the MAIN PAGE 3 to carry on other setups.

NOTE: If use C/R model, please skip the following section and go to 3.3.18 ALARM SETTING to continue setup of alarm related setting.

C Model:                         F/U Model:

MAIN PAGE 3

IR FUNCTION          NONE

ALARM SETTINGENTER

ALARM DETECTENTER

PRIVACY MASKENTER

TIME SETTING         ENTER

SCHEDULEENTER

EXIT OSDYES

MAIN PAGE 3

IR FUNCTION          AUTO

ALARM SETTINGENTER

ALARM DETECTENTER

PRIVACY MASKENTER

TIME SETTING         ENTER

SCHEDULEENTER

EXIT OSDYES

R Model:                          K Model:

MAIN PAGE 3

IR FUNCTION          NONE

ALARM SETTINGENTER

WDR FUNCTIONNONE

PRIVACY MASKENTER

TIME SETTING         ENTER

SCHEDULEENTER

EXIT OSDYES

MAIN PAGE 3

IR FUNCTION          AUTO

ALARM SETTINGENTER

WDR FUNCTIONOFF

PRIVACY MASKENTER

TIME SETTING         ENTER

SCHEDULEENTER

EXIT OSDYES

### 3.3.17	IR FUNCTION (Removable IR Cut)

With the IR cut filter, the dome can still catch clear image at night time or in the very dark light condition. During day time, the IR cut filter will be on to block the infrared light for clear image; during night time, the IR cut filter will be removed to catch infrared light, and the displayed images will become black and white. Moreover, in the K model, users are able to view color images when the IR function is activated.

Refer to the description below to operate the IR function.

F/U Model:

- AUTO

The Internal circuit will automatically decide the occasion to remove the IR cut filter according to the value of light condition calculated by the internal light algorithm.

- ON

Select the item to remove the IR cut filter.

K Model:

- AUTO

The Internal circuit will automatically decide the occasion to remove the IR cut filter according to the image brightness level.

- ON

Select the item to remove the IR cut filter.

IR FUNCTION

THRESHOLDLOW

IR COLORCOLOR

EXITYES

THRESHOLD

The dome will remove the filter immediately when the threshold value is reached. The threshold options are <LOW>, <MID> and <HI>. <LOW> threshold indicates a higher sensitivity and can improve reliability of lens.

IR COLOR

When the IR function is enabled, the video output can be programmed as color or B/W (black and white).

EXIT

Exit the IR function menu and go back to the MAIN PAGE 3 to carry on setup of alarm setting.

MAIN PAGE 3

IR FUNCTION          AUTO

ALARM SETTINGENTER

WDR FUNCTIONOFF

PRIVACY MASKENTER

TIME SETTING         ENTER

SCHEDULEENTER

EXIT OSDYES

### 3.3.18	ALARM SETTING

The integrated high speed dome provides eight alarm inputs and one alarm output (N.O. or N.C) to connect alarm devices. With this function, the dome can cooperate with alarm system to catch events’ images. For wiring, please refer to the installation guide and/or qualified service personnel. Adjustable alarm parameters are listed below.

ALARM SETTING

ALARM PIN1

ALARM SWITCHOFF

ALARM TYPEN.C.

ALARM ACTIONPRESET

PRESET POINT001

DWELL TIMEALWAY

EXITYES

- ALARM PIN

The dome provides 8 alarm inputs and 1 output (N.O. / N.C.). Select an alarm connector which you want to set its alarm-related parameters with this item, and then set its alarm-related parameters in the Alarm Setting menu. For alarm pin definitions, please refer to 2.7 Alarm Pin Definition or the installation guide.

- ALARM SWITCH

The item is used to enable or disable the selected alarm pin function. Use the left/right direction keys on the control keyboard to change the setting.

- ALARM TYPE

There are two kinds of alarm types: Normal Open and Normal Close, which are illustrated as below. Select an alarm type that corresponds with the alarm application.

Normal Open

Alarm In

Normal Close

Alarm In

- ALARM ACTION

The alarm actions include PRESET, SEQUENCE, AUTOPAN and CRUISE functions. Select one of these modes so that certain action will be executed when an alarm is triggered. Use the right direction key of the control keyboard to select a particular action mode, and the items listed below will change in accordance with your selected alarm action.

- PRESET POINT

Select a preset point where the dome should go when an alarm pin is triggered. The preset point(s) should be set prior either in the PRESET setup menu or through the keyboard.

SEQUENCE LINE

Select a sequence line that the dome camera should execute when an alarm pin is triggered. The sequence line(s) should be defined prior either in the SEQUENCE setup menu or through the keyboard.

AUTOPAN LINE

Select an auto-pan line that the dome camera should execute when an alarm pin is triggered. The auto-pan line(s) should be defined prior either in the AUTOPAN setup menu or through the keyboard.

CRUISE LINE

Select a cruise line that the dome camera should execute when an alarm pin is triggered. The cruise line should be defined prior either in the CRUISE setup menu or through the keyboard.

- DWELL TIME

The DWELL TIME is duration of executing an alarm action. If select the PRESET mode, when alarm takes place, the dome will go to the selected preset position and stay there for a user-defined period of time (1~127seconds/Always). If select other modes (SEQUENCE/AUTOPAN/ CRUISE), the dome will keep executing the selected mode (DWELL TIME: ALWAYS) until alarm condition is released or users rotate the dome.

NOTE: The dwell time is only adjustable when selecting Preset as the alarm action.

- EXIT

Exit the ALARM SETTING menu and go back to the MAIN PAGE 3 to carry on setup of alarm detect (C/F/U model only).

C Model:                         F/U Model:

MAIN PAGE 3

IR FUNCTION          NONE

ALARM SETTINGENTER

ALARM DETECTENTER

PRIVACY MASKENTER

TIME SETTING         ENTER

SCHEDULEENTER

EXIT OSDYES

MAIN PAGE 3

IR FUNCTION          AUTO

ALARM SETTINGENTER

ALARM DETECTENTER

PRIVACY MASKENTER

TIME SETTING         ENTER

SCHEDULEENTER

EXIT OSDYES

If use the K model, after exiting the ALARM SETTING menu, go back to the MAIN PAGE 3 to carry on setup of WDR function. Thus please skip the next section and go to 3.3.20 WDR FUNCTION to continue setup of WDR function.

K Model:

MAIN PAGE 3

IR FUNCTION          AUTO

ALARM SETTINGENTER

WDR FUNCTIONOFF

PRIVACY MASKENTER

TIME SETTING         ENTER

SCHEDULEENTER

EXIT OSDYES

If use the R model, please skip the next two sections and go to 3.3.21 PRIVACY MASK to continue setup of privacy mask.

R Model:

MAIN PAGE 3

IR FUNCTION          NONE

ALARM SETTINGENTER

WDR FUNCTIONNONE

PRIVACY MASKENTER

TIME SETTING         ENTER

SCHEDULEENTER

EXIT OSDYES

### 3.3.19	ALARM DETECT (C/F/U Model Only)

When the alarm detect function is activated, the camera will detect movement within a monitoring area and then send an alarm signal automatically. Before activating this function, alarm connection setups must be completed in advance.

ALARM DETECT

DETECT SWITCH    OFF

DETECT MODE    INT FOCUS

EXIT    YES

- DETECT SWITCH

The item is used to enable or disable the ALARM DETECT function.

- DETECT MODE

Four alarm detect modes are provided for different application.

INT FOCUS (Internal Focus)

The alarm will be triggered if the internal focus changes; if the focus returns to the original position, the alarm will stop.

FIX FOCUS

If focus movement is detected, the alarm will be triggered, and the alarm stops when focus returns to the original position. If the detected focus movement keeps changing for more than four seconds, the new focus position will be memorized as the reference, and the alarm will stop.

NOTE: The INT FOCUS and FIX FOCUS detect modes will be activated only with the Auto Focus mode.

INT AE (Internal AE)

When Auto Exposure (AE) movement is detected, the alarm will be triggered; if the Exposure Level returns to the original level, the alarm will stop.

FIX AE

The alarm will be triggered if the Exposure value changes; if the adjusted AE value retains for four seconds, the value will be saved as the reference, and the alarm will stop.

- EXIT

Exit the ALARM DETECT menu and go back to the MAIN PAGE 3 to carry on setup of privacy mask (see 3.3.21 PRIVACY MASK).

C Model:                        F/U Model:

MAIN PAGE 3

IR FUNCTION          NONE

ALARM SETTINGENTER

ALARM DETECTENTER

PRIVACY MASKENTER

TIME SETTING         ENTER

SCHEDULEENTER

EXIT OSDYES

MAIN PAGE 3

IR FUNCTION          AUTO

ALARM SETTINGENTER

ALARM DETECTENTER

PRIVACY MASKENTER

TIME SETTING         ENTER

SCHEDULEENTER

EXIT OSDYES

### 3.3.20	WDR FUNCTION (K Model Only)

The Wide Dynamic Range (WDR) function is especially effective in solving indoor and outdoor contrast issues to enhance better image quality and video display. It enables the dome to catch detailed data from the dark part (Indoor) without any saturation from the bright part (Outdoor).

NOTE: The Backlight function will be turned off automatically when the WDR function is enabled because the WDR function has better effects than Backlight Compensation.

- AUTO

In this mode, the dome camera will operate the WDR function automatically.

- ON

Under the item, users can define three parameters’ value: RATIO LEVEL (000 ~ 128), SHUTTER SPEED (000 ~ 128) and IRIS OFFSET (000 ~ 128), as shown in the following column.

WDR MODE

RATIO LEVEL000

SHUTTER SPEED000

IRIS OFFSET000

EXITYES

- OFF

Exit the WDR FUNCTION menu and go back to the MAIN PAGE 3 to carry on setup of privacy mask.

MAIN PAGE 3

IR FUNCTION          AUTO

ALARM SETTINGENTER

WDR FUNCTIONOFF

PRIVACY MASKENTER

TIME SETTING         ENTER

SCHEDULEENTER

EXIT OSDYES

### 3.3.21	PRIVACY MASK

The Privacy Mask function aims to avoid any intrusive monitoring. Users can adjust the camera view position using the joystick, and adjust the mask size and area via the direction keys on the control keyboard. The dome camera will memorize the center of the selected view as an original point, so the joystick will be locked as users enter the SET MASK menu (mentioned later). Refer to the following description for setting privacy masks.

NOTE: The Image Flip function (for all models) and the Image Inverse function (for K model) will be disabled automatically while the Privacy Mask function is enabled.

C/F/U Model:

PRIVACY MASK MENU

PRIVACY SWITCHOFF

TRANSPARENCYOFF

COLORBLACK

SET MASK01

CLEAR MASK              01

EXITYES

- PRIVACY SWITCH

Users can enable or disable the Privacy Mask function through this item. Set this item to <ON> before configuring mask zones.

- TRANSPARENCY

The color of privacy mask can be set as transparent. Select <ON> to display transparent masks.

- COLOR

The color of privacy mask can be set through this item. The available colors are black, gray, light gray (LI. GRAY), white, red, green, blue, cyan, yellow and magenta.

- SET MASK

Use the control device to move the dome camera to the area where you want to set a mask. Press <ENTER> to enter the SET MASK menu. The dome will memorize the present position as a privacy mask position. Up to 24 masks can be set.

MASK01 MENU

H CENTERL/R

V CENTERD/U

H SIZE000

V SIZE000

EXIT+SAVEYES

H CENTER

The original horizontal center of a mask zone is the center of a screen; it is able to move a mask zone to the other position by adjusting the horizontal value with the LEFT/RIGHT keys on the keyboard. The camera will pan right or left according to user’s control.

V CENTER

The original vertical center of a mask zone is the center of a screen; it is able to move a mask zone to the other position by adjusting the vertical value with the LEFT/RIGHT keys on the keyboard. The camera will tilt up or down according to user’s control.

H SIZE (00~80)

Users can adjust the horizontal size of a privacy mask through this item. Set the H and V size to 0 can also delete the selected mask.

V SIZE (00~60)

Users can adjust the vertical size of a privacy mask through this item. Set the H and V size to 0 can also delete the selected mask.

- CLEAR MASK

Users can delete a preset mask zone with this item. Please follow the steps listed below.

1. Select the mask zone that will be erased (e.g. 01).

2. Press <ENTER> to confirm the selection. Consequently, the screen will display the instructions to reset after the mask is cleared.

3. Select <RESET> under the CLEAR MASK item and press <ENTER> to proceed with resetting.

- EXIT

Exit the PRIVACY MASK menu and go back to the MAIN PAGE 3 to carry on setup of time related setting.

C Model:                        F/U Model:

MAIN PAGE 3

IR FUNCTION          NONE

ALARM SETTINGENTER

ALARM DETECTENTER

PRIVACY MASKENTER

TIME SETTING         ENTER

SCHEDULEENTER

EXIT OSDYES

MAIN PAGE 3

IR FUNCTION          AUTO

ALARM SETTINGENTER

ALARM DETECTENTER

PRIVACY MASKENTER

TIME SETTING         ENTER

SCHEDULEENTER

EXIT OSDYES

K Model:

PRIVACY

PRIVACY SWITCHOFF

SHADEBLACK

SET MASK01

CLEAR MASK01

MASK DISPLAYFIRST

EXITYES

- PRIVACY SWITCH

The item is used to enable or disable the masking function. Set this item to <ON> before configuring mask zones.

- SHADE

The color of a privacy mask can be selected through this item. The available colors are black, gray and white.

- SET MASK

After pressing <ENTER> to enter the sub-menu of SET MASK, the dome will memorize the present position as a privacy mask position; up to 8 masks can be set. The model restricts the mask zones to be set too close with each other.

NOTE: For the K model, the available area for setting a privacy mask is restricted within tilt angle 45°, and two mask zones are allowed to set in a view area.

MASK01 MENU

H CENTER000

V CENTER000

H SIZE000

V SIZE000

EXIT+SAVEYES

H CENTER (000~255)

The original center of a mask zone is the center of a screen. Users can move the center of a mask zone to another position through adjust this value by pressing the LEFT/RIGHT keys on the keyboard.

V CENTER (000~255)

The original center of mask zone is the center of screen. User can move the center of mask zone to another position through adjust this value by pressing the LEFT/RIGHT keys on the keyboard.

H SIZE (000~127)

Users can adjust the horizontal size of a privacy mask through this item. Set the H and V size to 0 can also delete the selected mask.

V SIZE (000~127)

User can adjust the vertical size of a privacy mask through this item. Set the H and V size to 0 can also delete the selected mask.

- CLEAR MASK

Users can delete a preset mask zone with this item. Please follow the steps listed below.

1. Select the mask zone that will be erased (e.g. 01).

2. Press <ENTER> to confirm the selection. Consequently, the screen will display the instructions to reset after the mask is cleared.

3. Select <RESET> under the CLEAR MASK item and press <ENTER> to proceed with resetting.

- MASK DISPLAY

This item is used to set the time to display a privacy mask.

FIRST

If select this mode, the camera will detect the mask zone at the next preset position and display the mask in advance, and then pan the dome to the preset point.

LAST

If select this mode, the camera will move the dome to the preset point, and then display the mask zone.

- EXIT

Exit the PRIVACY MASK menu and go back to the MAIN PAGE 3 to carry on setup of time related setting.

R Model:                        K Model:

MAIN PAGE 3

IR FUNCTION          NONE

ALARM SETTINGENTER

WDR FUNCTIONNONE

PRIVACY MASKENTER

TIME SETTING         ENTER

SCHEDULEENTER

EXIT OSDYES

MAIN PAGE 3

IR FUNCTION          AUTO

ALARM SETTINGENTER

WDR FUNCTIONOFF

PRIVACY MASKENTER

TIME SETTING         ENTER

SCHEDULEENTER

EXIT OSDYES

### 3.3.22	TIME SETTING

The time setting function is used to set the TIME related parameters of the integrated high speed dome. Each item in the menu is listed as follows.

TIME SETTING

TIME DISPLAYOFF

SET YEAR00

SET MONTH01

SET DAY00

SET HOUR00

SET MINUTE00

EXIT+SAVEYES

- TIME DISPLAY

Select <ON> to display time information on screen or <NO> not to display.

- YEAR / MONTH / DAY

The items are for setting up the system date.

- HOUR / MINUTE

The items are for setting up the system time.

- EXIT+SAVE

Exit the TIME SETTING menu and go back to the MAIN PAGE 3 to carry on setup of schedule.

C Model:                         F/U Model:

MAIN PAGE 3

IR FUNCTION          NONE

ALARM SETTINGENTER

ALARM DETECTENTER

PRIVACY MASKENTER

TIME SETTING         ENTER

SCHEDULEENTER

EXIT OSDYES

MAIN PAGE 3

IR FUNCTION          AUTO

ALARM SETTINGENTER

ALARM DETECTENTER

PRIVACY MASKENTER

TIME SETTING         ENTER

SCHEDULEENTER

EXIT OSDYES

R Model:                         K Model:

MAIN PAGE 3

IR FUNCTION          NONE

ALARM SETTINGENTER

WDR FUNCTIONNONE

PRIVACY MASKENTER

TIME SETTING         ENTER

SCHEDULEENTER

EXIT OSDYES

MAIN PAGE 3

IR FUNCTION          AUTO

ALARM SETTINGENTER

WDR FUNCTIONOFF

PRIVACY MASKENTER

TIME SETTING         ENTER

SCHEDULEENTER

EXIT OSDYES

### 3.3.23	SCHEDULE

The schedule function enables users to program a preset point or function (Sequence/Auto-pan/Cruise) automatically to perform in a specific period of time.

SCHEDULE

SWITCHOFF

POINT00

HOUR00

MINUTE00

MODEPRESET

PRESET POINT001

SCHEDULE RESETYES

EXITYES

- SWITCH

Select <ON> to enable or <OFF> to disable the schedule function.

- POINT

Users are allowed to arrange 32 sets of schedule point, i.e. each set of schedule point can be assigned one kind of schedule modes.

- HOUR / MINUTE

The items are for setting up the time to execute each schedule point.

- MODE

This is for setting the schedule function of the selected schedule point; the options are listed as follows.

NONE

No action will be executed for the schedule if select the item.

PRESET

Users can select the PRESET mode as an action carried out in a schedule point.

SEQUENCE

Users can select the SEQUENCE mode as an action carried out in a schedule point.

AUTOPAN

Users can select the AUTOPAN mode as an action carried out in a schedule point.

CRUISE

Users can select the CRUISE mode as an action carried out in a schedule point.

IR FUNC. (IR Function)

If select the IR function mode, the AUTO IR FUNCTION will be activated for a schedule point.

- SCHEDULE RESET

Users can reset the whole schedule with the item.

- EXIT

Exit the SCHEDULE menu and go back to the MAIN PAGE 3.

### 3.3.24	EXIT OSD

To exit the OSD setup menu, users can either select this item on the bottom of MAIN PAGE 3 or press the ESC button on the control keyboard.

# Appendix A: Technical Specification

|  | Items |  | R Model | C Model | F Model |  | K Model | U Model |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CAMERA |  |  |  |  |  |  |  |  |
|  | Effective Pixels | NTSC | 380k |  |  |  |  |  |
|  |  | PAL | 440k |  |  |  |  |  |
|  | Horizontal Resolution | NTSC | 480 TV lines |  |  |  |  |  |
|  |  | PAL | 480 TV lines |  |  |  |  |  |
|  | Scanning Area |  | Progressive 1/4" CCD | 1/4" type IT CCD | 1/4" CCD ExView |  | Progressive 1/4" CCD | 1/4" CCD ExView |
|  | Scanning System |  | PAL, NTSC |  |  |  |  |  |
|  | Synchronization |  | Internal / Line Lock |  |  |  |  |  |
|  | Video Output |  | 1.0 Vp-p / 75, BNC |  |  |  |  |  |
|  | S/N Ratio (AGC OFF) |  | More than 49dB |  |  |  |  |  |
|  | Minimum Illumination |  | 2 lux | 1.0 lux | 0.7 lux; 0.01lux (B/W) |  | 0.1 lux; 0.02lux (B/W) | 0.07 lux; 0.01lux (B/W) |
|  | Focal Length |  | 4~88 mm | 4.1~73.8 mm |  |  | 3.6~82.8 mm | 3.5~91 mm |
|  | Zoom Ratio |  | 22× optical zoom | 18× optical zoom | 18× optical zoom |  | 23× optical zoom | 26× optical zoom |
|  | Digital Zoom |  | ×1 ~ ×12 variable |  |  |  |  |  |
|  | Focus Mode |  | Auto / Manual |  |  |  |  |  |
|  | White Balance |  | Auto / Manual |  |  |  |  |  |
|  | Iris Control |  | Auto / Manual |  |  |  |  |  |
|  | Electronic Shutter | NTSC | 1/60~1/30k sec. | 1/1~1/10k sec. |  |  | 1/2~1/30k sec. | 1/1~1/10k sec. |
|  |  | PAL | 1/50~1/30k sec | 1/1~1/10k sec |  |  | 1/1.5~1/30k sec | 1/1~1/10k sec |
|  | AGC control |  | Auto / Manual |  |  |  |  |  |
|  | Back Light Compensation |  | On / Off |  |  |  |  |  |
| OPERATION |  |  |  |  |  |  |  |  |
|  | Built-in Protocol |  | DynaColor, Pelco, VCL, Philips, AD-Manchester, AD-422, JVC, Kalatel, etc. |  |  |  |  |  |
|  | Multi-Language OSD |  | English, French, German, Italian, Japanese, Polish, Portuguese, Russian, Spanish |  |  |  |  |  |
|  | Pan Travel |  | 360° endless |  |  |  |  |  |
|  | Tilt Travel |  | -10°~100° |  |  | -10°~190° |  |  |
|  | Manual Speed |  | 1°~90°/s |  |  |  |  |  |
|  | Presets |  | 256 |  |  |  |  |  |
|  | Preset Accuracy | Pan | 0.225° |  |  |  |  |  |
|  |  | Tilt | 0.45° |  |  |  |  |  |
|  | Preset Speed** | Pan | 5°~400°/s, High Resolution (Both D&E Type Motor) |  |  |  |  |  |
|  |  | Tilt | 5°~400°/s, Standard Resolution (D Type Motor). 5°~400°/s, High Resolution (E Type Motor) |  |  |  |  |  |
|  | Cruise |  | 1 |  |  |  |  |  |
|  | Sequence |  | 8 |  |  |  |  |  |
|  | Auto Pan |  | 4 |  |  |  |  |  |
|  | Privacy Mask |  | - | 24 | 24 |  | 8 | 24 |
|  | Pan & Tilt Speed Proportional to Zoom Ratio |  | Yes | Yes | Yes |  | Yes | Yes |
|  | P/T/Z Auto-Restoring |  | Yes | Yes | Yes |  | Yes | Yes |
|  | Auto Turn Around |  | Yes | Yes | Yes |  | Yes | Yes |
|  | Zone Title |  | Yes | Yes | Yes |  | Yes | Yes |
|  | Home Function |  | Yes | Yes | Yes |  | Yes | Yes |
|  | Digital Flip |  | - | - | Yes |  | Yes | Yes |
|  | Digital Slow Shutter |  | - | - | Yes |  | Yes | Yes |
|  | Motion Detection |  | - | Yes | Yes |  | - | Yes |
|  | Wide Dynamic Range |  | - | - | - |  | Yes | - |
|  | Day/Night: IR Cut Filter |  | - | - | Yes |  | Yes | Yes |
|  | Alarm Input |  | 8 |  |  |  |  |  |
|  | Alarm Output |  | 1 |  |  |  |  |  |
| GENERAL |  |  |  |  |  |  |  |  |
|  | Environment |  | Outdoor |  |  |  |  |  |
|  | Controller Interface |  | RS-485 |  |  |  |  |  |
|  | Operating Temperature |  | -30°C~45°C (-22°F~104°F) |  |  |  |  |  |
|  | Water Proof Standard |  | IP 66 Standard |  |  |  |  |  |
|  | Dimension |  | ∅262×305mm (10.4x12 Inches), with sunshield |  |  |  |  |  |
|  | Weight |  | 5.8 kg (12.9 lbs) |  |  |  |  |  |
|  | Power Source |  | AC 24V |  |  |  |  |  |
|  | Power Consumption |  | 30 W / 65 W (with Heater) |  |  |  |  |  |
|  | Regulatory |  | CE, FCC, IP66 |  |  |  |  |  |

**There are D&E motors, differentiated by the type of motors of high speed dome cameras and presents various “Preset Speed” and resolutions. Standard Resolution: 800 steps/circle. High Resolution: 1600 steps/circle

# OSD Menu Notes

The following OSD menu tables are provided for users to record various dome setting.

## <C/F/U Model>

| Item | Layer 1 |  | Layer 2 | Layer 3 | Notes |
| --- | --- | --- | --- | --- | --- |
| LANGUAGE | <ENGLISH>, <CHINESE>, <FRENCH>, <GERMAN>, <ITALIAN>, <JAPANESE>, <POLISH>, <PORTUGUESE>, <RUSSIAN>, <SPANISH> |  |  |  |  |
| DEFAULT CAMERA | <ON>, <OFF> |  |  |  |  |
| BACKLIGHT | <ON>, <OFF> |  |  |  |  |
| FOCUS | AUTO |  | AF MODE <NORMAL>, <INTERVAL>, <ZOOM TRIG> |  |  |
|  | MANUAL |  | FOCUS SPEED <01>~<08> |  |  |
| AE MODE | EXPODURE COMP. |  | <OFF>, EXPOSURE VALUE: <-10.5dB> ~ <10.5dB> |  |  |
|  | AE MODE | AUTO |  |  |  |
|  |  | BRIGHT |  | BRIGHT VALUE <00> ~ <31> |  |
|  |  | SHUTTER |  | SHUTTER SPEED <1> ~ <1/10000> SEC. |  |
|  |  | IRIS |  | IRIS VALUE <CLOSE>, <F1.6> ~ <F28> |  |
|  |  | MANUAL |  | BRIGHT VALUE: AUTO |  |
|  |  |  |  | SHUTTER SPEED <1/10000> ~ <1> |  |
|  |  |  |  | IRIS VALUE <F1.6> ~ <F28> |  |
|  |  |  |  | GAIN VALUE <-3>dB ~ <28>dB |  |
|  | EXIT |  | YES |  |  |
| WBC MODE | AUTO (Auto White Balance) |  |  |  |  |
|  | INDOOR |  |  |  |  |
|  | OUTDOOR |  |  |  |  |
|  | ATW (Auto-tracing WBC) |  |  |  |  |
|  | MANUAL |  | R GAIN <000> ~ <127> |  |  |
|  |  |  | B GAIN <000> ~ <127> |  |  |
| SETUP MENU 1 | ZOOM SPEED |  | <1> ~ <8> |  |  |
|  | DIGITAL ZOOM |  | <ON>, <OFF> |  |  |
|  | SLOW SHUTTER (F/U Model only) |  | <ON>, <OFF> |  |  |
|  | IMAGE INVERSE (F/U Model only) |  | <ON>, <OFF> |  |  |
|  | APERTURE |  | <01> ~ <16> |  |  |
|  | EXIT |  | YES |  |  |
| SETUP MENU 2 | FLIP |  | C Model: <OFF>, <M.E.> F/U Model: <OFF>, <M.E.>, <IMAGE> |  |  |
|  | ANGLE ADJUSTER |  | MIN ANGLE<-10 ~ +10 DEG>, |  |  |
|  |  |  | MAX ANGLE <080 ~ 100 DEG> |  |  |
|  | SPEED BY ZOOM |  | <ON>, <OFF> |  |  |
|  | AUTO CALI. |  | <ON>, <OFF> |  |  |
|  | SYSTEM RESET |  | YES |  |  |
|  | EXIT |  | YES |  |  |
| ID DISPLAY | <ON>, <OFF> |  |  |  |  |
| TITLE DISPLAY | <ON>, <OFF> |  |  |  |  |
| TITLE SETTING | <01> ~ <16> |  |  |  |  |
| PRESET | PRESET SET |  | <001>~<256> |  |  |
|  | PRESET RUN |  | <001>~<256> |  |  |
|  | EXIT |  | YES |  |  |
| SEQUENCE | SEQUENCE LINE |  | <1> ~ <8> |  |  |
|  | SEQUENCE POINT |  | <01> ~ <32> |  |  |
|  | PRESET POS. |  | <001> ~ <255>, <END> |  |  |
|  | SPEED |  | <01> ~ <15> |  |  |
|  | DWELL TIME |  | <000> ~ <127> SEC. |  |  |
|  | RUN SEQUENCE |  | ENTER |  |  |
|  | EXIT |  | YES |  |  |
| AUTOPAN | AUTOPAN LINE |  | <1> ~ <4> |  |  |
|  | START POINT |  | <TO FIND>, <TO SAVE> |  |  |
|  | END POINT |  | <TO FIND>, <TO SAVE> |  |  |
|  | DIRECTION |  | <RIGHT>, <LEFT> |  |  |
|  | SPEED |  | <01> ~ <04> |  |  |
|  | RUN AUTOPAN |  | ENTER |  |  |
|  | EXIT |  | YES |  |  |
| CRUISE | RECORD START |  | ENTER |  |  |
|  | RECORD END |  | ENTER |  |  |
|  | RUN CRUISE |  | ENTER |  |  |
|  | EXIT |  | YES |  |  |
| HOME SETTING | HOME FUNCTION |  | <ON>, <OFF> |  |  |
|  | SELECT MODE |  | <PRESET>, <SEQUENCE>, <AUTOPAN>, <CRUISE> |  |  |
|  | PRESET POINT SEQUENCE LINE AUTOPAN LINE CRUISE LINE |  | <001> ~ <256> |  |  |
|  |  |  | <1> ~ <8> |  |  |
|  |  |  | <1> ~ <4> |  |  |
|  |  |  | <1> |  |  |
|  | RETURN TIME |  | <1> ~ <128> MIN. |  |  |
|  | GO |  | ENTER |  |  |
|  | EXIT |  | YES |  |  |
| IR FUNCTION (F/U Model only) | <AUTO>, <ON> |  |  |  |  |
| ALARM SETTNG | ALARM PIN |  | <1> ~ <8> |  |  |
|  | ALARM SWITCH |  | <ON>, <OFF> |  |  |
|  | ALARM TYPE |  | <NO> (Normal Open), <NC> (Normal Close) |  |  |
|  | ALARM ACTION |  | <PRESET>, <SEQUENCE>, <AUTOPAN>, <CRUISE> |  |  |
|  | PRESET POINT SEQUENCE LINE AUTOPAN LINE CRUISE LINE |  | <001> ~ <256> |  |  |
|  |  |  | <1> ~ <8> |  |  |
|  |  |  | <1> ~ <4> |  |  |
|  |  |  | 1 |  |  |
|  | DWELL TIME |  | <001> ~ <127> Sec., <ALWAYS> |  |  |
|  | EXIT |  | YES |  |  |
| ALARM DETECT | DETECT SWITCH |  | <ON>, <OFF> |  |  |
|  | DETECT MODE |  | <INT FOCUS>, <FIX FOCUS>, <INT AE>, <FIX AE> |  |  |
|  | EXIT |  | YES |  |  |
| PRIVACY MASK | PRIVACY SWITCH | <ON>, <OFF> |  |  |  |
|  | TRANSPARENCY | <ON>, <OFF> |  |  |  |
|  | COLOR | <BLACK>, <LI. GRAY>, < GRAY>, <WHITE>, <RED>, <GREEN>, <BLUE>, <CYAN>, <YELLOW>, <MAGENTA> |  |  |  |
|  | SET MASK | <01> ~ <24> |  | H CENTER: L/R |  |
|  |  |  |  | V CENTER: D/U |  |
|  |  |  |  | H SIZE <000> ~ <080> |  |
|  |  |  |  | V SIZE <000> ~ <060> |  |
|  |  |  |  | EXIT + SAVE |  |
|  | CLEAR MASK | <01> ~ <24> |  |  |  |
|  | EXIT | YES |  |  |  |
| TIME SETTING | TIME DISPLAY |  | <ON>, <OFF> |  |  |
|  | SET YEAR |  | <00> ~ <99> |  |  |
|  | SET MONTH |  | <01> ~ <12> |  |  |
|  | SET DAY |  | <00> ~ <31> |  |  |
|  | SET HOUR |  | <00> ~ <23> |  |  |
|  | SET MINUTE |  | <00> ~ <59> |  |  |
|  | EXIT+SAVE |  |  |  |  |
| SCHEDULE | SWITCH |  | <ON>, <OFF> |  |  |
|  | POINT |  | <01> ~ <32> |  |  |
|  | HOUR |  | <00> ~ <23> |  |  |
|  | MINUTE |  | <00> ~ <59> |  |  |
|  | MODE |  | NONE | NO FUNCTION |  |
|  |  |  | PRESET | PRESET POINT <001> ~ <256> |  |
|  |  |  | SEQUENCE | SEQUENCE LINE <1> ~ <8> |  |
|  |  |  | AUTOPAN | AUTOPAN LINE <1> ~ <4> |  |
|  |  |  | CRUISE | CRUISE LINE <1> |  |
|  |  |  | IR FUNC. | IR FUNCTION AUTO |  |
|  | SCHEDULE RESET |  | YES |  |  |
|  | EXIT | YES |  |  |  |
| EXIT OSD | YES |  |  |  |  |

## <R/K Model>

| Item | Layer 1 | Layer 2 |  |  |  | Layer 3 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LANGUAGE | <ENGLISH>, <CHINESE>, <FRENCH>, <GERMAN>, <ITALIAN>, <JAPANESE>, <POLISH>, <PORTUGUESE>, <RUSSIAN>, <SPANISH> |  |  |  |  |  |  |
| DEFAULT CAMERA | <ON>, <OFF> |  |  |  |  |  |  |
| BACKLIGHT | <ON> | BLC LEVEL <00> ~ <30> |  |  |  |  |  |
|  | <OFF> |  |  |  |  |  |  |
| FOCUS | AUTO | TUNING VALUE <1CM>, <10CM>, <30CM>, <1M> |  |  |  |  |  |
|  | MANUAL | FOCUS SPEED <0> ~ <3> |  |  |  |  |  |
| AE MODE | AUTO | IRIS OFFSET <00> ~ <15> |  |  |  |  |  |
|  | SHUTTER | SHUTTER SPEED K Model:<1/30000> ~ <1/2> R Model:<1/30000> ~ <1/50> (PAL); <1/30000> ~ <1/60> (NTSC) |  |  |  |  |  |
|  | IRIS | <00> ~ <09> |  |  |  |  |  |
|  | AGC | <00> ~ <05> |  |  |  |  |  |
| WBC MODE | AUTO (Auto White Balance) |  |  |  |  |  |  |
|  | MANUAL | R GAIN <00> ~ <99> B GAIN <00> ~ <99> |  |  |  |  |  |
| SETUP MENU 1 | ZOOM SPEED | <FAST>, <SLOW> |  |  |  |  |  |
|  | DIGITAL ZOOM | <OFF>, <02> ~ <12> |  |  |  |  |  |
|  | SLOW SHUTTER (K Model only) | <1/2> ~ <1/60> (NTSC) |  |  |  |  |  |
|  |  | <1/1.5> ~ <1/50> (PAL) |  |  |  |  |  |
|  | IMAGE INVERSE | <ON>, <OFF> |  |  |  |  |  |
|  | APERTURE | <AUTO> |  |  |  |  |  |
|  |  | <MANUAL> |  |  | H APERTURE <00> ~ <31> V APERTURE <00> ~ <31> |  |  |
|  | EXIT | YES |  |  |  |  |  |
| SETUP MENU 2 | FLIP | <OFF>, <M.E.>, <IMAGE>(K Model only) |  |  |  |  |  |
|  | ANGLE ADJUSTER | MIN ANGLE<-10 ~ +10 DEG>, |  |  |  |  |  |
|  |  | MAX ANGLE <080 ~ 100 DEG> |  |  |  |  |  |
|  | SPEED BY ZOOM | <ON>, <OFF> |  |  |  |  |  |
|  | AUTO CALI. | <ON>, <OFF> |  |  |  |  |  |
|  | SYSTEM RESET | YES |  |  |  |  |  |
|  | EXIT | YES |  |  |  |  |  |
| ID DISPLAY | <ON>, <OFF> |  |  |  |  |  |  |
| TITLE DISPLAY | <ON>, <OFF> |  |  |  |  |  |  |
| TITLE SETTING | <01> ~ <16> |  |  |  |  |  |  |
| PRESET | PRESET SET | <001>~<256> |  |  |  |  |  |
|  | PRESET RUN | <001>~<256> |  |  |  |  |  |
|  | EXIT | YES |  |  |  |  |  |
| SEQUENCE | SEQUENCE LINE | <1> ~ <8> |  |  |  |  |  |
|  | SEQUENCE POINT | <01> ~ <32> |  |  |  |  |  |
|  | PRESET POS. | <001> ~ <255>, <END> |  |  |  |  |  |
|  | SPEED | <01> ~ <15> |  |  |  |  |  |
|  | DWELL TIME | <000> ~ <127> SEC. |  |  |  |  |  |
|  | RUN SEQUENCE | ENTER |  |  |  |  |  |
|  | EXIT | YES |  |  |  |  |  |
| AUTOPAN | AUTOPAN LINE | <1> ~ <4> |  |  |  |  |  |
|  | START POINT | <TO FIND>, <TO SAVE> |  |  |  |  |  |
|  | END POINT | <TO FIND>, <TO SAVE> |  |  |  |  |  |
|  | DIRECTION | <RIGHT>, <LEFT> |  |  |  |  |  |
|  | SPEED | <01> ~ <04> |  |  |  |  |  |
|  | RUN AUTOPAN | ENTER |  |  |  |  |  |
|  | EXIT | YES |  |  |  |  |  |
| CRUISE | RECORD START | ENTER |  |  |  |  |  |
|  | RECORD END | ENTER |  |  |  |  |  |
|  | RUN CRUISE | ENTER |  |  |  |  |  |
|  | EXIT | YES |  |  |  |  |  |
| HOME SETTING | HOME FUNCTION | <ON>, <OFF> |  |  |  |  |  |
|  | SELECT MODE | <PRESET>, <SEQUENCE>, <AUTOPAN>, <CRUISE> |  |  |  |  |  |
|  | PRESET POINT SEQUENCE LINE AUTOPAN LINE CRUISE LINE | <001> ~ <256> |  |  |  |  |  |
|  |  | <1> ~ <8> |  |  |  |  |  |
|  |  | <1> ~ <4> |  |  |  |  |  |
|  |  | <1> |  |  |  |  |  |
|  | RETURN TIME | <1> ~ <128> MIN. |  |  |  |  |  |
|  | GO | ENTER |  |  |  |  |  |
|  | EXIT | YES |  |  |  |  |  |
| IR FUNCTION (K Model only) | <AUTO>, <ON> | THREADHOLD <MID>, <HI>, <LOW> IR COLOR <B/W>, <COLOR> EXIT <YES> |  |  |  |  |  |
| ALARM SETTNG | ALARM PIN | <1> ~ <8> |  |  |  |  |  |
|  | ALARM SWITCH | <ON>, <OFF> |  |  |  |  |  |
|  | ALARM TYPE | <NO> (Normal Open), <NC> (Normal Close) |  |  |  |  |  |
|  | ALARM ACTION | <PRESET>, <SEQUENCE>, <AUTOPAN>, <CRUISE> |  |  |  |  |  |
|  | PRESET POINT SEQUENCE LINE AUTOPAN LINE CRUISE LINE | <001> ~ <256> |  |  |  |  |  |
|  |  | <1> ~ <8> |  |  |  |  |  |
|  |  | <1> ~ <4> |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  |
|  | DWELL TIME | <001> ~ <127> Sec., <ALWAYS> |  |  |  |  |  |
|  | EXIT | YES |  |  |  |  |  |
| WDR FUNCTION (K Model only) | <ON> | RATIO LEVEL <000> ~ <128> SHUTTER SPEED <000> ~ <128> IRIS OFFSET <000> ~ <128> EXIT <YES> |  |  |  |  |  |
|  | <OFF> |  |  |  |  |  |  |
| PRIVACY MASK (K Model only) | PRIVACY SWITCH | <ON>, <OFF> |  |  |  |  |  |
|  | MASK SHADE | <GRAY>, <WHITE>, <BLACK> |  |  |  |  |  |
|  | SET MASK | <01> ~ <08> |  | H CENTER <000> ~ <255> |  |  |  |
|  |  |  |  | V CENTER <000> ~ <255> |  |  |  |
|  |  |  |  | H SIZE <000> ~ <127> |  |  |  |
|  |  |  |  | V SIZE <000> ~ <127> |  |  |  |
|  |  |  |  | EXIT + SAVE |  |  |  |
|  | CLEAR MASK | <01> ~ <08>, <RESET> |  |  |  |  |  |
|  | MASK DISPLAY | <FIRST>, <LAST> |  |  |  |  |  |
|  | EXIT | YES |  |  |  |  |  |
| TIME SETTING | TIME DISPLAY | <ON>, <OFF> |  |  |  |  |  |
|  | SET YEAR | <00> ~ <99> |  |  |  |  |  |
|  | SET MONTH | <01> ~ <12> |  |  |  |  |  |
|  | SET DAY | <00> ~ <31> |  |  |  |  |  |
|  | SET HOUR | <00> ~ <23> |  |  |  |  |  |
|  | SET MINUTE | <00> ~ <59> |  |  |  |  |  |
|  | EXIT+SAVE |  |  |  |  |  |  |
| SCHEDULE | SWITCH | <ON>, <OFF> |  |  |  |  |  |
|  | POINT | <01> ~ <32> |  |  |  |  |  |
|  | HOUR | <00> ~ <23> |  |  |  |  |  |
|  | MINUTE | <00> ~ <59> |  |  |  |  |  |
|  | MODE | NONE | NO FUNCTION |  |  |  |  |
|  |  | PRESET | PRESET POINT <001>~<256> |  |  |  |  |
|  |  | SEQUENCE | SEQUENCE LINE <1>~<8> |  |  |  |  |
|  |  | AUTOPAN | AUTOPAN LINE <1>~<4> |  |  |  |  |
|  |  | CRUISE | CRUISE LINE <1> |  |  |  |  |
|  |  | IR FUNC. | IR FUNCTION AUTO |  |  |  |  |
|  | SCHEDULE RESET | YES |  |  |  |  |  |
|  | EXIT | YES |  |  |  |  |  |
| EXIT OSD | YES |  |  |  |  |  |  |