Event Script Programmers Guide

Table of Contents

1. Introduction	3

2. Event Script Format	3

3. Comments	3

4. Trigger Condition	3

4.1 Time and Date Event Fields	4

4.2 Trig source and Run Option Field	4

5. Action Commands	6

5.1 Alert	6

5.1 FTP	7

5.3 Log	9

5.4 Mail	10

5.5 PTZ	11

5.6 Relay	12

5.7 Reset	12

5.8 Sleep	13

5.9 LED	13

5.10 Motion	14

5.10.1 motion_init	14

5.10.2 motion_del	14

5.10.3 motion_start	14

5.10.4 motion_stop	15

5.11 serial	16

6. Related CGI Commands	17

6.1 Get Script File	17

6.2 Set Script File	17

6.3 Get Script Command Return Code	17

6.4 Set Script Boot Flag	18

6.5 Start Script	18

6.6 Stop Script	18

6.7 Check Script Status	18

6.8 Show Log File	18

# 1. Introduction

The Event Script provides a flexible way for Administrator to configure Video Server and Network camera as automatic intelligent devices in applications and a comprehensive set of “trigger conditions” and “action commands” allowed to fit in varies of applications.

# 2. Event Script Format

An event script for the Video Server includes one or several entries and is entered into the editor in the following format:

- # <comments>

- <triggering condition> :

- <action commands>;

- %

where:

- The <comments> are optional.

- The <trigger condition> includes the following fields: <minute> <hour> <day> <month> <weekday> <optional trigger source and run option>

- The <commands> include one or several event script action commands.

- The % is end symbol

# 3. Comments

It is a good programming practice to start each new entry with a comment to describe its function. Comments are optional but must be proceeded by a # character, as detailed below:

# <comment>

Example

# This programming entry will...

# 4. Trigger Condition

The commands contained within each specific entry are triggered by a defined trigger condition. The trigger condition is specified by six separate fields and must be terminated with a colon ":".

<Minute> <Hour> <Day> <Month> <Day of the week> <(optional) Trig Source and Run Option Field >:

## 4.1 Time and Date Event Fields

The first five fields specify the time and date events, i.e. Minute, Hour, Day, Month and Day of the week.

The syntax for each field within a trigger condition is governed by the following rules:

- Each time and date field can contain several numerical event variables that are delimited by commas and hyphens.

- Each field is delimited by an open space.

- An asterisk (*) represents the full range of event variables within the relative time and date field, i.e. “* * * * *” means every minute, every hour, every day, every month, every day of the week.

- Numerical event variables separated by a hyphen indicate an inclusive range; for example, 2-6 means 2 to 6.

Example

Trigger every month, between the fourth and eighth at 10.03, 12.03 and 14.03, using the 24-hour clock:

3 10,12,14 4-8 * * :

## 4.2 Trig source and Run Option Field

The sixth field is a trigger source and runs option field that defines the trigger source other than timer and event run options.

The table below outlines the available trigger variables:

| Trigger | Description |
| --- | --- |
| RESET | Run the action when system resets |
| RUN | Run the action when script starts running |
| \V1 | Video channel one sync signal lose |
| /V1 | Video channel one sync signal back |
| \V2 | Video channel two sync signals lose |
| /V2 | Video channel two sync signals back |
| \V3 | Video channel three sync signals lose |
| /V3 | Video channel three sync signals back |
| \V4 | Video channel four sync signals lose |
| /V4 | Video channel four sync signals back |
| \I1 | GPIN port one falling (high to low) edge |
| /I1 | GPIN port one rising (low to high) edge |
| HI1 | GPIN port one high level (*1) |
| LI1 | GPIN port one low level (*1) |
| \I2 | GPIN port two falling (high to low) edge |
| /I2 | GPIN port two rising (low to high) edge |
| HI2 | GPIN port two high level (*1) |
| LI2 | GPIN port two low level (*1) |
| \I3 | GPIN port three falling (high to low) edge |
| /I3 | GPIN port three rising (low to high) edge |
| HI3 | GPIN port three high level (*1) |
| LI3 | GPIN port three low level (*1) |
| \I4 | GPIN port four falling (high to low) edge |
| /I4 | GPIN port four rising (low to high) edge |
| HI4 | GPIN port four high level (*1) |
| LI4 | GPIN port four low level (*1) |
| \N | Network link lose connection |
| /N | Network link back connection |
| M1 | Motion Detection of video channel one |
| M2 | Motion Detection of video channel two |
| M3 | Motion Detection of video channel three |
| M4 | Motion Detection of video channel four |
| G0 | CGI to trigger, http://IP/cgitrig?cmd=0 |
| G1 | CGI to trigger, http://IP/cgitrig?cmd=1 |
| G2 | CGI to trigger, http://IP/cgitrig?cmd=2 |
| G3 | CGI to trigger, http://IP/cgitrig?cmd=3 |
| G4 | CGI to trigger, http://IP/cgitrig?cmd=4 |
| G5 | CGI to trigger, http://IP/cgitrig?cmd=5 |
| G6 | CGI to trigger, http://IP/cgitrig?cmd=6 |
| G7 | CGI to trigger, http://IP/cgitrig?cmd=7 |
| G8 | CGI to trigger, http://IP/cgitrig?cmd=8 |
| G9 | CGI to trigger, http://IP/cgitrig?cmd=9 |

Note *1: The GPIN level trigger could be cascaded with other trigger variable with ‘&’ symbol, e.g. HI1&/I2&LI3, when rising edge of I2 occurs, if both I1 is high and I3 is low level at that time, then proceed the action node.

# 5. Action Commands

The table below outlines the available action commands.

| ALERT | Alert message to remote site through socket call |
| --- | --- |
| FTP | Ftp to remote site |
| LOG | Log message to internal file |
| MAIL | Email to remote site |
| PTZ | Pan Tilt Zoom device control through serial port |
| RELAY | Relay activities |
| RESET | Invoke system reset |
| SLEEP | Produce delay in specified event |
| LED | LED indicator activities |
| MOTION | Motion detection activities include _init, _start, _stop, _del |
| SERIAL | Serial port message send |

A comprehensive catalog of available programming commands, complete with their syntax and valid options are described below:

## 5.1 Alert

The alert command sends a message to a remote host.

##### Syntax

alert [-host HOST] [-port PORTNUMBER] [-message STRING];

##### Options

-host 
Specify the host IP address of the remote host. 
-port
Specify the TCP port number in the range [0..65534]. 
-message
Specify the message that will be sent to the remote host. The message must be surrounded by quotation marks. Hexadecimal coded messages must be preceded by \0x, e.g. "\0x04" for the non-printed character EOT. To include a quotation mark, it must be preceded by a backslash, e.g. \".

Example

# This command will send the message "Alarm # "0013" from Video Server to the remote host when # input port 1 goes high. * * * * * /I1: alert -host 172.21.1.202 -port 2703 -message "Alarm \"0013\" from Video Server"; %

## 5.1 FTP

The ftp command uses the standard File Transfer Protocol (FTP) for transferring an image from your Video Server to a remote host.

##### Note

The ftp command keeps the connection open during each file transfers.

##### Syntax

ftp [-host HOST] [-user USERNAME] [-pass PASSWORD] [-filename DESTINATIONFILE][-cam VIDEOCHANNEL] 
[-fmt VIDEOFORMAT] [-interval TIME] [-duration TIME] [-imagenum IMAGENUMBER] ;

##### Options

-host 
Specify the IP address of the remote host (ftp server).
-user 
Specify the user name for logging on to the remote host.
-pass
Specify the password for logging on to the remote host. 
-filename
Specify the name and whole path of the destination file. The maximum length of a destination path/file name is 255 characters; simple file name without path must be in 30 characters. 
You can add time stamps, such as time, date and file indexes, to the destination path/file name. This is useful for example when you are transferring a series of files. The following table lists the available time stamp variables. Every substitution must be proceeded by a dollar sign "$".

$s

The current second in the range 00-59.

$m

The current minute in the range 00-59.

$h

The current hour in the range 00-23.

$d

The current day of the month in the range 01-31.

$n

The current month in the range 01-12.

$y

The current year without the century in the range 00-99.

$Y

The current year including the century.

$rS-E

Inserts an index number starting from number S through all the numbers including the number E. If you leave out number E, it will be interpreted as a very large number.

-cam
Specify the camera source, i.e. 1- 4
-fmt
Specifies the image format (HxV), where: 
 	qsif=176x112, 
 	sif=352x240 and 
 	full=704x480
-interval
Specify the time interval between consecutive snapshots in unit of 10ms system tick. If omitted, default value is 50 (i.e. 500ms).
-duration
Specify the total time for the command to run. Accept value is
-1: 	Infinite loop,
0: 	Run Once,
>0: 	Value in unit of 10ms system tick
If omitted, the default value is 0 which will be issued only once. If you specify –1 (infinite loop), the command will continue to executed until stop running of event script.
-imagenum
Specify the images numbers to FTP
0 or Omit: FTP the images with above duration setting.
>0: Continue FTP until reaching specified image number.

##### Example 1

# This command will log in as "user" with the # password "pass" on the host with the Internet # address 123.123.123.123, download a full # image with the name "user.jpg" and put # it in the directory "home/user" on the remote # host. * * * * * : ftp -host 123.123.123.123 -user user -pass pass -fmt full –cam 1 -filename home/user/user.jpg; %

##### Example 2

# This command will every hour, every five seconds # for twenty minutes, download a sif image # from camera 3 and save the images with a time # stamp in the filename. 0 * * * * : ftp -host 192.168.0.1 -user user -pass pass -fmt sif -filename tmp/sif$h$m$s.jpg -interval 500 -duration 120000 -cam 3; %

##### Example 3

# This command will send 1001 consecutive sif # images to "your.host.name". The first and last # images will be named "sifsize1000.jpg" and # "sifsize2000.jpg", respectively. * * * * * : ftp -host 192.168.0.1 -user user -pass pass -fmt sif –cam 1 -filename home/camera1/tmp/sifsize$r1000-2000.jpg; %

## 5.3 Log

This command writes a message to the Video Server log file.

##### Note:

To display the log file, you type the following in the address/location field of your Web browser: http://IP/evscript?showlog=filename , where filename indicate the LOGFILENAME below.

##### Syntax

log [-filename LOGFILENAME] 
[-message LOGMESSAGE];

##### Options

-filename
Specifies the file name to log, note that the file will put in “Sys” directory.
-message
The message string must be surrounded by quotation marks.

##### Example

# This command writes a message to the log file when the Video Server # Network is lose. * * * * * \N : log "The Network lose link." ; %

## 5.4 Mail

This command uses the Simple Mail Transfer Protocol (SMTP) for sending images and files from your Video Server to e-mail addresses.

##### Note

The IP address of the mail server and a valid return address on the TCP/IP page must be specified when using the mail command.

##### Syntax

mail [-host SMTPSERVER] [-filename ATTACHFILENAME] [-msgbody MAILMESSAGEBODY] [-returnaddr RETURNADDR] [-recipient RECIPIENTS] [-subject MAILSUBJECT]
[-cam VIDEOCHANNEL] [-fmt VIDEOFORMAT] [-interval TIME] [-duration TIME] [-imgnum ATTACHNUMBEROFIMAGE] ;

##### Options

-host:
Specifies the SMTP Server name or IP address
-filename
Attachment filename, such as FTP filename, accept time stamp character variable.
-msgbody
Text message contain in email body.
-returnaddr
Return mail address
-recipient
Destination address to receive this email, could be multiple address separate by semicolon “;”.
-subject
Specifies email subject string
-cam
Specifies the camera source, i.e. 1- 4
-fmt
Specifies the image format (HxV), where: 
 	qsif=176x112, 
 	sif=352x240 and 
 	full=704x480
-interval
Specifies the time interval between consecutive snapshots in unit of 10ms system tick. If omitted, default value is 50 (i.e. 500ms).
-duration
Specifies the total time for the command to run. Accept value is
-1: 		Infinite loop,
Omit or 0: 	Run Once,
>0: 		Value in unit of 10ms system tick
If omitted, the default value is 0 which will be issued only once. If you specify –1 (infinite loop), the command will continue to executed until stop running of event script.
-imagenum
Specifies the image number of attachment to mail
0 or Omit:

##### Example 1

# This command will send an e-mail with the # subject "Hello" and the attached file # sif size to someone@company.com * * * * * : mail -subject Hello -fmt sif –cam 1 -recipient someone@company.com -host mail.company.com –filename test.jpg –imagenum 1; %

## 5.5 PTZ

This command sends PTZ control protocol through serial port. Supported model of PTZ devices should be configured within administration page in advance.

##### Syntax

ptz [-cam VIDEOCHANNEL] [-speed SPEED] [-preset PRESET] [-pan –1|+1] [-tilt –1|+1] [-zoom –1|+1];

##### Options

-cam
Specifies the camera source, i.e. 1- 4.
-speed
Sets the move speed of PTZ camera, range 1(slowest) ~ 9(fastest).
-preset
Move PTZ camera to preset position, range 1~20.
-pan
Specified the pan factor of PTZ camera, -1 means pan left, and +1 means pan right.
-tilt
Specified the tilt factor of PTZ camera, -1 means tilt down, and +1 means tilt up.
-zoom
Specified the zoom factor of PTZ camera, -1 means zoom in, and +1 means zoom out.

Note that above PTZ control (preset, pan, tilt, zoom) should be run just one option at a time, multiple options may produce unpredictable result.

##### Example

# This command sets the PTZ camera to preset position 5 when GPIO Input # one is triggered: * * * * * /I1 : ptz –cam 1 –preset 5; % #This command pan left with middle speed for every one minute * * * * *: ptz –cam 1 –speed 5 –pan –1; %

## 5.6 Relay

This command sets the relay on or off within the Video Server.

##### Syntax

relay [-on | -off | -toggle | -onoff ONTIME | -offon OFFTIME];

##### Options

-on
sets the relay on.
-off
sets the relay off.
-toggle
Toggle the relay
-onoff
Set the relay to on when start trigger, after ONTIME, set back to off, ONTIME is time in unit of 10ms system tick.
-offon
Set the relay to off when start trigger, after OFFTIME, set back to on, OFFTIME is time in unit of 10ms system tick.

##### Example

# This command sets the relay ON and after 3 seconds back to OFF # when digital input 1 goes high: * * * * * /I1 : relay –onoff 300; %

## 5.7 Reset

This command resets the Video Server.

##### Syntax

reset [-flag FLAG];

##### Options

-flag
Specifies to reboot flag
0:		Do Nothing
1:		Reboot system immediately

##### Example

# This command will reset the hardware once a day. 0 0 * * * * : reset –flag 1; %

## 5.8 Sleep

This command adds a pause to the programming script execution. The command is typically used if you want to assure that the previous command is finished before continuing the script.

##### Syntax

sleep [-time PAUSETIME];

##### Options

-time
Specifies the time in unit of 10ms system tick to wait before proceeding with the next action command in the event entry.

##### Example

# This command will result in a 30 seconds pause in the programming script execution. * * * * * * : sleep –time 3000; %

## 5.9 LED

This command change the status of LED indicator, there are two indicators in Video Server, one is Power/Status, the other is Network, both indicator could be change by Action commands.

##### Syntax

led [-on | -off | -toggle | -norm | -blinkfast | -blinkslow]

##### Options

-on:
Set the P/S LED to on state, the color became orange.
-off:
Set the P/S LED to off state, the color become red.
-toggle
Toggle the P/S LED between orange and red.
-norm
Set both the P/S and Network LED back to factory default state.
-blinkfast
Set the Network LED to fast blinking.
-blinkslow
Set the Network LED to slow blinking.

## 5.10 Motion

This command control the motion detection mechanism, in priors to generate motion detection trigger, must enable (init and start) the motion detection in background task. And due to the motion detection take system overhead and resources, when no use of motion detection, could be disable (del).

Note: Before start up event script, must select the detection area in user interface of motion detection setting, otherwise below motion script won’t take into action.

### 5.10.1 motion_init

The motion_init command initiates and allocate memory space for the motion detection image buffer. This is needed in order to detect image change using the motion_start commands.

##### Syntax

motion_init [-cam VIDEOCHANNEL] [-sensitivity SENSITIVITY];

##### Options

-cam
Specifies the camera source, i.e. 1- 4
- sensitivity
Specifies the sensitivity of motion detection, omit will set to default value as 5, set less for more sensitive.

### 5.10.2 motion_del

The motion_del command free all allocated memory space and stop running task of the motion detection. None of argument needed.

### 5.10.3 motion_start

The motion_start command start the motion detection task in background. Once detect the difference between images, will activate action command following motion trigger.

##### Syntax

motion_start [-cam VIDEOCHANNEL] [-interval TIME] 
[-duration TIME] [-holdtime TIME];

##### Options

-cam
Specifies the camera source, i.e. 1- 4
-interval
Specifies the time interval between consecutive snapshots in unit of 10ms system tick. If omitted, default value is 50 (i.e. 500ms).
-duration
Specifies the total time for the command to run. Accept range is
Omit or –1 or 0: 	Infinite loop,
>0: 			Value in unit of 10ms system tick
If you specify infinite loop, the command will continue to be executed until it is interrupted by motion_start, motion_del.
-holdtime
Specifies the pause time when detect the image changes, after the holdtime, will restart the motion detection task, if omit, normally image change will detect twice (IN and OUT of object), this option used to improve system performance, and to avoid double alarm when detect the image changes.

##### Example

# This command will init motion detection when start event * * * * * RUN: motion_init -cam 1; motion_start –cam 1 –interval 500 –duration –1; %

### 5.10.4 motion_stop

For real-time system, to avoid consuming system performance, or, to stop motion detection in certain period, use motion_stop to suspend the motion detection task.

It’s useful when event frequently suspended and reactivated of motion detection.

##### Syntax

motion_stop [-cam VIDEOCHANNEL];

##### Options

-cam
Specifies the camera source, i.e. 1- 4

##### Example

# This command will init motion detection when start event * * * * * RUN: motion_init -cam 1; motion_start –cam 1 –interval 500 –duration –1; % # Will suspend motion detection for 10 seconds when GPIN 1 triggered * * * * * /I1: motion_stop –cam 1; sleep –time 1000; motion_start –cam 1 –interval 500 –duration –1; %

## 5.11 serial

The serial command sends a message to serial com port.

##### Syntax

serial [-comport COM] [-message STRING];

##### Options

-comport
Specifies the com port in range [1..2]. if omit, default is com2. 
-message
Specifies the message that will be sent to the remote host. The message must be surrounded by quotation marks. Hexadecimal coded messages must be preceded by \0x, e.g. "\0x04" for the non-printed character EOT. To include a quotation mark, it must be preceded by a backslash, e.g. \".

##### Example

# This command will send the message "Alarm # "0013" from Video Server" to com2 when # input port 1 goes high. * * * * * /I1: serial -port 2 -message "Alarm \"0013\" from Video Server"; %

# 6. Related CGI Commands

To access Video Server event system from CGI command which provide convenient way to integrate from System Integrator.

The CGI syntax like http://IP/evscript?[cmd], for example, if Video Server IP address is 192.168.0.101, then enter http://192.168.0.101/evscript?runcmd=check will get result code regarding event parsing / running / bootflag status.

The event script represented as file contain in Video Server “Sys/scr00.txt” user could either update the file by FTP upload, or submit from CGI command http://IP/evscript?setscript=EVENTFILE , below descript all event script related CGI command.

Note: To save any changes of the script and flag into flash mush click “save changes” in user interface, or enter http://IP/eepromsave CGI, otherwise will restore the old one after reset.

## 6.1 Get Script File

##### Syntax

#### http://IP/evscript?getscript

This command load the script file “Sys/scr00.txt” and response to browser as simple text.

## 6.2 Set Script File

##### Syntax

#### http://IP/evscript?setscript=EVENTFILE

This command receive the EVENTFILE and save as “Sys/scr00.txt” , then do the parsing job, below command will query the parsing result.

## 6.3 Get Script Command Return Code

##### Syntax

#### http://IP/evscript?getresult

This command return the result of last parsing or start event result, to get correct result, recommend delay one second before issue this CGI command to allow Video Server complete the parsing or event startup job, the return will be:
OK
Fail
or Text which indicate the begging of syntax error in script file.

## 6.4 Set Script Boot Flag

##### Syntax

#### http://IP/evscript?setboot=[0/1]

This command set the boot flag to indicate Video Server to startup event after reset.
0:	Do not start up event
1: Start up event after reset

## 6.5 Start Script

##### Syntax

#### http://IP/evscript?runcmd=start

This command will load the script file, parsing, and start running the event script.

## 6.6 Stop Script

##### Syntax

#### http://IP/evscript?runcmd=stop

This command will stop the running script.

## 6.7 Check Script Status

##### Syntax

#### http://IP/evscript?runcmd=check

This command will return the status of event system. As below:
Parsing=0/1:		1 means parsing done, 0 means not yet parsing or err
RunStatus=0/1:	1 means event started, 0 means event stop
BootFlag=0/1:	1 means start event after reset, 0 means not.

## 6.8 Show Log File

##### Syntax

#### http://IP/evscript?showlog=FILENAME

This command will return the log file specified by FILENAME