Firmware 1.4 Release Note, 2006/07/20

[Base Version] FW1.31

[Important Note]

1. Upload patch file before upgrade firmware.

2. There are different firmware files:

2.1 “system.bin.14_intel” is for on board VGA, using on VP404/4048 units & 945G based MB.

2.2 “system.bin.14_std” needs VGA card to see the videos.

2.3 “system.bin.14_all” firmware will detect VGA card automatically.

2.4 There are different firmware files for PAL and NTSC standard.

2.5 For 32MB DOM, firmware and patch files are under “DVR Firmware\Fw1.4\32MBDOM” directory. You can check your DOM for 32MB by entering BIOS and see IDE HDD device “PQI HDD”.

2.6 The patch file “system.bin.en.14_32mb_patch” is dedicated for 32MB DOM. DO NOT use this file for other size DOM patch.

2.7 Please upgrade 32MB DOM firmware from remote IE browser. (See Read Me file)

3. It is recommended to upload patch file once ONLY.

4. When using SATA HDD, Disk mode should be “Enhance mode” under BIOS setting.

5. File recorded by MPEG4 compression can NOT do “Step Backward” playing.

6. When using VP2X series capture card, if NO GPIO card installed, GPI light on GUI screen will remain at trigger state.

7. Audio should be assigned to corresponding cameras before start to record. Otherwise, audio will NOT be recorded into hard disk. (From FW1.4, VPON supports multiple Audio sources in ONE DVR system)

[New Features]

1. Supporting newly released: VP24/VP28 (PCI), VP42 (PCI-E) cards.

2. All compression algorithm supporting Water Mark.

3. New Weight Recording function to increase recording frame rate when motion or alarm trigger.

4. Support “Network Neighborhood”. PC client under Windows OS can access recording files inside DVR remotely.

5. Can export or import system configuration file for setting multiple DVR units.

6. Support alarm recording and playback.

7. Enhanced recording files search such as POS text and alarm searching

8. Easier PTZ supporting by FTP PTZ Scrip File to add new PTZ Device.

9. Camera setting can be copied to multiple cameras.

10. Default supporting one POS unit FREE. POS connection can be expanded to 4 or 16.

11. Support 4 Ports Net I/O (VP-204Q) devices for POS integration.

12. Support Intel On-Board VGA.

[Known Problem]

1. Sun light saving time at London area is NOT yet correct.

2. Camera video standard should be set manually to NTSC or PAL.

3. When remote playback, “STOP” button should be pressed before next file can be played.

4. When using alarm trigger recording, it is highly recommended to set at least one of the cameras as continuous recording or motion trigger recording to prevent from system hang up when try to playback the recording file that is currently been written.

5. On VP2X series, when a camera is removed, its previous image will remain on the screen. It is recommended to turn on “video loss” function.

6. Keypad under OSD mode is NOT yet correct. Please use a keyboard to enter the words.