When upgrade firmware to version 1.4, please do by following steps.

- It is suggested to upgrade firmware from “Remote”. (Use http://aa.bb.cc.dd/revise.htm to do upgrade VPON firmware. Where aa.bb.cc.dd means VPON IP address.)

- If you do upgrade from local VGA screen, please change VPON to “OSD mode”. (VPON will reboot if you are under “GUI mode”)

- Upgrade "system.bin.en.14_32mb_patch" file first. After complete, VPON will reboot twice to be ready.

- After step#2 is ready, upgrade again using "system.bin.en.14_0_POSREC_std_32mb" or appropriate file. After complete, VPON will reboot again to be ready.

- Firmware upgrade may get failed if user does NOT follow above instruction.

- Check release note after you upgrade to FW1.4.

— Standard Firmware Selection —

| Firmware Name | DVR / Camera standard |  |
| --- | --- | --- |
| system.bin.en.14a_std_pal_32mb | VGA card user | PAL |
| system.bin.en.14a_std_32mb |  | NTSC |
| system.bin.en.14a_intel_pal_32mb | On board VGA user | PAL (For VP404/4048 units & 945G based M/B) |
| system.bin.en.14a_intel_32mb |  | NTSC (For VP404/4048 units & 945G based M/B) |

— Special Firmware Selection —

| Firmware Name | DVR / Camera standard |  |
| --- | --- | --- |
| system.bin.en.14_0_MOVS_std_32mb | VGA card user | W/ Mobile phone supporting function |
| system.bin.en.14_0_POSREC_std_32mb |  | W/ POS Text & Event recording function |