WebMonitor Plus Hardware Specification (Revised) :

| ### Audio Input / Output | - Support half duplex audio I/O - Two audio output - Three audio input |
| --- | --- |
| ### Video Input / Output | - Support NTSC/PAL - Six video output - Six video input with video standard auto detection |
| Compression | - Support H.263, M-JPEG, JPEG video compression format |
| Resolution | - Image size: 320x240 (JPEG), 176x144 (H.263) - 24 bits true color |
| Performance | - Up to 10 frame/second (JPEG), 10 frame/second (H.263) |
| Typical Bandwidth | - 100K Byte/second (JPEG) - 5K Byte/second(H.263) |
| Hardware | - 32bits CPU, 8MB DRAM - 2MB Flash-ROM for firmware - One MODEM Port - One Camera Control Port - Four digital input and four output for surveillance - Front panel LEDs power indication and status display - Built-in a pre-configured mini web server |
| Installation | - Assign IP address, ISP phone number, login name, password |
| System Compatibility | - All PC running Navigator 3.0 or later |
| Protocols | - Support TCP/IP, PPP, HTTP |
| Connectors | - One 10BaseT Ethernet port - Six RCA connectors for video input - One DB9 Male Connector for configuration and Modem connection - One DB9 Male Connectior for Camera control - One DB9 Female Connector for video output - Three RCA connectors for audio input - Two RCA connectors for audio output - Two 8-pins Mini-DIN connectors for digital input/output |
| Power Supply | - 25 watts power consumption, 90-264VAC, 47-63HZ |
| Size | - 300mm(W) x 165mm(D) x 55mm(H) |
| Accessory | - RS232 Cable - Power Cord - Manual - System Configuration Software - Mini-DIN to Terminal Block Cable - DB9-to-six-RCA Cable |

WebMonitor Plus Software Functions

|  | - 線上切換中英文網頁, 用一般瀏覽器即可查看 - 可同時查看所有攝影機影像 - 可針對特定區域影像做 ZOOM-IN 效果 - 線上選擇想要查看的攝影機影像 - 可多人同時查看 - 即時顯示系統時間 - 即時擷取單張影像存檔 - 4組警報輸出入接點, 可外接各類偵測器材 - 提供Netscape Plug-ins, 提升低頻寬環境的 Frame Rate - 可搭配 OYO Viewer, 做為整合性即時監控系統 - 可以URL連接到任何網站, 不須開發任何程式 - 系統設定值查看 - Ethernet 設定值 (MAC Address, IRQ, I/O base...) - TCP/IP 設定值 (IP Address, DNS, Gateway, Subnet Mask) - COM Port Status - 攝影機設定 - 亮度 ,明暗對比, 色彩飽和度 , 彩度 , 影像品質 - 即時查看設定結果 - 設定 WebMonitor 系統時間 - 設定監視系統參數 - 其他系統設定 - TCP/IP stack configuration(IP Address,DNS, Gateway, Subnet Mask) - ISP configuration (Phone Number,Login Name,Password,Modem Baud Rate) - Authenticate configuration (User Name, Password) - Set Video Compression Algorithm |
| --- | --- |

## WebMonitor Plus Software Functions

- On-line alter Home pages of English version or Chinese version, view the video with Netscape Navigator

- View the video of all cameras connected to WebMonitor at the same time

- ZOOM-IN the video of a specified portion

- On-line choose to view the video of a certain camera

- Multi-users login in at the same time

- Show the real time clock of the system

- Provide Netscape Plug-Ins to increase Frame Rate in the narrow bandwidth environment

- Use OYO Viewer ( a proprietary software) to apply at the integrated real time surveillance system

- Connect to any Web Site by URL, don’t need to develop any program

- Check the system setting

- Ethernet configuration (MAC Address, IRQ, I/O base…)

- TCP/IP configuration (IP Address, DNS, Gateway, Subnet Mask)

- COM Port Status

- Configure the cameras attribution

- Brightness, Contrast, Color Situation, Hue, Quality of the video

- Real time check the configuration

- Set WebMonitor’s system time

- Set surveillance system parameters

- Other system configuration

- TCP/IP stack configuration (IP Address, DNS, Gateway, Subnet Mask)

- ISP configuration (Phone number, Login Name, Password, Modem, Baud Rate)

- Set Video Compression Algorithm

OYO Viewer 1.0 Software Functions:

- 透過MODEM 撥接WebMonitor

- 提供驅動MODEM程式介面, 撥接連線PC和WebMonitor

- 快速撥號/電話簿

- 提供快速撥號, 記憶已撥接電話號碼.

- 電話簿功能, 可管理撥接電話號碼.

- 密碼限制查看權限

- 撥接連線完成後, 需通過密碼檢查才能查看WebMonitor

- 可同時查看所有攝影機影像

- 線上選擇想要查看的攝影機影像

- 即時顯示系統時間

- 即時擷取影像存檔

- 擷取螢幕顯示之影像存檔, 大小及影像品質與螢幕顯示相同

- 事件驅動自動回撥傳回影像

- GPI輸入點可驅動WebMonitor自動回撥功能, 撥接連線完成後, WebMonitor會將事件發生瞬間以後的影像傳回OYO Viewer. 之間若有異常斷線, OYO Viewer會主動再撥接連線將影像取回

- 遠方傳回影像自動存檔

- 傳回影像會自動存檔, 以供播放

- 播放遠方傳回影像

- 提供影像播放功能可播放任何已存檔之影像

- 線上系統設定

- 可線上設定亮度 ,明暗對比, 色彩飽和度 , 彩度 , 影像品質

Viewer Functional Block Diagram

Viewer Application

Viewer Kernel DLL

Win32

COMM	API

Video

Decoder

DDRAW

COMM

Driver

VGA	Driver